3GPP TSG-RAN WG4 Meeting #112bis												R4-2415269

Hefei, China, 14 – 18 October, 2024

Agenda item:			6.12.2.1

Source:	Huawei, HiSilicon

Title:	on XR OTA test

Document for:

# Introduction

The WF [1] in RAN4 #112 groups XR devices into two categories.

Issue 2-1-1: Discussions on Reply LS from CTIA on XR OTA test phantom

Agreements:

  - Two types of XR, i.e., glasses and head mounted display are prioritized in this WI.

  - Need to send this two XR types to CTIA for phantom positioning and MU study.

Issue 2-1-4: Whether WI should focus on the XR devices those connected to gNB directly

Agreements:

  - Only test XR devices with 5G link to gNB in networks, not those with side link or wired connections.

The performance metrics associated with the two types XR devices are discussed.

# Discussion

As agreed in Issue 2-1-4, XR devices with 5G link to gNB will be included. This means that the TRP and TRS performances should be omni directional. Namely traditional TRP and TRS test methodology and metrics apply.

In addition, agreement in Issue 2-1-1 identifies two device types, which would have different levels of performance due to the space available for antennas. For example, glasses would have less space for antennas compared to head mounted displays. Generally, for the same antenna type, the larger the space available for antennas, the higher the total antenna efficiency. Obviously, different antenna type or more antennas can be placed into larger space available. In other words, it would not be appropriate to compare the performances between XR glasses and head mounted displays.

Proposal 1: glasses and head mounted displays should be tested using the existing TRP and TRS measurement methodology.

Proposal 2: glasses and head mounted displays should have separate performance metrics.

It is agreed in WF from the last meeting that supportOf2RxXR-r18 is used to identify XR devices with 2Rx. As pointed out in a previous contribution [2], an associated IE at gNB needs to be set accordingly to avoid XR devices with 2Rx being barred from accessing gNB.

Proposal 3: set cellBarred2RxXR-r18 (ENUMERATED) to {false} during OTA tests for XR devices with 2Rx.

If 2Rx XR devices can be identified, there would be two performance metrics, namely 2Rx and 4Rx, for each device category defined as glasses and head mounted displays. This undoubtedly increases measurement effort. An alternative approach would be to use the 2Rx XR reference sensitivity allowance to derive 2Rx XR TRS performance from the 4Rx one.

Table 7.3.2-2b: Two antenna port XR UE reference sensitivity allowance ΔRXR,2R

| Operating band | ΔRXR,2R (dB) |
| --- | --- |
| n7, n38, n41, n48, n77, n78, n79 | -1.0 |

Proposal 4: as an alternative, derive 2Rx XR device TRS performance from 4Rx TRS using Table 7.3.2-2b from TS 38.101-1

Subscriber Profile ID 252 could be another way to identify XR devices with 2Rx as shown in Table D-1 from TS 38.300 [4]

Table D-1: gNB local configuration in idle and connected mode for SPID = 252

| Configuration parameter | Value | Meaning |
| --- | --- | --- |
| 2Rx XR device subscriber | true | The selection provides information that subscriber is an XR device and is permitted to utilise a minimum of two Rx antenna ports for the NR bands where four Rx antenna ports are baseline as given in TS 38.101-1 [18]. |

Observation 1: Subscriber Profile ID 252 in Table D-1 from TS 38.300 could be another way to identify XR devices with 2Rx.

# Conclusion

This contribution makes the following proposals.

Proposal 1: glasses and head mounted displays should be tested using the existing TRP and TRS measurement methodology.

Proposal 2: glasses and head mounted displays should have separate performance metrics.

Proposal 3: set cellBarred2RxXR-r18 (ENUMERATED) to {false} during OTA tests for XR devices with 2Rx.

Proposal 4: as an alternative, derive 2Rx XR device TRS performance from 4Rx TRS using Table 7.3.2-2b from TS 38.101-1

Observation 1: Subscriber Profile ID 252 in Table D-1 from TS 38.300 could be another way to identify XR devices with 2Rx.

# Reference

[1] R4-2413603 WF for TRP_TRS_MIMO_OTA

[2] R4-2407104 on test methodology for FR1 non-RedCap headworn XR devices

[3] TS 38.101-1 v18.5 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[4] TS 38.300 v18.3 NR and NG-RAN Overall Description