3GPP TSG-RAN WG4 Meeting #112-bis                                                          R4-2415271

Hefei, China, Oct 14 – Oct 18, 2024

Agenda item:          6.17.1

Title:	Discussion on general aspects for AI/ML air

WI/SI:	NR_AIML_air-Core

Release:	Rel-19

Source:	Korea Testing Laboratory

Document for:	Discussion

# 1. Introduction

In this document, we provide a technical view of the additional conditions and the associated ID agreed upon at the last RAN1 #118 meeting[1,2] and the signalling procedures regarding the RAN4  WF testing goal on AI/ML [3] and additionally regarding the overfitting problem all ML confront.

2. Discussion

2.1 Additional conditions and associated ID

At the last RAN1 #118 meeting, it was agreed that the model identification type A with more details related to use cases and the model identification type B with more details related to all use cases will be studied in Table 1 and the associated ID for UE sided model in beam management was agreed in Table 2 that it can at least be configured within the CSI framework. The associated ID for UE sided model in beam management is directly linked to the additional conditions in Table 3  which could have too many combinations. Here is the concept of associated ID. AI/ML has the characteristics of overfitting and generalization. When the performance is excellent only on a specific data set and the performance deteriorates on other datasets, it may be overfitting. The characteristics of the generalization have the expected performance presented on various data sets while the generalization has limitations. In other words, it can be defined that its function can work well under a similar property environment between the training and the inference. This means that there is a range of the data set where the expected performance is guaranteed. Therefore, the set (range) of datasets including the generalization performance can be the requirement for the operation of the AI/ML use case, which can be called the associated ID (or Dataset ID). Similarly, it is possible to distinguish whether a particular dataset can provide the expected performance if it is related to NW-sided additional conditions called ID-A for example, which is the identification between gNB and UE, directly correlated with the generation of a particular dataset by configured and indicated under the name of ID-A that is the additional condition such as the same downlink spatial domain transmission filter, specific mobile environments, the gNB configuration, etc. It should be noted that it was agreed that additional conditions do not form part of the UE capability for the AI/ML-enabled feature/FG and does not imply that additional conditions are necessarily specified. We can develop multiple AI/ML models based on the additional conditions or data sets and identify them according to the additional conditions or data sets that are independently separated. RAN1 allows the identification(indication) or recognition of a specific AI/ML model according to the associated ID, which implicitly represents a similar property provided by the mobile environment. The NW-side additional conditions and UE-side additional conditions are independent environments.

In summary, it is said that “it does not imply that additional conditions are necessarily specified” and “information and/or indication on NW-side additional conditions is provided to UE”. In Table 2, it was agreed that UE sided model in beam management supports the associated ID. For AI/ML beam management conformance testing, the association ID is provided between a test setup(TE) and DUT, which means that the test setup can be configured based on the additional condition of the conformance specification. Our understanding it that there could be many combinations of both, and it is unclear how the scope of RAN4 conformance test could be adapted.  Fortunately, there are open discussion areas in Table 3, as it has been decided that the need for specification impact is a separate discussion.

Observation 1: There are numerous combinations of NW-side additional conditions and UE-side additional conditions.

Observation 2: The additional conditions must be defined at least for the conformance testing when the associated ID is used between a test setup and a DUT.

Proposal 1:  It is necessary for RAN4 to study and discuss the NW-side additional conditions and the UE-side additional conditions for the conformance testing.

Table 1 : TPs to capture the outputs of 9.1.3.3 [1]

From RAN1#118, R1-2407520 --------------------------------------------------------Text omitted --------------------------------------------------------- ### 4.2.1 LCM Flavours The LCM procedure is studied for the case that an AI/ML model has a model ID with associated information and/or for the case that a given functionality is provided by some AI/ML operations. Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. From RAN1 perspective, an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Legacy 3GPP framework of feature is taken as a starting point. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. RAN1 studies the model identification type A with more details related to use cases. RAN1 studies the following options as starting point for model identification type B with more details related to all use cases. - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) - MI-Option 2: Model identification with dataset transfer - MI-Option 3: Model identification in model transfer from NW to UE - FFS: The boundary of the options - Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose Additionally, the following options are proposed for model identification type B by some companies: - MI-Option 4: Model identification via standardization of reference models (for CSI compression) . MI-Option 4 refers to the inter-vendor collaboration Option 1 of AI-based CSI compression - MI-Option 5: Model identification via model monitoring Regarding MI-Option 1 (Model identification with data collection related configuration(s) and/or indication(s)) of model identification type B, RAN1 further study the following aspects: - Relationship between model ID and data collection related configuration(s) and/or indication(s) - Information transmitted from NW to UE (if any) - Information transmitted from UE to NW (if any) - The associated procedure - Usage/Applicable use case(s) of MI-Option 1 - Note: whether MI-Option 1 is needed or not is a separate discussion. From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification. From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models may not be identified at the Network, and UE may perform model-level LCM. Whether and how much awareness/interaction NW should have about model-level LCM requires further study. For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. After functionality identification, necessity, mechanisms, for UE to report updates on applicable functionality(es) among functionality(es) are studied, where the applicable functionalities may be a subset of all functionalities. Applicable functionalities can be reported by the UE. In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. After model identification, necessity, mechanisms, for UE to report updates on applicable UE part/UE-side model(s), are studied, where the applicable models may be a subset of all identified models. Applicable models can be reported by the UE. How to handle the impact of UE’s internal conditions such as memory, battery, and other hardware limitations on functionality/model operations and AI/ML-enabled Feature is to be studied. Note: it does not preclude any existing solutions. For functionality/model-ID based LCM, once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring. Model ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations. ---------------------------------------------------------Text omitted ---------------------------------------------------------

Table 2 : Session notes for 9.1 [2]

From RAN1#118, Xiaodong's Session Notes RAN1#118 (9.1 AI&ML) v08.doc Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Table 3 : TPs to capture the ouitputs of 9.1.3.3 [1]

From RAN1#118, R1-2407520 ----------------------------------------------------------Text omitted --------------------------------------------------------- ### 4.2.3 Additional conditions For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. It does not imply that additional conditions are necessarily specified. Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions. Note: whether specification impact is needed is a separate discussion. For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary): - Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side - Model training at NW and transfer to UE, where the model has been trained under the additional condition - Information and/or indication on NW-side additional conditions is provided to UE - Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality) - Other approaches are not precluded - Note: the possibility that different approaches can achieve the same function is not denied Regarding the associated ID [defined in Section 4.2.1] for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study). -----------------------------------------------------------Text omitted ---------------------------------------------------------

2.2 Testing goals

From RAN4#115 Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

At the last RAN4 #115 meeting, it was agreed that the testing goal is to verify that the minimum performance of AI/ML functionality/feature can be achieved. Currently, it is currently understood to test the minimum performance of AI/ML functionality/feature, for example, the association ID #1 representing AI/ML beam management while with together operating the LCM function such as the performance monitoring.

For gNB sided decision for the LCM operation, based on the results of the model monitoring from UE, gNB can decide on model updating/switching or fallback to legacy schemes. There are implementation areas such as the fallback performance thresholds that determine when and how gNB commands the fallback to the UE depending on the performance monitoring.  When configuring the conformance test environment, the test setup may decide that since gNB manages the instructions for the LCM operation, the test results are pass or fail based on the performance monitoring reports. In this case, the DUT would not fall back to the legacy scheme operation even if the performance is poor.

For UE sided inference, the LCM function, provided the performance monitoring and also selection, switch, activation, deactivation and fallback, can fallback to non-AI/ML function or a legacy algorithm during the conformance testing and re-activate the AI/ML functionality/feature, the association ID #1, or switch to another AI/ML physical model UE implemented under the scope of the identical AI/ML functionality, the association ID #1. Of course, this circumstance means that UE sided inference and LCM are running. As we all know, the AI/ML performance provides the stochastic performance variation such as “AI/ML can achieve about 70%~80% beam prediction accuracy” in TR 38.843. This means that the AI/ML performance results can be defined within a certain interval the minimum requirements. In other words, it is a general way to make a decision by observing the average monitoring results whether the minimum performance of the AI/ML functionality/feature would be pass or fail.

It could be an efficient way to consider separating the conformance testing processes for AI/ML functionality/feature as follows.

Firstly, during conformance testing when the LCM management is operating on the UE sided, performance monitoring and reporting is required but the transition to deactivation and fallback shall be avoided. Our understanding is that this can be configured by setting the threshold or the management of the fallback/deactivation function is set to a very low performance requirement (AI/ML test mode),in order to avoid the deactivation and the fallback.

Secondly, it is the normal operation state that the conformance test performs AI/ML functionality/feature where the UE’s LCM management can report to gNB such as “Management Request” or “Management Decision Request” [38.843]. In this signalling proceudure we can expect(inference) various outcomes. It simply passes if there is not a single “Management Request” or “Management Decision Request” such as fallback request during the conformance test. It might be acceptable if there is an occasional “Management Request” or “Management Decision Request” such as fallback.  It is inadequate if  “Management Request” or “Management Decision Request” is sent to gNB(Test setup) most of the time during the test.  It is difficult to decide whether the minimum requirement has been met as the AI/ML performance provides the stochastic performance variation in case of almost full of signalling procedures. However, there is the monitoring function that we can utilize as shown in the first scenario. Namely, the first setup is more efficient process without wasting test time and cost.

Thirdly, to verify the function of LCM management, a separate or a fake AI/ML inference mimic environment.

In summary, RAN4 can do the conformance test in a way of efficiency way by using them if RAN1/RAN2 define the detail fallback and the deactivation mechanism considering on the view or opinion of RAN4’s. It should be discussed within RAN4 group for the way of efficiency conformance test by using the mechanism of the fallback / the deactivation if it might not be possible to define the mechanism in detail since the reason of the implementation area.

Table 4 Signalling procedures for model and functionality life cycle management [38.843]

| For example: Network decision, network-initiated AI/ML management (replace Network to TE) | For example: Network decision, UE-initiated AI/ML management(replace Network to TE) |
| --- | --- |
| For example: UE decision, event-triggered as configured by the network(replace Network to TE) | For example: UE autonomous, decision reported to the network(replace Network to TE) |

Observation 3: The AI/ML performance provides the stochastic performance variation.

Observation 4: It has different behavior to verify the minimum performance of AI/ML functionality/feature depending on “Signalling procedures for model and functionality LCM” in TR 38.843.

Proposal 2: It should be discussed within RAN4 group for the way of efficiency conformance test using the mechanism of the fallback / the deactivation if it might not be possible to define the mechanism in detail since the reason of the implementation area.

Proposal 3: It is efficiency way for the conformance test such as the performance test, except the protocol test, to decide one of between “Network decision, network-initiated AI/ML management (replace Network to TE)” and “UE decision, event-triggered as configured by the network (replace Network to TE)”

Overfitting is a common problem in machine learning (ML) models, where the model performs well on training data but poorly on new, unseen data. To prevent overfitting, there are various methods that can be used, such as early stopping, data augmentation, regularization, and modifying the ML model with techniques like adding dropout layers or batch normalization. From the RAN4 perspective, it is emphasized that data augmentation is crucial for controlling test environments, while other approaches should be considered by UE vendors.  Required efforts are made to configure models to reflect real-field environments for AI/ML use cases, however, it's important to note that in the conformance testing, RAN4 sets minimum requirements for each test case. While data augmentation is a powerful tool for mitigating overfitting, it may not completely prevent the issue in all cases when solely relied upon. Consequently, it is necessary to validate whether test configurations, especially those considering data augmentation, can effectively prevent overfitting. If there is uncertainty, other approaches should be taken into account.

It's important to recognize that an AI/ML model could exhibit poor performance in a field deployment condition due to overfitting and/or data drift, among other factors. While the test environment may have been designed to mitigate overfitting, it can still be challenging to completely avoid overfitting (local minima) in field deployment conditions. It is necessary to validate whether a DUT has the overfitting problem under the test configuration(setup) may not completely prevent overfitting. For an example, if a DUT can be trained and validated under a test setup to check the overfitting problem, a test setup (a TE) can send a signal to a DUT to prohibit the LCM management of a DUT such as fallback/deactivation/switch(another AI/ML model) or set a very low threshold for fallback/deactivation/switch(another AI/ML model). In this configuration, while the validation process is running, a DUT can simultaneously report the performance monitoring result to a test setup(TE) and through this procedure(step) we can check if the overfitting problem exists.

Observation 5: Required efforts are made to configure models to reflect real-field environments for AI/ML use cases, however, it's important to note that in the conformance testing, RAN4 sets minimum requirements for each test case.

Observation 6: While data augmentation is a powerful tool for mitigating overfitting, it may not completely prevent the issue in all cases when solely relied upon.

Proposal 4: It is necessary to validate whether test configurations can effectively prevent overfitting. If there is uncertainty, other approaches should be taken into account.

Proposal 5: It is necessary to validate whether a DUT has the overfitting problem when test configurations may not completely prevent overfitting.

3. Conclusion

In this contribution, we provided our viewpoints on the additional conditions and the associated ID and test goal of general aspect and overfitting.

Observation 1: There are numerous combinations of NW-side additional conditions and UE-side additional conditions.

Observation 2: The additional conditions must be defined at least for the conformance testing when the associated ID is used between a test setup and a DUT.

Proposal 1:  It is necessary for RAN4 to study and discuss the NW-side additional conditions and the UE-side additional conditions for the conformance testing.

Observation 3: The AI/ML performance provides the stochastic performance variation.

Observation 4: It has different behavior to verify the minimum performance of AI/ML functionality/feature depending on “Signalling procedures for model and functionality LCM” in TR 38.843.

Proposal 2: It should be discussed within RAN4 group for the way of efficiency conformance test using the mechanism of the fallback / the deactivation if it might not be possible to define the mechanism in detail since the reason of the implementation area.

Proposal 3: It is efficiency way for the conformance test such as the performance test, except the protocol test, to decide one of between “Network decision, network-initiated AI/ML management (replace Network to TE)” and “UE decision, event-triggered as configured by the network (replace Network to TE)”

Observation 5: Required efforts are made to configure models to reflect real-field environments for AI/ML use cases, however, it's important to note that in the conformance testing, RAN4 sets minimum requirements for each test case.

Observation 6: While data augmentation is a powerful tool for mitigating overfitting, it may not completely prevent the issue in all cases when solely relied upon.

Proposal 4: It is necessary to validate whether test configurations can effectively prevent overfitting. If there is uncertainty, other approaches should be taken into account.

Proposal 5: It is necessary to validate whether a DUT has the overfitting problem when test configurations may not completely prevent overfitting.

4. Reference

[1] R1-2407520, “TPs to capture the outputs of 9.1.3.3”

[2] R1-24xxxxx, “Xiaodong's Session Notes RAN1#118 (9.1 AI&ML) v08.doc

[3] R4-2414323, “WF on AI/ML”