3GPP TSG-RAN WG4 Meeting#112bis						         			R4-2415273

Hefei, China, 14 – 18 October 2024

Title: 	Using single CC MPR and ULCA related emissions when only one CC has RBs allocated

Source: 	Skyworks Solutions, Inc.

Agenda Item:	6.1.1.2.2 MPR applicability for FR1 intra-band UL CA [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN4#112, our contribution [1] proposed that in order to enable the use of single CC MPR when only one CC has active RBs allocated, and when a single LO is used for contiguous or non-contiguous ULCA, the UE may not meet single CC SEM/ACLR unless it retunes its LO and thus require and UL interruption. Based on options captured in way forward [2], it seems that our proposal was not well explained resulting in some confusion on which relaxed MPR was proposed. In this contribution, we re-explain the issues and make a more explicit proposal on whether the emissions requirement should be based on a single CC or on the configured ULCA to enable the use of the single CC MPR.

# Discussion

## 2.1 Way forward inputs

In way forward [2], the following options were captured on MPR improvements for intra-band contiguous ULCA.

#### Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell

- Proposals

  - Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply. (Samsung)

  - Proposal 2: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied (Samsung, CATT,, Nokia, Huawei, LGE)

    - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

    - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

    - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

- Proposal 3: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements (Qualcomm).

    - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4

    - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4

    - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4

- WF: FFS in next meeting

#### Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE

- Proposals

  - Proposal 1: Integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW. (Samsung)

  - Proposal 2: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR. (Qualcomm, Huawei)

- WF: FFS in next meeting

#### Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell

- Proposals

  - Proposal 1: Even when transmitted RBs are allocated in only one of the CC, the emissions requirement based on the configured intra-band contiguous ULCA are used (SEM, ACLR, spurious emissions) and all allocations can use the single CC MPR for inner. FFS if a similar approach is applicable to some additional emission requirements. (Skyworks)

  - Proposal 2: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. (CATT, Samsung, ZTE, vivo)

  - Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, the spurious emissions/ACLR/SEM are kept for aggregated/configured CBW. (Huawei, Ericsson)

- WF: FFS in next meeting

#### Issue 2-1-4: Draft Rel-19 CR on MPR applicability for intra-band contiguous CA with single CC with activated cell

- Proposals

  - Proposal 1: Add the following description into clause 6.2A.2.1:

    - “For intra-band contiguous carrier aggregation with single CC with activated cell, MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.”

- WF: FFS the details of draft CR

In way forward [2], the following options were captured on MPR improvements for intra-band non-contiguous ULCA.

#### Issue 2-2-1: General considerations

- Agreement: The work scope for FR1 NC CA should be limited to only study MPR applicability.

#### Issue 2-2-2: Applicable MPR for FR1 intra-band non-contiguous UL CA

- Proposals

  - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required. (Qualcomm, ZTE, Huawei)

  - Proposal 2: When DualPA is not signalled or TxD or UL MIMO is signalled, transmit interruption may be needed to allow LO switching and the single carrier MPR can be sued when transmitted RBs are allocated in only one of the CC. (Skyworks)

  - Proposal 3: for non-contiguous UL CA configurations with sub-blocks consisting of one cell and supported by dual PA architecture, the non-CA (single CC) MPR applies for one cell active among the configured uplink serving cells, the other cell deactivated. (Ericsson)

  - Proposal 4: for non-contiguous UL CA configurations with sub-blocks consisting of one cell not supported by a dual PA architecture, applicability of the non-CA (single CC) MPR for one active cell among the configured uplink serving cells, the other cell deactivated, is subject to UE capability, e.g. indication of [mpr-singleCC-activated-FR1] for the band combination. (Ericsson)

- WF: FFS in next meeting

This contribution will address the issues listed above and specifically clarify under which emission requirement conditions some PA architectures may apply the single band MPR when RB are allocated in only one of the UL CC.

## 2.2 Single PA and 2Tx implementations for ULCA without LO switching.

For intra-band ULCA implemented with two LO (DualPA IE), each carrier has its own LO thus when only one CC is allocated, one of Tx path can be switched off, thus the UE is transmitting in the same mode than for a single carrier transmission. In that case, the single carrier emission (ACLR, SEM, spurious emissions) can be applied and the single carrier MPR is valid.

In single PA or 2Tx cases however, both CCs are transmitted by one or two PAs using a single LO frequency placed in the middle of the total 2CC bandwidth: If the LO is not switched to avoid the transmit interruption, the transmitted signal is no longer the same as that of a single CC as the image and IMD products are in a different position and outside the single CC bandwidth.

To illustrate this, the two following figures show the contiguous and non-contiguous cases with TRX impairments for a single LO frequency intra-band ULCA implementations when only one of the carriers is allocated some RBs:

- Figure 1 for the intra-band contiguous case with two 100MHz contiguous carriers:

  - The fully allocated lower carrier: 100 full label (cyan)

  - The lower carrier allocated with lowest RB: 100 1RB0 label (dark blue)

  - The lower carrier allocated with highest RB: 100 1RB272 label (purple)

  - The intra-band contiguous ULCA SEM: 100+100 SEM label (orange dash)

  - The lower carrier single carrier SEM: 100 SEM label (red dash).

- Figure 2 for the intra-band non-contiguous case with 60MHz lower carrier and 40MHz upper carrier separated by 100MHz (allowable case for two CC per PA as the in-gap SEM requirement is -13dBm/MHz):

  - The fully allocated lower carrier: 50 full label (cyan)

  - The lower carrier allocated with lowest RB: 60 1RB0 label (dark blue)

  - The lower carrier allocated with highest RB: 60 1RB161 label (purple)

  - The intra-band non-contiguous ULCA SEM: 50+G100+50 SEM label (orange dash)

  - The lower carrier single carrier SEM: 50 SEM label (red dash).

Figure 1: Intra-band contiguous ULCA case

Figure 2: Intra-band non-contiguous ULCA case

Observations for intra-band contiguous ULCA case:

- If the single carrier SEM requirement is used when RBs are allocated in only one CC, and assuming the 28dBc image allowance at MPR0:

  - The 1RB0 image fails the -13dBm/MHz SEM limit. Any RB allocation < 6.48MHz bandwidth would fail. Also note that the 3rd order IMDs related to the wanted signal and its image may fail the -30dBm/MHz region of the single CC SEM.

  - The fully allocated image passes due to the PSD spread.

- When the configured intra-band contiguous ULCA SEM is used, the image still falls in band of the two CCs thus there is no issue with the SEM requirement. Similarly, the 3rd order IMDs related to the wanted signal and its image fall into the -13dBm/MHz region of the CA SEM.

- Unless the LO is switched when there is allocation only one CC, the single carrier MPR is not sufficient to meet the single carrier based SEM, but using the configured contiguous ULCA based SEM works.

Observations for intra-band non-contiguous ULCA case:

- If the single carrier SEM requirement is used when RBs are allocated in only one CC, and assuming the 28dBc image allowance at MPR0: the image of any RB allocation fails the -30dBm/MHz spurious emissions.

- When the configured intra-band non-contiguous ULCA SEM is used:

  - The 1RB0 image still falls within the higher CCs thus there is no issue with the SEM requirement.

  - The image for the fully allocated case falls within the in-gap region and both fails the 60MHz single carrier SEM.

  - The image of the fully allocated cases passes the -13dBm/MHz in-gap SEM.

  - The image of the 1RB161 fails, as would any RB allocation < 6.48MHz bandwidth.

- If the LO is not switched, even when using the configured intra-band non-contiguous ULCA SEM, in the case of asymmetric CCs, some allocations would fail SEM if the single CC MPR is used.

## 2.3 Alternatives for contiguous intra-band ULCA.

In order to enable the use of the single carrier MPR, a solution shall be found for the issue with the image failing the single carrier SEM for the implementations with a single LO frequency (either single PA or 2Tx implementations that supports TxD and/or UL MIMO on top of UCCA).

One option is to always allow LO switching, but this may result in transmit interruptions. In that case, it may be worth allowing interruptions only when DualPA is not signalled or TxD or UL MIMO is signalled.

Another option is to always use the configured intra-band contiguous ULCA SEM even when only one of the carriers is allocated with transmitted RBs. On top of the benefit of not requiring LO switching, this means that all allocation can be considered as inner since the SEM and ACLR is defined for the aggregated bandwidth.

Proposal for intra-band contiguous ULCA when RBs are active in only one CC:

- For UE supporting contiguous ULCA with the dualPA IE:

  - The MPR in tables Table 6.2.2-1 and Table 6.2.2-2 apply for PC3 and PC2 respectively.

  - The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively.

  - For band specific emissions, the NS requirement applies when RBs are active in only one CC.

- For UE supporting contiguous ULCA without signalling the dualPA IE or signalling PC1.5 or TxD or UL MIMO support:

  - The MPR in tables Table 6.2.2-1, 6.2.2-2, 6.2D.2-1 and 6.2D.2-2 apply for PC3, PC2 1Tx, PC2 2Tx and PC1.5 respectively.

  - The applicable SEM, ACLR and spurious emissions are the configured contiguous ULCA CC emissions defined in Table 6.5A.2.2.1-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively.

  - For band specific emissions, the CA_NS requirement applies when RBs are active in only one CC.

- UL interruption for LO retuning is not allowed.

If the above proposal is not agreed, then UL interruptions needs to be allowed for the UEs supporting contiguous ULCA with single LO and would result in the following alternative proposal (not preferred).

Not preferred alternative proposal for intra-band contiguous ULCA when RBs are active in only one CC:

- The MPR in tables Table 6.2.2-1, 6.2.2-2, 6.2D.2-1 and 6.2D.2-2 apply for PC3, PC2 1Tx, PC2 2Tx and PC1.5 respectively.

- The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively.

- For band specific emissions, the NS requirement applies when RBs are active in only one CC.

- UL interruption for LO retuning is allowed for UE supporting contiguous ULCA without signalling the dualPA IE or UE signalling PC1.5 or TxD or UL MIMO support.

## 2.4 Alternatives for non-contiguous intra-band ULCA.

For the non-contiguous ULCA case, using the configured intra-band non-contiguous ULCA SEM does not prevent some allocation image to fail when the LO is not switched. In this case there would too much complexity to pick on which allocations would still benefit from the ULCA defined emissions and thus LO switching should be allowed but it only applies to the cases where DualPA is not signalled. In this case there is no additional benefit to use the ULCA defined emissions requirements versus the single CC defined emissions requirements, but there is no harm either.

Proposal for intra-band non-contiguous ULCA when RBs are active in only one CC:

- The MPR in tables Table 6.2.2-1, 6.2.2-2, 6.2D.2-1 and 6.2D.2-2 apply for PC3, PC2 1Tx, PC2 2Tx and PC1.5 respectively.

- The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively.

- For band specific emissions, the NS requirement applies when RBs are active in only one CC.

- UL interruption for LO retuning is allowed for UE supporting non-contiguous ULCA without signalling the dualPA IE or UE signalling PC1.5, TxD or UL MIMO support.

## 2.5 Implementation in the specification.

In way forward [2], a draft CR text was proposed below.

#### Issue 2-1-4: Draft Rel-19 CR on MPR applicability for intra-band contiguous CA with single CC with activated cell

- Proposals

  - Proposal 1: Add the following description into clause 6.2A.2.1:

    - “For intra-band contiguous carrier aggregation with single CC with activated cell, MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.”

WF: FFS the details of draft CR

In our view, the text added into clause 6.2A.2.1 (and to the similar clause for non-contiguous ULCA) should not only clarify which MPR applies with single CC with activated cell depending on power class and PA architecture but also clarify which emission requirement applies (single CC or CA) as we suggest in above proposals. Furthermore, as PC1.5 ULCA is part of the R19, it should also cover PC1.5 as we suggest in above proposals.

Proposal on text added to the ULCA MPR clauses for the case with activate RBs in only one CC:

- Text should be added to both contiguous and non-contiguous clauses in section 6.2A

- The text should clarify under which conditions the single CC MPR applies:

  - Carrier aggregation with single CC with activated cell?

  - Carrier aggregation with active RBs only in one CC?

- The text should point at which emission requirements should be met (single CC or CA based) by citing the clause or the tables numbers.

- The text should cover PC3, PC2 1Tx, PC2 2Tx and PC1.5 2Tx as the latter is part of Release 19.

- FFS if similar approach can also apply to PC5 and PC3 contiguous ULCA for NR-U.

# Conclusion

In this contribution, we examined the intra-band contiguous and non-contiguous ULCA architectures and discussed the issues related to the image of the allocated RBs failing the single carrier SEM when only one carrier is allocated, and a single LO frequency is used and made the following proposals.

Proposal for intra-band contiguous ULCA when RBs are active in only one CC:

- For UE supporting contiguous ULCA with the dualPA IE:

  - The MPR in tables Table 6.2.2-1 and Table 6.2.2-2 apply for PC3 and PC2 respectively.

  - The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively.

  - For band specific emissions, the NS requirement applies when RBs are active in only one CC.

- For UE supporting contiguous ULCA without signalling the dualPA IE or signalling PC1.5 or TxD or UL MIMO support:

  - The MPR in tables Table 6.2.2-1, 6.2.2-2, 6.2D.2-1 and 6.2D.2-2 apply for PC3, PC2 1Tx, PC2 2Tx and PC1.5 respectively.

  - The applicable SEM, ACLR and spurious emissions are the configured contiguous ULCA CC emissions defined in Table 6.5A.2.2.1-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively.

  - For band specific emissions, the CA_NS requirement applies when RBs are active in only one CC.

- UL interruption for LO retuning is not allowed.

Proposal for intra-band non-contiguous ULCA when RBs are active in only one CC:

- The MPR in tables Table 6.2.2-1, 6.2.2-2, 6.2D.2-1 and 6.2D.2-2 apply for PC3, PC2 1Tx, PC2 2Tx and PC1.5 respectively.

- The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively.

- For band specific emissions, the NS requirement applies when RBs are active in only one CC.

- UL interruption for LO retuning is allowed for UE supporting non-contiguous ULCA without signalling the dualPA IE or UE signalling PC1.5, TxD or UL MIMO support.

Proposal on text added to the ULCA MPR clauses for the case with activate RBs in only one CC:

- Text should be added to both contiguous and non-contiguous clauses in section 6.2A

- The text should clarify under which conditions the single CC MPR applies:

  - Carrier aggregation with single CC with activated cell?

  - Carrier aggregation with active RBs only in one CC?

- The text should point at which emission requirements should be met (single CC or CA based) by citing the clause or the tables numbers.

- The text should cover PC3, PC2 1Tx, PC2 2Tx and PC1.5 2Tx as the latter is part of Release 19.

- FFS if similar approach can also apply to PC5 and PC3 contiguous ULCA for NR-U.

# References

[1] R4-2411049 On improved MPR for intra-band ULCA when only one CC has RBs allocated, Skyworks Solutions Inc., RAN4#112

[2] R4-2414441 WF on power domain enhancement	, Huawei, RAN#112