3GPP TSG-RAN WG4 Meeting#112bis						         			R4-2415274

Hefei, China, 14 – 18 October 2024

Title: 	Reduced MPR by redefining allocation type based on BS signalled transmission bandwidth

Source: 	Skyworks Solutions, Inc.

Agenda Item:	6.1.1.2.1 Power domain enhancements for single carrier [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN4#112, a way forward [2] captured agreements related to the Release 19 power domain enhancement work item. Notably, it covered the MPR reduction for the case where the UE CBW is smaller than the BS CBW. In this contribution, and further developing from our contribution [1] in RAN4#112, we propose a simple method for reusing the inner/outer MPR concept and applying it to the BS signalled transmission BW.

# Discussion

## 2.1 Way forward inputs

As shown below in italics, we extracted the agreements and way forwards relative to single CC MPR reduction as captured in [2].

#### Issue 1-1-1: Approaches for scenario 1

- WF

  - Further evaluate the reduced MPR values with relaxation of ACLR/SEM to certain levels or waived

  - Other ways to reduce MPR are not precluded

#### Issue 1-2-1: Clarification of BS CBW

- Proposals

  - Proposal 1: Clarify that the BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios. (Huawei)

- WF: FFS in next meeting

#### Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation

- Proposals

  - Approach 1: UE CBW based

    - Option 1: The extended UE CBW based method as captured in the WF in last meeting.

  - Approach 2: BS CBW based

    - Option 2: BS CBW based inner/outer method. (Skyworks)

      - BS NRB instead of UE NRB.

      - Adding the amount of RB by which the UE RBs are shifted within the BS RBs to the UE RBstart.

      - The base station shall signal the extended NRB and RBstart shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR.

      - For margin or stricter emissions, the BS can signal a reduced NRB and RBshift.

    - Option 2a: the feature ‘narrower UE channel BW within wider BS bandwidth’ is specified for a UE configured with a UE-specific CHBW within a wider BS bandwidth, the said UE compliant with selected OOBE requirements applicable for the wider BS bandwidth (the cell bandwidth). (Ericsson, Nokia)

- WF

  - Focused on extended UE CBW on common issues, which could also be applied for BS CBW based approach

#### Issue 1-2-3: whether extended UE CBW edge could exceed the BS CBW edge

- Proposals

  - Proposal 1: The extended UE CBW should not exceed the BS CBW. (Samsung, Sony, vivo, MTK, LGE)

  - Proposal 2: One is that there is no adjacent in-band/out-of-band co-existence issue, in which case, it is feasible that extended UE CBW edge exceeds the BS CBW edge. The other is that there exists adjacent in-band/out-of-band co-existence issue, and extended UE CBW edge cannot exceed the BS CBW edge. (ZTE)

- WF

  - proposal 1 is agreeable for singe CC case, FFS multi-CC/BS transmitted BW case

#### Issue 1-2-4: Whether and where to use IBE in the larger BS CBW

- WF

  - Focus on how to extend IBE for usage between edges of UE CBW and extended UE CBW

#### Issue 1-2-5: Boundary to apply ACLR and SEM

- Proposals

  - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. (Samsung, Sony, ZTE, vivo, LGE)

  - Proposal 2: ACLR, SEM and spurious emissions would be defined as of today but based on BS channel bandwidth. (Nokia, Ericsson)

- WF

  - FFS in next meeting

#### Issue 1-2-6: Boundary to apply SE

- WF

  - [SE applicability depends on the extended CBW]

#### Issue 1-2-7: Which CBW is utilized as the basis for the integral region of OOBE

- Proposals

  - Option 1: The integral region and the boundary of OOBE should be based on UE CBW. (Samsung, vivo, ZTE, China Telecom)

  - Option 2: The integral region and boundary of OOBE is based on extended UE CBW. (Sony)

- WF

  - FFS in next meeting

#### Issue 1-2-8: Ratio size of extended CBW between UE CBW and larger BS channel BW

- Proposals

  - Proposal 1: (Apple)

    - If the goal is to convert all outer RB allocations into inner RB allocations, then the minimum BS channel needs to be at least twice the size of the UE channel.

    - If the sole goal is to obtain output power improvements, then it is possible to have a more convenient ratio between UE and BS channel such as two-third.

  - Proposal 2: (Qualcomm)

    - If minimum excess BW equal to half of the original UE BW can be placed on both sides of the original UE BW this extended UE BW can be treated as the new UE BW and all performance metrics can be based on this new BW.

    - If the minimum extended BW on both sides of original UE BW is less than half the original UE BW then the original UE BW should not be extended, and all performance metrics should be based on the original UE BW.

  - Proposal 3: The required extended bandwidth of the both sides could be set to 2/5 UE CBW as a starting point. (vivo)

  - Proposal 4: Consider extended UE CBW approach to improve MPR starting from 1.5*CBW (OPPO)

  - Proposal 5: (Huawei)

    - For the shifted start point of ΔfOOB, 1/2 UE CBW can be the default value for UE indication.

    - Consider to introduce shorter shifted frequency e.g. 1/3, 1/4 UE CBW in conjunction with the requirement that will be introduced to the shifted frequency.

      - For instance, the modified general part of IBE requirements which is further relaxed by replacing the EVM corresponding to the scheduling modulation order with the one for BPSK

- WF: FFS with consideration of above proposals in next meeting

#### Issue 1-2-9: Whether and how to consider the Asymmetrical extended CBW approach

- WF: Asymmetrical extended CBW approach can be considered, FFS the details

#### Issue 1-2-11: Signaling aspects

- Proposals

  - Proposal 1: no changes of signalling are specified for the feature ‘narrower UE channel BW within wider BS bandwidth’ except possibly specification of a capability bit to indicate support of the feature. (Ericsson)

  - Proposal 2: Need to consider capability for extended UE CBW procedure. (LGE)

  - Proposal 3: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per band capability. (Sony)

  - Proposal 4: The frequency intervals for the UE CBW and the BS CBW should be judged by the NW and the indication of MPR reduction could be directly sent to the UE when the corresponding conditions are met. (vivo)

  - Proposal 5: Establish a start point and continue to details for BS indication with the deepen in MPR reduction mechanism. (CTC)

- WF: To discuss the signalling aspects after sufficient evaluation of power boosting and/or MPR reduction in terms of relaxed requirements.

In this contribution, we discuss a simple method to achieve MPR reduction by reusing the inner/outer concept and applying it to the BS advertised BW, rather than the UE CBW. This covers issues 1-2-1 to 1-2-11. For issue 1-1-1 we believe that this can also be covered by our proposal.

## 2.2 Redefining inner/outer/edge allocations based on advertised BS BW

For the QPSK and 16QAM, which are the main candidates for reduced MPR as they are not limited by EVM (while EVM for 64 QAM may become marginal if MPR is reduced), the inner/outer/edge aspects have different limitations:

- Inner QPSK and 16QAM are IBE limited

- Outer QPSK and 16QAM are SEM or ACLR limited.

- Edge allocations are limited by SEM and related to the windowing filtering done at the BB.

- For 64QAM and 256QAM, all allocations are EVM limited. Thus, only edge allocations may experience an improved MPR.

When the spectrum available at the BS is larger than the UE CBW, then essentially the SEM and ACLR should apply based on that BS BW, as the interference with RBs within the BS spectrum but outside the UE CBW should be guaranteed by respecting IBE, which both inner and outer allocations MPR guarantees.

A simple method to improve the MPR for a large number of allocations is to re-assess the inner, outer, and edge boundaries based on the BS advertised CBW as illustrated in Figure 1:

- On the left side, the classical allocation triangle is based on the UE CBW where the inner allocations (green) region has LCRB limited to half the UE CBW and their position is at half the allocation (LCRB) size from the channel edge; other allocations are outer (yellow), except for PC2 and PC1.5 where edge (red) allocations are needed.

- On the right side, the allocation triangle is now based on the BS CBW and a new UE allocation triangle can be defined by shifting the UE based triangle within the BS defined inner, outer, and edge regions by its RB0 position within the BS RBs.

Figure 1: Illustration of UE based, and BS based allocation triangle defining inner, outer and edge allocations. BS CBW < 2xUE CBW

Observations on BS based allocation triangle in Figure 1:

- Since the BS CBW < 2xUE CBW and UE CBW is not centred within the BS CBW, there are still some outer allocations left.

- The inner allocation LCRB can now reach half of the BS CBW (if the UE CBW is > ½ BS CBW).

- For many LCRB values, the allocations “become” inner whatever their position. There are no more edge allocations and the outer region is very small.

- This definition can apply to any modulation order or waveform type and thus:

  - Reduced MPR or power boosting is achievable for many BPSK (w/wo shaping), QPSK and 16QAM edge or outer allocations that “become” inner for both DFT-s-OFDM and CP-OFDM.

  - For 64QAM and 256QAM only edge allocations may see reduced MPR as the MPR is EVM limited for inner and outer allocations.

Note that in order to “turn” all UE edge or outer allocations to inner, the BS CBW needs to be twice the UE CBW but also the UE RBs need to be centred within the BS CBW, this case if illustrated in Figure 2 and Figure 3 shows the case where BS CBW is 2xUE CBW but the UE CBW is not centred within the BS CBW.

Figure 2: Illustration of UE based, and BS based allocation triangle defining inner, outer and edge allocations. BS CBW = 2xUE CBW and UE CBW centered within BS CBW (RB0shift=BS/NRB/4)

Observations on BS based allocation triangle in Figure2 above: Since the BS CBW = 2xUE CBW and UE CBW is centred within the BS CBW, there are no outer allocations left.

Observations on BS based allocation triangle in Figure 3 below: BS CBW = 2xUE CBW but UE CBW is not centred within the BS CBW, thus there are still some outer allocations left.

Figure 3: Illustration of UE based, and BS based allocation triangle defining inner, outer and edge allocations. BS CBW = 2xUE CBW and UE CBW not centered within BS CBW

In our view it is far too restrictive to limit to the cases where the BS CBW is >2xUE CBW and UE CBW is shifted by at least BS NRB/4 to turn all UE allocations to inner for most deployment to benefit from this improved MPR. Thus, we believe that any allocation that has a chance to be modified from edge/outer to inner MPR should be covered, which the method described above that reuses existing mechanism, equations and specified MPR can do.

This change can simply be made by the UE by recalculating the inner, outer, and edge regions using the following parameter that will be advertised by the BS in the inner and edge equations:

- UE_NRB is replaced by BS_NRB (note that this NRB may not correspond to the BS CBW NRB)

- UE RBstart is replaced by RBstart+RB0shift (shifted RBstart0)

- UE LCRB is kept

- For the UE to autonomously calculate this, it only needs to get from the BS the BS_NRB and RB0shift.

Proposal on mechanism to redefine allocation type based on advertised BS transmission BW:

- A UE capability is introduced for UEs that can re-assess the inner, outer and edge allocation definition based on existing equations for single CC MPR but based on BS advertising a larger transmission BW than the UE CBW with:

- Using BS NRB (BS_NRB) instead of UE NRB.

- Adding the amount of RB by which the UE RB0 is shifted (RB0shift) within the BS RBs to the UE RBstart value.

- The base station shall signal the extended NRB (BS_NRB) and RB0shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR/boost power for allocations that have changed their type.

Proposal on inner/outer/edge allocation equations: For UEs supporting this capability, inner/outer/edge allocations equations in section 6.2.2 are modified in the TS based on BS_NRB and RB0shift parameters signalled by the BS in the following way:

- Replace NRB by BS_NRB in inner allocation equations.

- Replace RBstart by RBstart+RB0shift in inner allocation equations and for edge allocations.

The examples shown in Figures 1, 2 and 3 shows the case where the advertised BS transmission BW (BS_NRB and RB0shift) corresponds to the BS channel bandwidth. However, this is an unnecessary limitation for what the BS could advertise for which the UE could recalculate the inner/outer/edge allocation types in some use cases:

- There are use cases where the advertised BS_NRB transmission BW and UE RB0shift may be smaller than whose that would be defined based on the BS CBW:

  - For bands with additional emission requirement (NS), in order to make sure the emissions of the UE reach the right level at the NS specified offset for OOB emissions.

  - For bands with a nearby protected band, in order to make sure that the UE meets the spurious emission limit within the protected band.

  - An alternative solution is that the UE is aware of such use cases based on NS and band protection requirements and then it restricts the allocations that are “turned” to inner to make sure additional NS OOB emissions or band protection levels are met. This may not be optimal though, compared to the BS reducing its advertised BS_NRB and RB0shift

- There are use cases where the advertised BS_NRB transmission BW and UE RB0shift may be larger than whose that would be defined based on the BS CBW:

  - For bands where the UE CC is part of a BS supporting contiguous ULCA, in order to cover the contiguous ULCA aggregated BW and Foob based on CA BW.

  - For bands where the UE CC is part of a BS with collocated adjacent channels that are FFT aligned in a tower sharing coordinated deployment

  - In these cases, the UE cannot determine alone the deployment scenario and thus the advertised BS_NRB and RB0shift needs to be larger than whose that would be defined based on the BS CBW

We think it is too early to analyse all the use cases where the advertised BS_NRB and RB0shift may be smaller or larger than those derived from the BS_CBW or even discuss the UE behavior where the UE would not fully use the advertised BS_NRB and RB0shift and leave that for further meetings. However, we believe it is essential that the BS signalling is future proof and allows to at least signal up to twice the maximum UE and BS CBW.

To make all UE allocation inner allocations, when this is applicable, it is sufficient that the BS advertises a BS_NRB=2xUE_NRB and an RB0shift ≥ NRB_UE/2.

- With the current maximum NRB of 273, means that BS_NRBmax=546 and RB0shift=137. However, RB0shift may be closer to BS_NRB when UE CBW is smaller than the BS upper CBW in a contiguous ULCA case.

- With this information, it seems sufficient to code advertised BS transmission BW (BS_NRB) and UE RB0 shift (RB0shift) on 10bits.

Proposal on advertised BS transmission BW signalling:

- To enable more use cases to be covered in the future, the BS advertised transmission BW may be smaller than, equal to or larger than the BS CBW, depending on where the ACLR/SEM and spurious emission limits need to be met.

- It is advertised via:

  - BS_NRB parameter that can be up to 566 as it is limited to twice the largest UE CBW.

  - RB0shift parameter to signal where the UE RBs are placed within the BS RBs, this parameter should be able to place any UE CBW within twice the largest UE CBW.

  - Where 10bit seems sufficient to code both parameters.

In our view, since at best there is 1dB back-off (difference between inner and outer in most cases) to be gained, we do not see much difference between relaxing ACLR and SEM using the UE CBW versus applying existing ACLR and SEM but at for a bandwidth larger than the UE bandwidth as intrinsically in “turns” outer allocations into inner allocations.

Proposal on scenario 1 (reduced MPR with relaxed ACLR/SEM):

- Given the 1dB differences between the inner and outer MPR, it is of little benefit to define MPR for relaxed values of ACLR or SEM, and instead we should focus on mechanism that can turn outer allocations into inner allocations.

- Relaxation of SEM values may not be possible anyhow as it is part of the regulations. However, it should be feasible to define it for different frequency offsets based on the BS transmission BW in the deployment.

- The proposed BS advertised transmission BW mechanism can be used for this scenario as it permits a relaxed ACLR and SEM at some offsets from the UE channel edges.

# Conclusion

In this contribution, we extended the inner, outer and edge allocation triangle concept used for the UE MPR by using the BS parameters instead and make the following proposal.

Proposal on mechanism to redefine allocation type based on advertised BS transmission BW:

- A UE capability is introduced for UEs that can re-assess the inner, outer and edge allocation definition based on existing equations for single CC MPR but based on BS advertising a larger transmission BW than the UE CBW with:

- Using BS NRB (BS_NRB) instead of UE NRB.

- Adding the amount of RB by which the UE RB0 is shifted (RB0shift) within the BS RBs to the UE RBstart value.

- The base station shall signal the extended NRB (BS_NRB) and RB0shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR/boost power for allocations that have changed their type.

Proposal on inner/outer/edge allocation equations: For UEs supporting this capability, inner/outer/edge allocations equations in section 6.2.2 are modified in the TS based on BS_NRB and RB0shift parameters signalled by the BS in the following way:

- Replace NRB by BS_NRB in inner allocation equations.

- Replace RBstart by RBstart+RB0shift in inner allocation equations and for edge allocations.

Proposal on advertised BS transmission BW signalling:

- To enable more use cases to be covered in the future, the BS advertised transmission BW may be smaller than, equal to or larger than the BS CBW, depending on where the ACLR/SEM and spurious emission limits need to be met.

- It is advertised via:

  - BS_NRB parameter that can be up to 566 as it is limited to twice the largest UE CBW.

  - RB0shift parameter to signal where the UE RBs are placed within the BS RBs, this parameter should be able to place any UE CBW within twice the largest UE CBW.

  - Where 10bit seems sufficient to code both parameters.

Proposal on scenario 1 (reduced MPR with relaxed ACLR/SEM):

- Given the 1dB differences between the inner and outer MPR, it is of little benefit to define MPR for relaxed values of ACLR or SEM, and instead we should focus on mechanism that can turn outer allocations into inner allocations.

- Relaxation of SEM values may not be possible anyhow as it is part of the regulations. However, it should be feasible to define it for different frequency offsets based on the BS transmission BW in the deployment.

- The proposed BS advertised transmission BW mechanism can be used for this scenario as it permits a relaxed ACLR and SEM at some offsets from the UE channel edges.

# References

[1] R4-2412579 On reduced MPR when BS CBW is larger than UE CBW, Skyworks Solutions Inc., RAN4#112

[2] R4-2414441 WF on power domain enhancement, Huawei, RAN4#112