3GPP TSG-RAN WG4 Meeting#112bis							         		R4-2415275

Hefei, China, 14 – 18 October 2024

Title: 	On PC1.5 ULCA PCmax and UE behavior under large PSD imbalance

Source: 	Skyworks Solutions, Inc.

Agenda Item:	6.1.1.1.2 Intra-band contiguous and non-contiguous UL CA with PC1.5 [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#112, we discussed the emissions issues related to PSD imbalance for ULCA in [1] and our proposal to evaluate MPR for up to 6dB PSD imbalance was captured in way forward [2]. Also, our inputs on PCmax limitations in earlier meetings is discussed in [2]. In this contribution, we discuss how PCmax limitations are captured for dualPA architecture. In addition, we further propose that the MPR accounts for up to 6dB PSD imbalance and discuss what UE behavior may be expected for larger PSD imbalance.

# Discussion

## 2.1 Way forward inputs

The way forward [2] captured our input on PSD imbalance in [1] in the following agreement in italic.

Online agreement:

- MPR and CANS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on TxD architecture and may account for PSD imbalance with up to 6dB.

  - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB

- PC1.5 contiguous intra-band ULCA based on dualPA architecture with two LOs is not specified in R19

- MPR studies for PC1.5 non-contiguous intra-band ULCA focusses on dualPA architecture with two LOs and may account for PSD imbalance with up to 6dB.

  - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB

- PC1.5 non-contiguous intra-band ULCA based on TxD architecture is not specified in R19

Furthermore, it also discussed ULCA PCmax aspects similar to what was done in previous meetings within the following agreements in italic.

Adhoc agreement:

- For intra-band UL NC CA with dualPA-architecture, the upper bound of Pcmax,c is 26dBm per CC

  - Whether or how to reflect the above agreement in spec is FFS

- For intra-band UL contiguous CA with TxD, the upper bound of Pcmax,c is 29dBm per CC

  - Whether or how to reflect the above agreement in spec is FFS

Online agreement with adhoc modification:

- For intra-band contiguous ULCA with 2Tx-TxD, the upper bound of Pcmax is 29dBm

- For intra-band UL NC CA with dual PA architecture, the upper bound of Pcmax is 29dBm

In this contribution, we first discuss the UE behavior when large PSD imbalance occurs and what the specified MPR should account for. We then discuss the PCmax aspects.

## 2.2 UE ULCA behavior under large PSD imbalance.

In [1], after showing that allowing large PSD difference between the CC will result in additional back-off that will affect the CC with the largest allocated bandwidth, it was proposed and captured in way forward [2] to evaluate MPR for equal PSD but also for 6dB PSD difference.

The preliminary measurement results and differences in back-off between equal PSD and 6dB PSD differences will be discussed in separate papers for intra-band contiguous ULCA [3] and intra-band non-contiguous ULCA [4] in this meeting. However, 6dB PSD imbalance is already agreed as the maximum PSD imbalance in DL between two CCs received by the same receiver chain for which the REFSENS, blocking, ACS requirements are specified. Beyond this level of imbalance, the UE receive performance is basically unknown.

Although it may be feasible that the BS schedules the two CCs with larger imbalance, it seems reasonable that the specified MPR accounts for up to 6dB PSD imbalance and still meets emissions.

Assuming MPR accounts for up to 6dB PSD imbalances, the question is what happens for higher PSD imbalances? First, if the MPR already supports 6dB imbalance, the fact that the power class comes with a +2/-3dB range to cope with process, voltage, frequency and temperature variations, gives some headroom at nominal conditions. Furthermore, this tolerance increases with larger backoffs, providing more headroom for allocations requiring higher MPR. That being said, in extreme cases, the UE may not be able to meet its emissions requirements for PSD imbalances >6dB and thus should be allowed to drop the SCC in such case, and that should be doable by the UE autonomously.

It should be noted anyhow that PSD imbalances >6dB will anyhow impair the transmission beyond the potential emission level failure in the contiguous ULCA case as, for the CC with lower PSD, the SNR is affected and thus the EVM and potentially IBE will be degraded.

If for PC1.5, the 6dB imbalance will be evaluated (at least by us), it has not been the case for lower power classes where the evaluation always used equal PSD. Since we have been involved in all the ULCA studies since Release 16 for NR and NR-U, we believe that the margin available from the MPR values together with the power class and power steps range should safely allow 6dB PSD imbalance.

Proposal on PSD imbalance:

- PC1.5 contiguous and non-contiguous ULCA MPR accounts for up to 6dB PSD imbalance

- In extreme cases when PSD imbalance > 6dB may cause the UE to fail the required emission levels, the UE is allowed to drop the SCC.

## 2.3 PCmax for PC1.5 ULCA.

In way forward [2], two PCmax definitions are discussed together with their limitations depending on PA architecture.

We first want to reformulate the agreements with a more exact description of the PA architecture:

For intra-band contiguous and non-contiguous ULCA the PC1.5 upper bound is.

- For architectures based on DualPA: 26dBm for PCmax,c and 29dBm for PCmax with limitations for cases with bandwidth imbalance

- For architectures based on 2Tx: 26dBm for PCmax,c and 29dBm for PCmax

Since in Release 19:

- Contiguous ULCA is only specified for 2Tx approach, PCmax=PCmax,C=29dBm

- Non-contiguous ULC ULCA is only specified for DualPA approach, PCmax=29dBm-BW imbalance term and PCmax,C=26dBm

Although accurate, we are not sure that introducing PCmax,c is of interest since ULCA MPR applies to PCmax,ca. In other words, the MPR is applied equally to each CC and thus to PCmax,c and PCmax,ca altogether.

If we recognize that for the DualPA case, PCmax,c is limited to 26dBm for PC1.5, this is the only case for which it happens. On top of that limitation, there are also limitations to the maximum power of the other CC due to allocated BW differences in each CC. We believe it is better to only discuss the CA PCmax equations and embed the PC1.5 DualPA limitations in the ΔPPowerClass,CA value for PC1.5.

In order to formulate the limitations properly, and in accordance with the proposal in the previous chapter, a PSD imbalance of up to 6dB should be accounted for.

For the PC1.5 contiguous ULCA case with UE NOT signalling DualPA or signalling TxD or UL MIMO, ΔPPowerClass,CA = 6 dB since 29dBm total power or power per CC is achievable regardless of allocated RB BW per CC or PSD imbalance.

For the PC1.5 non-contiguous ULCA case with UE signalling DualPA, ΔPPowerClass,CA can be lower than 6 dB when the allocated BW is different in each CC and PSD imbalance is kept <6dB as one CC cannot exceed 26dBm.

Table 1 provides the calculated per CC power, total power and ΔPPowerClass,CA for different allocations cases and for equal PSD and 6dB PSD imbalance. The calculation are made assuming 26dBm on the CC with the largest allocated RB BW and other CC power is deducted from equal PSD or equal PSD+6dB.

Table 1: Power per CC, Total power and ΔPPowerClass,CA vs RB ratio for equal PSD and 6dB PSD imbalance.

| CC1 RB | RB1 | 273 | 273 | 273 | 273 | 273 | 273 | 273 | 273 | 273 | 200 | 200 | 200 | 200 | 200 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CC2 RB | RB2 | 273 | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | 200 | 100 | 50 | 20 | 1 |
| PCC1 | 26dBm | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
| PCC2 equal PSD | 26-10*log(RB1/RB2) | 26 | 22.7 | 19.7 | 16.7 | 13.7 | 10.7 | 7.7 | 4.6 | 1.6 | 26 | 23 | 20 | 16 | 3 |
| PCC2 6dB PSD imb. | Min(26,32-10*log(RB1/RB2)) | 26 | 26 | 25.7 | 22.7 | 19.7 | 16.7 | 13.7 | 10.6 | 7.6 | 26 | 26 | 26 | 22 | 9 |
| Ptotal equal PSD | 10*log(10^(PCC1/10)+10^(PCC2/10)) | 29 | 27.7 | 26.9 | 26.5 | 26.2 | 26.1 | 26.1 | 26 | 26 | 29 | 27.8 | 27 | 26.4 | 26 |
| Ptotal 6dB PSD imb. | 10*log(10^(PCC1/10)+10^(PCC2/10)) | 29 | 29 | 28.9 | 27.7 | 26.9 | 26.5 | 26.2 | 26.1 | 26.1 | 29 | 29 | 29 | 27.5 | 26.1 |
| ΔPPowerClass,CA equal PSD | Ptotal-23 | 6 | 4.7 | 3.9 | 3.5 | 3.2 | 3.1 | 3.1 | 3 | 3 | 6 | 4.8 | 4 | 3.4 | 3 |
| ΔPPowerClass,CA 6dB PSD imb. | Ptotal-23 | 6 | 6 | 5.9 | 4.7 | 3.9 | 3.5 | 3.2 | 3.1 | 3.1 | 6 | 6 | 6 | 4.5 | 3.1 |

Observations:

- When the allocation BW is the same in each CC equal PSD is equivalent to equal power and 29dBm total power can be reached

- When 6dB PSD imbalance is allowed, equal power and 29dBm can be achieved for allocated RB BW ratios between the two CC that are ≤ 4

After some further calculations and simplifications, the total power for equal PSD or 6dB imbalance can be calculated with following equations where RB1 ≥ RB2:

Ptotal(equalPSD) [dBm] = 26+10*log(1+RB2/RB1)

Ptotal(6dB imbalance) [dBm] = min{29,26+10*log(1+4*RB2/RB1)}

Further, applying this to ΔPPowerClass,CA and making it generic for any allocation ratio and allowing different SCS in each CC, the following equations are provided:

ΔPPowerClass,CA (equalPSD) [dBm] = 3+10*log(1+min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))

ΔPPowerClass,CA (6dB imbalance) [dBm] = min{6,3+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))}

Where RB1 and RB2 are the number of active RBs in CC1 and CC2 respectively and SCS1 and SCS2 are the SCS used in CC1 and CC2 respectively.

Based on the above and allowing up to 6dB PSD imbalance we make the following proposal to amend the ULCA PCmax equations for PC1.5 ULCA.

Proposal on PC1.5 PCmax accounting for DualPA architecture limitations and allowing up to 6dB PSD imbalance:

For a UE supporting PC1.5 intra-band contiguous or non-contiguous ULCA and:

- Not signalling DualPA IE,

  - ΔPPowerClass,CA = 6 dB

- Signalling DualPA IE,

  - ΔPPowerClass,CA = min{6, 3+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dB

  - Where RB1 and RB2 are the number of active RBs in CC1 and CC2 respectively and SCS1 and SCS2 are the SCS used in CC1 and CC2 respectively.

# Conclusion

In this contribution, based on our previous work on the topic and further extending calculations to allow up to 6dB PSD imbalance, we propose the following for the PC1.5 intra-band ULCA power class and power reduction.

Proposal on PSD imbalance:

- PC1.5 contiguous and non-contiguous ULCA MPR accounts for up to 6dB PSD imbalance

- In extreme cases when PSD imbalance > 6dB may cause the UE to fail the required emission levels, the UE is allowed to drop the SCC.

Proposal on PC1.5 PCmax accounting for DualPA architecture limitations and allowing up to 6dB PSD imbalance:

For a UE supporting PC1.5 intra-band contiguous or non-contiguous ULCA and:

- Not signalling DualPA IE,

  - ΔPPowerClass,CA = 6 dB

- Signalling DualPA IE,

  - ΔPPowerClass,CA = min{6, 3+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dB

  - Where RB1 and RB2 are the number of active RBs in CC1 and CC2 respectively and SCS1 and SCS2 are the SCS used in CC1 and CC2 respectively.

# References

[1] R4-2411302 On equal PSD vs equal power spectral regrowth, Skyworks Solutions Inc., RAN4#112

[2] R4-2414277 WF on HPUE for CA in TN, Samsung, RAN4#112

[3] R4-2415644 Preliminary input on 2Tx PC1.5 contiguous ULCA MPR based on measurements, Skyworks Solutions Inc., RAN4#112b

[4] R4-2415645 Preliminary input on DualPA PC1.5 non-contiguous ULCA MPR based on measurements, Skyworks Solutions Inc., RAN4#112b