3GPP TSG-RAN WG4 Meeting # 112bis								    R4-2415276

Hefei, China, 14th – 18th October 2024

Agenda Item: 6.9.2

Source: Sharp

Title: Discussions on VSAT parameters for Ku Band coexistence testing

Document for: Discussion

# 1	Introduction

In RP#104, a new WI [1] for Ku band was approved with objectives asking RAN4 to discuss the system parameters, including the specification of RF requirements for satellite access node and NTN VSAT types. The relevant objectives are captured as below.

- Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band.

In this contribution we provide a set of VSAT RF parameters and associated values to be considered for the Ku band coexistence study specifically for Maritime Mobile application.

# 2	Definition of VSAT

VSATs and ESIMs are essentially the same. The Annex provides CEPT and ETSI references on the earth station in motion (ESIM) use in the Ku-band and ITU Definitions.

# 3	Maritime Mobile VSAT

One representative use case of a mobile VSAT communication terminal is a maritime terminal that can potentially use the communication services on GEO and LEO (low orbit) satellites and is mounted on a vessel. Legacy units for this use case may use a gyroscopic mount and mechanism that enables the antenna to track a desired satellite, while a phase array type unit may use electronically steered beams to track a desired satellite. Both units may provide connectivity to NGSO Satellite constellations.

Some of the use cases of the Maritime Mobile VSAT is as follows:

- Data connectivity to cruise liners and yachts

- Data connectivity to cargo and container ships

- Data connectivity to Marine vessels, fishing boat /trawlers and rescue boats

- Fleet monitoring and tracking

- IoT Connectivity to containers on cargo/container ships

Following illustrates a modern electronically steered Maritime Mobile VSAT [2].

# 4		Technical factors

Table 1 shows Sharp’s proposal on parameters and associated values which may be used for the Ku Band standardization discussions for VSAT connectivity to NGSO satellites. As shown in Table 1, we provide two set of simulation parameters for boresight and beam tilt where Rx/Tx antenna gains and G/T have respective values.

Table 1. VSAT parameters

| Characteristics | Unit | Values for boresight | Values for beam tilt relative to +/-55 deg Zenith |
| --- | --- | --- | --- |
| Frequency band |  | Ku band |  |
| Uplink Frequency | GHz | 14-14.5 |  |
| Downlink Frequency | GHz | 10.7-12.75 |  |
| Antenna type and configuration |  | Directional |  |
| Polarization |  | Circular |  |
| Field of View: Azimuth | deg | ±180 |  |
| Field of View: Zenith (+/-) | deg | ±55 |  |
| size | cm | 50 x 50 |  |
| Rx Antenna gain | dBi | 30 | 26.5 |
| Antenna temperature | K | 35 |  |
| Noise figure | dB | 3.5 |  |
| System temperature | K | 394 |  |
| G/T (Gain over Noise Temp) | dB/K | 4 | 1.5 |
| TX antenna gain | dBi | 30 | 26.5 |
| Tx power | dBm | 36 |  |
| EIRP | dBm | 66 | 62.5 |
| Operating Temp range | C | From -40 to +55 |  |
| Max Roll rate | Deg/sec | 10 Deg/sec max. |  |
| Max Pitch rate | Deg/sec | 10 Deg/sec max. |  |
| Max Yaw rate | Deg/sec | 3 Deg/sec max. |  |
| Max speed over ground | m/s | 15 m/s (29.1 Knots) |  |
| Beam Switch rate | μSEC | 500 μSEC Max. |  |

# 5	Discussions

In this contribution a set of VSAT parameters are provided and their associated values as derived from a device developed for commercial deployment of approximately 50cm x 50cm.

Proposal: We propose that RAN4 consider these antenna parameters and values for Ku Band standardization discussion.

# 6	Summary

In this paper, we provide our views on the VSAT parameters and values as may be used for Ku band coexistence testing. We have the following proposal.

Proposal : We propose that RAN4 consider these antenna parameters and values for Ku Band standardization discussion.

# 7	References

RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

https://k-tai.watch.impress.co.jp/docs/news/1624362.html

# Annex

## Regulatory aspects of the ESIMs

The following lists the CEPT and ETSI references on the ESIMs in the Ku band

| ECC Decision | Satellite | Frequency band | Type of ESIM | ETSI Standard |
| --- | --- | --- | --- | --- |
| ECC/DEC/(05)10 | GSO | 14-14.5 GHz (E2s) 10.7-11.7 GHz (s2E) 12.5-12.75 GHz (s2E) | Maritime | EN 302 340 |
| ECC/DEC/(05)11 | GSO | 14-14.5 GHz (E2s) 10.7-11.7 GHz (s2E) 12.5-12.75 GHz (s2E) | Aeronautical | EN 302 186 |
| ECC/DEC/(18)04 | GSO | 14-14.5 GHz (E2s) 10.7-12.75 GHz (s2E) | Land | EN 302 448 (Trains) EN 302 977 (Véhicule-Mounet) |
| ECC/DEC/(18)05 | non-GSO | 14-14.5 GHz (E2s) 10.7-12.75 GHz (s2E) | Maritime, aeronautical, land | EN 303 980 EN 303 981(Wide Band) |
| ECC/DEC/(19)04 | GSO & non-GSO | 12.75-13.25 GHz (E2s) 10.7-12.75 GHz (s2E) | Aeronautical | EN 302 186 (GSO only) |

Legend: E2s: Earth to Space; s2E: Space to Earth

## ITU Definition of VSAT and ESIM

Ubiquitous satellite user terminals consist of both VSAT and ESIM, operating in both GSO and NGSO satellite systems.

| VSAT | - Generally used for connectivity at fixed locations - Preferred means to easily establish reliable communications in remote areas (also for backhaul purposes) and during emergencies (e.g. humanitarian aid and relief efforts in disaster zones) |
| --- | --- |
| ESIM | - Installed on aeronautical (A-ESIM), maritime (M-ESIM) or land (L-ESIM) mobile platforms to provide seamless connectivity worldwide for passengers, crew members and operators. - Other acronyms also exist at ITU, e.g. - Earth station on board vessels (ESVs) equivalent to M-ESIM, see for example ITU-R Resolution 902 |