3GPP TSG-RAN WG4 Meeting  #112bis                                                                 R4-2415277

Hefei, China, October 14th – 18th, 2024

Agenda Item:	6.12.2.1

Source: 	Meta

Title:  	Discussion on OTA Test for XR Devices

Document for:	Discussion and Decision

# Introduction

In RAN4 #112 meeting,  XR devices test scenarios and configurations are discussed and the WF is as follows [1]:

### Sub-topic 2-1 XR test scenarios and configurations Issue 2-1-1: Discussions on Reply LS from CTIA on XR OTA test phantom Agreements: - Two types of XR, i.e., glasses and head mounted display are prioritized in this WI. - Need to send this two XR types to CTIA for phantom positioning and MU study. Issue 2-1-4: Whether WI should focus on the XR devices those connected to gNB directly Agreements: - Only test XR devices with 5G link to gNB in networks, not those with side link or wired connections. Issue 2-1-6: Prioritize 1Tx XR devices? Agreements: - Prioritize 1Tx XR Issue 2-1-7: How to identify 2Rx XR devices? Agreements: - Consider at least the following approach to identify 2Rx XR device - Use UE capability supportOf2RxXR-r18 to identify XR devices with 2Rx. - FFS device can set IE correctly - UE declaration as a backup approach Issue 2-1-8: TRP TRS requirements for XR Agreements: - Performance metric for XR OTA can be discussed, but Requirements work discussion can be postponed until device and phantom are available. Sub-topic 2-2 Testing time reduction for XR OTA Issue 2-2-1: Coarse measurement grids Agreements: The following Options can be further discussed next meeting: Option 1: Stop the measurement in the middle and charge the battery, carefully re-place the XR device into the Chamber to continue the test. Option 2: Test TRP at [X] dB below the maximum transmit power and scale up [X] dB to obtain TRP at maximum transmit power. Options 3: - Measure TRP: Averaging the measurement results from 4 of interpolated 62-points measurement grids by TRPave=10log⁡(10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104) TRSave=10log4/(110TRS110+110TRS210+110TRS310+110TRS410 to achieve the same accuracy level of using 266-points measurement grids during OTA tests. (Define 4 of 62-points interpolated measurement grids based on 266-points measurement grids at the same time, the locations of each four of 62-points interpolated measurement grids is different from each other on the sphere, they can be considered orthogonal to each other.) - Measure Peak EIPR: Full sphere coarse girds measurement + partial fine grids measurement (For example, using 62-points measurement grids in OTA test to measure peak EIPR first, then based on the direction of peak EIPR, apply corresponding sectors (pre-defined 8 or 16 sectors) of 266-points measurement grids to measure the peak EIPR again to increase the accuracy of measured peak EIRP)

In this paper, we discuss our views on XR OTA tests.

# Discussion

XR devices come in various form factors, as depicted in Figure 1. These form factors can differ in terms of size, processing capabilities, use scenarios, communication types, computing architectures, power consumption, and thermal limits.

Figure 1. XR Device Types and Form Factors

In [1], two types of XR devices, namely glasses and head-mounted displays, are prioritized in the WI. Due to the differences in form factors, the OTA test methodology, performance metrics, and performance requirements may also differ. Therefore, it is proposed that these two types of XR devices should have separate performance metrics and performance requirements.

Proposal 1: The two types of XR devices, namely glasses and head-mounted displays, should have separate performance metrics and performance requirements.

Regarding OTA testing time reduction for XR devices, three options were discussed. Option 1 is most straightforward. In Option 2, testing TRP at [X] dB below the maximum transmit power and scale up [X] dB to obtain TRP at maximum transmit power might not result in the desirable results as the transmit power may affect the RF and antenna properties and performance. In Option 3, the OTA testing is divided into sub-tests. This approach requires well-defined test methodology and verification of the effectiveness. Besides the three options, another option is to consider a coarse measurement grid. As this was studied before, it can be a good candidate for testing time reduction.

Proposal 2: For OTA testing time reduction for XR devices, prioritize the study of using a coarse measurement grid.

# Conclusion

In this paper, we discuss our views on XR OTA tests.

Proposal 1: The two types of XR devices, namely glasses and head-mounted displays, should have separate performance metrics and performance requirements.

Proposal 2: For OTA testing time reduction for XR devices, prioritize the study of using a coarse measurement grid.

# References

[1] R4-2413603, “WF for [112][333] TRP_TRS_MIMO_OTA”
[2] TR26.928, “Extended Reality (XR) in 5G”

[3] S4-231856, MeCAR Permanent Document v9.0.0, Nov. 2023
[4] RP-241539, “On the Support of 2Rx for Wristwear XR Devices”, Meta, Apple

[5] Meta’s smart glasses https://www.meta.com/smart-glasses/

[6] Meta’s AR Glasses https://about.meta.com/realitylabs/orion/