3GPP TSG-RAN WG4 Meeting #112bis	R4-2415280

Hefei, China, 14-18 October 2024

Title: 	On the SRS imbalance issue for further discussion

Agenda Item:	6.1.1.3.4

Source: 	Nokia

Document for:	Approval

# Introduction

In previous RAN4 meeting WF of 6Rx UE requirements was approved. This contribution discusses about issue of SRS imbalance. Following issues were listed as WF [1].

#### Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19

Way Forward: RAN4 to further discuss the following options.

-	Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

-	Option 2: Continue to pursue a solution to the SRS IL imbalance issue dependent on the outcome of Issue 4-1-2 in the approved WF in R4-2410751.

#### Issue 4-1-2: Initial Considerations for SRS IL imbalance issue

Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL.

-	Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL.

-	Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation.

#### Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue

Way forward: Companies encouraged to provide compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain

# Discussion

There have been a lot of discussions about SRS imbalance related to UE HW implementation, antenna interface and its impact on network performance started already Rel-18 both in RAN4 and RAN1 as stated in [2]. SRS imbalance issues enlarge as number of antennas increase. To address the issue there are solutions proposed since Rel-18 that are focusing on UE self-compensation of SRS imbalance or by signalling the imbalance to NW or both. The former can work only when PA has enough maximum linear power headroom on main antenna while providing SRS resources to other antennas. If the value of DTRxSRS is high, compensation may be partial but this is a feasible, UE based, solution to address imbalance issue [3]. However, there are arguments that even if UE can compensate SRS imbalance perfectly at UE conductive antenna port it does not guarantee that that when signal arrives at gNodeB receiver such a balance is preserved [2]. Furthermore in [4] it was shown that if there would be a signalling solution to indicate gNodeB UE SRS imbalance with a conclusion of “there is no DL performance benefit in the UE reporting SRS offset values and gNB performing compensation of IL compared to the UE performing IL compensation alone”. All in all, companies’ opinions about possible benefit having a mechanism addressing SRS imbalance are diverged. Previous RAN4 meeting didn’t bring much new information on the table.

Regarding first issue in WF, in past two RAN4 meetings we proposed not continue the discussion on how to solve the SRS imbalance issue by specification. We propose it again.

Proposal 1: RAN4 should not continue the discussion on how to solve the SRS imbalance issue.

Second issue listed in WF discusses initial considerations phase and subsequent study to discuss candidate solutions. If some progress in this topic is desired which in our opinion is not necessary, a deadline should be for initial considerations phase. For example, RAN4#112bis.

Observation 1: a deadline should be set to the initial considerations phase. For example, RAN4#113.

# Conclusion

In this contribution, we have made the following proposals.

Proposal 1: RAN4 should not continue the discussion on how to solve the SRS imbalance issue by specification.

If proposal 1 is not acceptable then

Observation 1: a deadline should be set to the initial considerations phase. For example, RAN4#113

# 4	References

[1] R4-2414285, WF on 6Rx UE requirements, RAN4#112

[2] R4-2404935, Views on 6Rx for handheld and FWA UE, RAN4#110bis, Samsung

[3] R4-2411458, Discussion on SRS IL imbalance for 6RX UE, RAN4#112, Spreadtrum

[4] R4-2405083, UE SRS IL imbalance for 6Rx UE, RAN4#110bis, Shanghai Chen Si Electronics