3GPP TSG-RAN WG4 Meeting #112bis	R4-2415283

Hefei, Anhui, China, 14th – 18th October 2024

Source:	Nokia

Title:	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements

Agenda item:	6.14.2

Document for:	Information

Introduction

During RAN#104 the study item on Spatial Channel Model was agreed, whereby the SI is due to start during RAN4#112  [1]. This informative contribution is intended to help other interested parties to implement a TS 38.827 based CDL channel model and enable alignment with our version. Furthermore, this tdoc serves as the companion contribution to our FS_NR_demod_SCM discussion [2] and contains the complete context and extended results for the simulation results used in the discussion. Previous results can be found in [3].

In summary, this contribution contains the following parts:

- Full set of updated CDL/TDL/TDL+antCorr simulation results.

- TP for 38.827 based CDL channel model (“CDL”) updated with agreements from RAN4#112.

- Implementation notes.

No observations nor proposals are made in this simulation contribution.

Simulation results

Simulation setup

All simulation results in this tdoc and our companion discussion tdoc [2] use the configuration detailed in Table 1, unless explicitly stated otherwise.

Table 1: Proposed simulation configuration for channel model alignment in study (8T/8R/rank 8-layer reference case).

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Simulated Channel |  |  | PDSCH |
| Duplex mode |  |  | FDD |
| FR / Carrier frequency |  |  | FR1 / 3.5 GHz |
| UE receiver type |  |  | Simple SU MMSE |
| Antenna configuration (BB branches) |  |  | 8 Tx / 8 Rx |
| Waveform |  |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| MCS |  |  | 17 (64 QAM table) |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static (N/A) |
|  | PRB bundling size |  | 2 (N/A) |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 (N/A) |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
|  | Transmission Rank |  | 8 for 2CW |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 (for rank > 4) |
|  | DMRS ports indexes |  | {1000-1007} for Rank8 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | RI Restriction |  | 10000000 for rank 8 |
| PDSCH & PDSCH DMRS Precoding | Common parameters |  | Wideband (but shouldn’t matter) |
|  | Fixed |  | i11=2, i12=0, i2=1 (i12 is fixed 0 in ULA with this configuration. All indices counting from “0”) |
|  | Random |  | Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with wideband granularity |
| NZP CSI-RS for CSI acquisition (Only used for 4Tx PMI results. No TRS, TOE based on DMRS.) | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 4, (0) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (13) |
|  | CSI-RS periodicity and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 4 |
| Analog, RF, fixed point impairments |  |  | None. Ideal results only. (No TO, no FO, no PA model, no TxEVM, no RxEVM, no IQ imbalance, etc.) |
| Number of HARQ Processes |  |  | 4 |
| Maximum HARQ transmissions |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 1 (Change w.r.t. initial contribtuions) |
| Channels |  |  | CDL-C UMa NLOS (“CDLC”) 30kph UE velocity, 365ns RMS DS, in the TR 38.827 based version detailed in Section 3. I.e., - Reiver uses n/2x1 cross-pol ULA with lambda/2 distance, i.e., [++…+], with omni AE radiation pattern. - Cosine/38.901 AE pattern on transmitter side. - CDL ray paring and polarization scalar initial phases fixed. TDLC300-100 (ULA) Low, (30kph DUT speed) TDLC300-100 (ULA) MedA, (30kph DUT speed) Further TDL antenna correlation matrices as defined in 38.101-4 |

Notes on SNR/SINR definitions: 	
Operating points and performance KPIs are usually defined in terms of “SNR”. We use the RAN4 demodulation definition of “SNR” (see for example TS 38.101-4, section 4.4.2). It can be described as the average signal over noise power ratio as measured at the BB input branches of the equalizer/demodulator, averaged over each demodulator input branch and RE, where the signal exists, i.e., it is the pre-equalizer SNR per Rx branch.	
RAN4 demod also uses a similar definition of pre-equalizer SINR per Rx branch (see for example TS 38.101-4 section 4.4.5), with the interference power added to the noise power. However, we will refrain from using SINR in this sense and opt to call all pre-EQ S(I)NR relationships simply “SNR”.	
In the rare instance that we talk about “SINR”, we use it to describe the post-equalizer per layer SINR, which is more common.

Note on channel model configuration choices:	
We chose TDLC300-100 (300ns DS) and CDLC UMa NLOS 30kph (363ns DS), as they are defined in specifications already, share similar RMS delay spread (300/365ns), speed/velocity (30kph), and are NLOS representative. I.e., they are comparable with each other concerning all non-spatial characteristics, which is not surprising as the TDLC details were derived/filtered from this CDLC variant in TR 38.901.	
However, combining TDL with any kind of antenna correlation matrix will turn out to increase SNR requirements to infeasible levels or become directly rank deficient, hence we will use TDL with LOS components (e.g., TDLA, TDLD) to show certain TDL properties, when necessary.	
CDLC UMa also has slightly elevated SNR requirements, but this can be resolved by choosing another profile (e.g., CDLA), once the TDL vs. CDL comparisons on spatial characteristics are agreed upon.

SU TPUT Scenarios and results

TPUT scaling and precoder effects vs. channel model

|  | Cannot achieve meaningful TPUT |  |
| --- | --- | --- |
| TDLC300-100 low | TDLC300-100 MedA | 827 CDLC UMa |

Figure 2: 8T8R, 2CW (MCS17 both) full rank relative TPUT, SU-MMSE receiver, Fixed and random precoding vs. pre-EQ SNR.

| TDLC300-100 MedA | TDLA30-10 MedA |
| --- | --- |

Figure 3: 8T8R, 2CW (MCS7 both) full rank relative TPUT, SU-MMSE receiver, Fixed and random precoding vs. pre-EQ SNR.

TPUT scaling and precoder effects vs. AE correlation in CDLC

| 827 CDLC UMa ULA lambda/2 | 827 CDLC UMa ULA lambda/4 | 827 CDLC UMa ULA lambda/8 |
| --- | --- | --- |

Figure 4: 8T8R, 2CW (MCS17 both) full rank relative TPUT, SU-MMSE receiver, Fixed and random precoding vs. pre-EQ SNR, for increasing CDL ULA correlation (AE distances reducing).

TPUT scaling and precoder effects vs. speed in different channel models

| TDLC300-100 low (30/50kph) | 827 CDLC UMa (30/50kph) |
| --- | --- |

Figure 4: 8T8R, 2CW (MCS17 both) full rank relative TPUT, SU-MMSE receiver, Fixed and random precoding vs. pre-EQ SNR for differing speeds and channel models. FOC off.

Sensitivity to receiver implementations

| TDLC300-100 low | TDLC300-100 MedA (MCS 7) |
| --- | --- |
| 827 CDLC UMa ULA |  |

Figure 5: 4T4R, 1CW (MCS17) full rank, random precoding, Differing SDM receiver implementation, relative TPUT vs. pre-EQ SNR.

SU PMI Scenarios and results

SU PMI profiles (4Rx)

| TDLC300-100 low SNR=15dB => CW Throughput = 96.24% | TDLC300-100 XP high SNR=25dB => CW Throughput = 90.54% |
| --- | --- |
| 827 CDLC UMa ULA SNR=14dB => CW Throughput = 92.57% |  |

Figure 2: Histograms of PMI feedback choices under different channel models (1000 CSI reports).
4T4R, 1CW, MCS13, full rank, PMI chosen based on (non-precoded) CSI-RS, N1=2, N2=1, O1=4, O2=1.

SU PMI profiles (8Rx)

CDLC 365ns UMa: 22 dB - TPUT = CW0 65.8%, CW1 86.32% TDLC 300ns ULA Low: 16 dB - TPUT = CW0, 91.69% CW1 88.69% TDLC 300ns XP High: 40 dB - TPUT = CW0 6.75%, CW1 47.75%

Figure 2: Histograms of PMI feedback choices under different channel models (500 CSI reports).
8T8R, 1CW, MCS13, full rank, PMI chosen based on (non-precoded) CSI-RS, N1=4, N2=1, O1=4, O2=1.

MU TPUT Scenarios and results

Advanced receivers

| TDLC300-100 low | TDLC300-100 XP Med |
| --- | --- |
| 827 CDLC UMa ULA |  |

Figure 3: Differing advanced receivers (R-ML, E-IRC, IRC) in presence of co-scheduled UE.
4T4R, 1CW (MCS13+MCS4), rank 2+2, MCS4 interference, orthogonal and random MU precoding.

| TDLC300-100 low | TDLC300-100 XP Med |
| --- | --- |
| 827 CDLC UMa ULA |  |

Figure 4: Differing advanced receivers (R-ML, E-IRC, IRC) in presence of co-scheduled UE.
4T4R, 1CW (MCS13), rank 2+2, MCS13 interference, orthogonal and random MU precoding.

MU PMI Scenarios and results

No results delivered this meeting.

Other use cases and channel model characteristics

Convergence speed/testing time

CDLC @ 22dB (50% TPUT) / TDLClow @ 19dB (50% TPUT) / TDLCmedA @ 50dB/MCS7 (32% TPUT)

Figure 5: Simulation convergence trials: 8T8R 2CW (MCS17 both) full rank, SU-MMSE receiver, Random precoding vs. simulated slots (at 50% TPUT operating point target).

TP for 38.827 based CDL channel model (SU version)

This section gives a description of a TR38.827 based SU CDL channel model (often referred to as “CDL” or “827 CDL”) in the form of a potential TP for 38.101-4, which was used to derive all our results in our contributions. Note that explanatory parts of TR38.827, that are not needed for implementation, have been removed, and the extension for conducted/virtual cable test setup have been added.	
Minor modification based on discussions and agreements in RAN4#112 have been included, where appropriate

Interested contributors are encouraged to implement 827 CDL based on the following TP and exchange with Nokia to achieve alignment.

<Start of TP>

## B.2.X1	Cluster Delay Line Channel Model

### B.2.X1.Y1	General

The channel model is based on the description and clarifications given in TR 38.827.

For NR FR1 and FR2 MIMO testing, the number of samples for sequence length at each testing point is FFS.

### B.2.X1.Y1	Channel Models

This section describes amendments to the stepwise procedure of the CDL subclause 7.7.1 in TR38.901 for generating fast fading radio channel realizations. This channel model methodology considers non-Jakes spectrum with the multi-path fading propagation conditions between the gNB emulator and DUT modelled based on Clustered Delay Line (CDL) methodology.

The tables provided in section B.2.X1.Y1 are to be used directly in the procedure in TR38.901 for generating fast fading radio channel realizations. The given coefficients have been scaled to achieve the following target delay spread values

Table 7.2-1. Target delay spread values.

| Frequency | Scenario | DSdesired |
| --- | --- | --- |
| FR1 | UMi | 100 ns |
| FR1 | UMa | 365 ns |
| FR2 | UMi | 60 ns |
| FR2 | InO | 30 ns |

Similarly, the cluster-wise rms azimuth spread of arrival angles (cluster ASA, cASA), and similar spreads in the tables, are scaled to achieve the following desired AS, for the given cluster AOA (n,AOA) and similar azimuth/zenith angles (Tables 7.7.1.1 – 7.7.1.5 of TR 38.901).

Table 7.2-4: Desired AS for UMi and UMa at 3.5 GHz (FR1)

| Model | ASdesiredASdesiredASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| UMi NLOS (CDL-A, B, C) | 23.9751 | 57.2457 | 0.7762 | 7.8320 |
| UMi LOS (CDL-D, E) | 15.0432 | 47.6149 | 0.6166 | 4.6204 |
| UMa NLOS (CDL-A, B, C) | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| UMa LOS (CDL-D, E) | 14.0180 | 64.5654 | 3.4674 | 8.9125 |
| Note: For UMa frequency fc = 6 as stated in [2], and other parameters hUMa = 25, hUMi = 10, hUT = 1.5, and D2D = 100. |  |  |  |  |

Once the channel realization procedure arrives at the step of coupling the AOD angles to AOA angles within a cluster n (step 2). Instead of random procedure, the coupling shall be performed using the fixed coupling pattern specified in Table 7.2-6. The same fixed coupling pattern is applied for all clusters n.

Table 7.2-6: Fixed coupling pattern of ray angles to be applied for each cluster

|  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 12 | 5 | 10 | 8 | 11 | 16 | 14 | 18 | 9 | 20 | 4 | 2 | 15 | 7 | 13 | 19 | 17 | 3 | 1 |
|  | 20 | 9 | 12 | 1 | 13 | 18 | 10 | 4 | 8 | 2 | 6 | 14 | 11 | 19 | 7 | 3 | 17 | 5 | 15 | 16 |
|  | 2 | 16 | 3 | 11 | 18 | 9 | 5 | 17 | 4 | 19 | 15 | 20 | 13 | 7 | 10 | 1 | 8 | 12 | 6 | 14 |
|  | 15 | 18 | 13 | 1 | 12 | 9 | 6 | 7 | 5 | 3 | 2 | 8 | 14 | 17 | 19 | 16 | 11 | 20 | 10 | 4 |

In the next steps, the linear cross polarization power ratios (XPR) are calculated for each ray m of each cluster n as

,	(7.2-7)

where X is the per-cluster XPR in dB from the tables provided in section B.2.X1.Y1.

The gNB beam pattern including the assumptions for gNB antenna for definitions and symbols of subclause 7.3 of TR38.901 for FR1 and FR2 are summarized in Table 7.2-7.

Table 7.2-7: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |  |
| --- | --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz | FR2 |
| Antenna panels in vertical dimension | Mg | 1 | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 | 16 |
| Number of polarizations per panel | P | 2 | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 | 0.5 |

Antenna element radiation patterns, including orientation of the element main polarization components as well as orientation of the antenna array for both FR1 and FR2 are as in the example pattern in Table 7.3-1 of TR38.901. The antenna element has ±45 polarization components and the radiation pattern parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi.

Note this chosen AE pattern may impact the RMS delay spread scaling and necessitates coordinate transformations as defined in TR 38.901 section 7.3.2, but this has not been taken into account for deriving the tables provided in section B.2.X1.Y1.

It is assumed the co-polarized elements of the array are combined to a single RF port, i.e. they compose an antenna array that can form beams by setting certain weights per element. Weight vector for the first polarization and for the second polarization is

α1,1…αMe,Ne=1MeNeexp-j2πrtx,max T ∙ dtx,me,neλ0∈C1×MeNe,	(7.2-8)

where dtx,me,ne is the location vector of transmit antenna element me=1,…,Me and ne=1,…,Ne, and rtx,max is a spherical unit vector denoting the target beam direction. Note that both target beam directions and directions of departure (DoD) angles of the transmitter array, i.e., ZoD and AoD, are referenced to broadside. Determination of beam directions rtx,max is done as follows:

-	For FR1: A grid of 60 fixed beams is constructed to a grid of five elevation angles from –20 to +20 with 10 steps and 12 azimuth angles from –80 to +80 with ~15 steps；

For FR1 4x4, the two strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

For FR1 8x8, the four strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

In detail, beam directions for channels model given in Section B.2.X1.Y1 are

-	For UMa CDL-C, the beam directions are:

-	Strongest beam: AoD: -7.27°, ZoD: 100°

-	2nd strongest beam: AoD: -21.82°, ZoD: 100°

-	3rd strongest beam: AoD: -36.36°, ZoD: 100°

-	4th strongest beam: AoD: -50.91°, ZoD: 100°

For the UE it is assumed that RF ports map directly to receive antenna elements, where drx,u is the location vector of receive antenna elements, which are configured as a ULA with Cross-pol (1, n/2, 2), i.e. [+ + + +] for n=8, antenna elements with omni radiation pattern, 0.5 spacing, and rotation/slant is rotated 45deg to the transmitter polarizations. Note that “n” is the number of Rx elements, and the directions of arrival (DoA) angles of the receiver array, i.e., ZoA and AoA, are referenced to broadside of the receive antenna array.

The random initial phases  are not used for the different polarization combinations (θθ, θϕ, ϕθ, ϕϕ). Instead, a fixed and pre-defined set of initial phases Φmθθ,Φmθϕ,Φmϕθ,Φmϕϕ of Table 7.2-8 and a scalar random initial phase term Φn,m~Uni(-π,π) is used for each ray m of each cluster n.

Table 7.2-8: Fixed initial phases for 2x2 polarization matrices. These values are drawn from uniform distribution

| m | Φmθθ [rad] | Φmϕθ [rad] | Φmθϕ [rad] | Φmϕϕ [rad] |
| --- | --- | --- | --- | --- |
| 1 | 1.7609 | -0.6928 | -1.6230 | -0.6037 |
| 2 | -2.5356 | -2.3124 | 2.7775 | 2.8660 |
| 3 | 0.4725 | -2.7660 | -1.6664 | -0.9226 |
| 4 | 2.0181 | -3.0448 | -2.8713 | -2.0798 |
| 5 | 0.9369 | 1.4560 | 0.9283 | -0.3084 |
| 6 | 0.2954 | -1.2798 | 1.5375 | -1.9544 |
| 7 | 1.1735 | -1.9886 | -0.8263 | 0.7893 |
| 8 | 1.7607 | -2.6319 | 2.6979 | 1.7324 |
| 9 | -0.0830 | -0.4030 | -0.3344 | -1.2167 |
| 10 | 0.0535 | 0.0677 | 1.9957 | 1.8525 |
| 11 | 0.9068 | -0.7627 | 1.9577 | 0.2062 |
| 12 | -0.9379 | 2.7583 | 2.3621 | 0.3151 |
| 13 | 0.7695 | 0.5469 | -1.8363 | -1.2488 |
| 14 | -0.1827 | -1.6934 | 2.1634 | -1.9179 |
| 15 | -1.7221 | -2.0690 | -1.7111 | -0.4040 |
| 16 | -1.1869 | 2.6602 | -0.4385 | -1.9804 |
| 17 | 2.5439 | 3.0143 | -0.3841 | -2.4434 |
| 18 | -1.5201 | -0.5735 | 0.5962 | -1.4941 |
| 19 | 0.6462 | 1.3271 | -1.7483 | -2.4038 |
| 20 | -1.2775 | -1.1386 | -0.4765 | 0.0494 |

To determine the channel all clusters are treated as "weaker cluster", i.e., no further sub-clusters in delay should be generated. The BS beamforming weights αme,neαme,nedefined in Equation 7.2-8 for antenna elements are used and the BS antenna signals are summed for BS beamforming. The BS transmits downlink signals with S beams. Index s=1,…,S denotes the formed beam index. Each beam may have different and thus the beamforming weight of eq. (7.2-8) becomes specific for index s as αs,me,ne; it should be noted though that there are always two orthogonally polarized beams to the same direction. Here, the random initial phases Φn,m are used for sub-paths, but not for the different polarization combinations (θθ, θϕ, ϕθ, ϕϕ). The channel coefficient for time instant t, Rx antenna/beam u, Tx beam s, and cluster n is defined by the following equations. They apply for the NLOS clusters:

Hu,s,nNLOSt=PnMm=1Mne=1Neme=1MeFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕFtx,s,me,ne,θθn,m,ZOD,φn,m,AODFtx,s,me,ne,φθn,m,ZOD,φn,m,AOD

αs,me,neexpjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,s,me,neλ0expj2πrrx,n,mT.vλ0t,	(7.2-9)

where Ftx,s,me,ne,θ, Ftx,s,me,ne,φ, and dtx,s,me,ne are the theta and phi polarized radiation patterns and the position vector of the BS antenna element me,ne of sub-array s, respectively. Symbols Frx,u,θ , Frx,u,ϕ, , , and , are determined as in TR 38.901, which includes the generation of AOA (ϕn,m,AOA), AOD (), ZOA (), and ZOD () to derive  and . The velocity vector is determined as

(7.2-11)

UE velocity v is defined as follows: 30km/h for FR1 vs 3 km/h (Indoor Office) and 12 km/h (UMi) for FR2. The UE travelling direction (v, v) are as follows for FR1:

-	(135°,90o) for UMi CDL A channel model

-	(127.05°],90o) for UMi CDL C channel model

-	(182.17°,90o) for UMa CDL A channel model

-	(65°,90o) for UMa CDL C channel model

The UE travelling direction (v, v) are as follows for FR2:

-	(112.51°,90°) for InO CDL-A channel model

-	(74.11°,90°) for UMi CDL-C channel model

Note: the FR2 cases may be mixed up in 38.827, though this has no impact in the CDL-C UMa case we are focusing on.

### B.2.X1.Y1	Channel Model Parameters for FR1

The Channel model parameter tables for CDL-A, B, C, D, and E for UMa and UMi at 3.5 GHz are presented in this subclause without the effect of base station antenna filtering.

For FR1, the baseline emulated propagation environment is 2D without elevation modelling, i.e., all ZOA are set 90° and ZSA is 0° in the following tables.

Tables 7.2.1-6—7.2.1-10 tabulate channel model parameters for UMa CDL-A—CDL-E models at 3.5 GHz, respectively.

In the determination of desired angle spreads (ASdesired), frequency is set 6 GHz as stated in Table 7.5-6 Part-1 of TR38.901.

For the determination of desired zenith spread of departure (ZSDdesired) from table 7.5-7 of TR38.901, the following parameters are used hUT = 1.5 m, and d2d = 100 m.

Table 7.2.1-8: Channel model parameters for UMa CDL-C at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

<End of TP>

Implementation notes

During the implementation of the channel model from section 3, a number of questions and remarks came up repeatedly. To aid other interested contributors with their implementation, we summarize our understanding of some less evident aspects of the implementation in the following.

Everything in this section is our interpretation of sometimes unclear wording or concepts in TS 38.827, hence all notes are open to re-interpretation during the study. See also the previous contribution [3] with earlier implementation notes.

TR 38.827 specific BB/RF transmit ports to AEs mapping

At RAN4#112 offline discussions were had concerning the interpretation of the 827 AP2AE mapping, i.e., the AE weights derived in (7.2-8).

It is our understanding that the formulation, its parameterization, and application, does not match a sub-array/sub-panel beam forming approach. Rather the formula (7.2-8) implies “collocated panel duplication” for each additional RF/BB pair. This has limited impact in the demodulation requirement setup and testability:

- In simulation or channel emulation this is permissible (and already implemented in commercially available channel emulators).

- Power wise, the duplication does not impact the results, as RAN4 SNR is defined on the RX pre-EQ side, so the channel is properly normalized in the whole simulation loop.

- The duplication, however, implies that the spatial filter (“beam”) for each RF/BB port is equally selective, no matter how many ports there are.

Hence, the 827 AP2AE mapping is fine in the definition of an emulated channel model. It is implementable in an 8x8 channel emulator “box”, which do not expose the channel model assumed “128” AEs to the transmitter or receiver. This is in line with the demod testing approach that excludes, or abstracts, any RF aspects of the deployment, and focuses on setting requirements for implementations “in front of/behind/ the transmitter/receiver baseband ports”.	
Additionally, changing a pre-specified channel model (e.g. 38.827) would mean that the theoretical and statistical verifications need to be redone.	
Finally, it is advantageous that the 827 model is a way to explicitly avoid replicating any one specific port-to-AE mapping that can be deployed, thus not incentivising making any implementation assumptions in the BB algorithms. Ultimately, modelling one fully realistic setup is not feasible and all that is needed is to capture typical MIMO effects to set demod requirement.

Global view of 38.827 CDL with BB/RF port to Tx/RX mapping focus

Figure 8: TR 38.827 CDL with FS_NR_demod_SCM modifications.
Intuitive global view with BB/RF port to Tx/RX mapping focus.

Conclusion

Within this contribution we have provided simulation results and CDL implementation reference for use in the Spatial Channel Model for Demodulation Performance Requirements study, the results will be used in our companion discussion contribution [2].

- References

- RP-241610; “New SID: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc et al, RAN#104, Shanghai, June 2024.

- R4-2415282, “Discussion on Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#112bis.

- R4-2411301, “Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#112.