3GPP TSG-RAN WG4 Meeting #112bis                                                                     R4-2415285

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.14.2

Source:	Orange, BT, Telecom Italia, Vodafone, Bouygues Telecom, Fraunhofer, Deutsche Telekom

Title:	TDL and CDL comparison with respect to the CSI resolution feedback

Release:	Rel-19

Related WI:	FS_NR_demod_SCM

Document for:	Discussion

# Introduction

During the RANP#104, it was decided to start a new study item [1] on Spatial Channel Model (SCM) for demodulation performance, due to the limitation of the current spatially agnostic Tapped Delay Line (TDL) channel model and to study practical spatial channel modelling methodology for both Single User (SU) and Multi-User (MU) MIMO demodulation requirements and Channel State Information (CSI) reporting requirements. During the RAN4#112 meeting, agreements on spatial channel modelling methodologies were reached [2] for alignment and comparison purposes; including TDL based methodology, Cluster Delay Line (CDL) based methodology and comparison methodology.

The main objective of this contribution is to compare the CDL-class of SCMs with the legacy RAN4 channel models (“TDL-class”) with respect to the CSI resolution feedback. We leverage our prior experience with 38.901 CDL implementation, instead of directly implementing the RAN4 derived 38.827 CDL. Our understanding is that 38.827 CDL is a simplified version of 38.901 CDL, compliant with test equipment implementation requirements, which keeps the essential spatial properties of the initial RAN1 model.  As a result, the use of 38.901 CDL is perfectly adapted to highlight the usefulness and need for CDL-class SCMs, when setting reasonable PMI minimum performance requirements. In this contribution, we compare the performance of Type-I (low resolution) and eType-II (high resolution) codebooks for TDL (TDLC300-100-10/Low/Med/High XP 16x4) and 901 CDL (CDL-C with RMS DS 363ns 16x4) channels models in a SU-MIMO setting with 16 transmit antenna ports (N1=4,N2=2,P=2) at the gNodeB and 4 receive antenna ports at the UE (M=1, N=2, P=2) . Simulation results are discussed with and without Link Adaptation (LA) at the speed of 3km/h and 30km/h, they confirm and extend the previous observations stated in [3,4]. Finally, based on our own observations, we suggest some proposals to fill the gap between RAN1 sophisticated massive MIMO techniques and RAN4 performance requirement methodologies such that the operators can reap the promised gain of their investments on advanced MIMO techniques.

# Simulation methodology

The simulations parameters are given in Annex B and Annex C. The link adaptation as well as the precoder selection algorithms are given in [5]. The SNR corresponds to the RAN4 definition, and its derivation is given in Annex A. We suppose that power amplifier configuration per antenna port does not introduce any local power constraint, i.e., the nominal EPRE power can be reached [6]. The 901 CDL channel was implemented following [7], with antenna port to antenna element virtualization as in Annex C. In order to have a “fair” comparison between TDL and 901 CDL, we select similar RMS delay spread (around 300ns) and the same number of antenna ports at the gNodeB, i.e., 2N1N2= 16. In the following, we present the simulation results with respect to the link adaptation being activated or not. When the LA is applied, the CSI computation optimizes jointly the MCS/PMI/RI in order to maximize the throughput for a 10% BLER target [5]. Since the number of receive antenna ports is fixed to 4, the rank is freely selected between 1 and 4. Without LA, the MCS is fixed to MCS 17 of PDSCH Table 2 of TS38.214 and the rank to 4. Finally, to observe the effect of speed, we simulate a low speed (3km/h) and medium speed (30km/h) channel model. To better analyse the results, we plot three performance bounds, namely, Eigen-beamforming (outdated CSI), Eigen-beamforming with no PMI delay (the CSI is up-to-date), and Type-I random PMI. The Eigen-beamforming bound corresponds, for each SubBand (SB), to a selected precoder from an ideal codebook comprising the eigenvectors of the SB equivalent channel assuming a power equipartition between layers where rank l precoder is made of the l eigenvectors associated to the best l eigenvalues. More precisely, if we denote by H(t)∈CNR×2N1 N2  the representative/equivalent channel of SB t , the rank l precoder is made of the l eigenvectors of  HtHH(t) associated to the best l eigenvalues where .H  is the conjugate transpose.  The Type-I random PMI for a fixed MCS and rank comes down to drawing randomly the PMI from Type-I codebook (i1,1,i1,2,i1,3 are randomly drawn per TTI, while i2 is randomly drawn per TTI per SB).

# TDL channel model

In this section, we try to illustrate the observations made in [3] on the deficiencies of the TDL channel model. Reference [3] analyses TDL channel model behaviour with respect to the Direction of Arrivals (DoAs) and Direction of Departures (DoDs) in the frequency domain. It yields the two following main observations.

- Observation 1: The low correlation TDL channel lacks the spatial property of long-term stable distinct directions (DoA or DoD) with significant power (see Fig. 2 and 3 of [3]). Thus, it is expected that CSI feedback under this channel model is particularly sensitive to UE speed and PMI delay to the point where a randomly selected PMI yields the same performance for a medium velocity (e.g., 30km/h). Furthermore, the channel ability to support high rank transmission is overestimated.

- Observation 2: The non-low correlation TDL channel exposes a main boresight DoD/DoA which makes it improper to test Type-II whose enhancements compare to Type-I is to exploit several spatial preferences or directions. Furthermore, the channel ability to support high rank transmission is underestimated.

Our goal in the following simulations is to demonstrate that Type-I and eType-II have very similar performances for low/medium/ high cross polarization (XP) correlation TDL channels. Indeed, it was shown in [4] that Type-I and eType-II have the same performance for the medium and high correlation TDLA30 cases. This is not surprising given the above observations and the fundamental understanding that TypeI PMI is enough to focus energy in a single (and one additional fixed) direction, while eTypeII allows for choosing “L” directions (or IDFT vectors) as a spatial basis and offers useable phase control intended for interference control between spatial layers both in SU/MU MIMO scenarios. We also want to demonstrate that the performance of Type-I quickly converges to a random PMI bound with increasing speed (assuming a 6.5ms PMI delay), i.e., to confirm that TDL channel lacks the spatial property of long or mid-term stable distinct directions (DoA or DoD) with significant power.

## With Link Adaptation

The simulations parameters are given in Annex B.

### UE speed of 3km/h

Figure 1: TDLC300-10 low correlation with link adaptation

Figure 2: TDL300-10 medium correlation with link adaptation

Figure 3: TDL300-10 high correlation with link adaptation

In Annex D, the differences in terms of SNR with respect to the Eigen-beamforming precoders are provided for both Type-I and eType-II codebooks at a fixed efficiency of 15 bpcu. In the specified simulation scenario, the SNR gap for eType-II is given by 8.25 dB at low correlation, while for Type-I, it is of 9.75 dB. Additionally, the gap between Type-I and eType-II is about 1.5 dB at low correlation and 0.75 dB at medium correlation.

Observation 1:  With LA, the throughput performance of codebook Type-I and eType-II in the case of low correlation are far from the Eigen-beamforming bound which confirms that RAN1 has not designed these codebooks for TDL@3km/h.

Observation 2: With LA, TDL@3km/h is not adapted for CSI reporting requirements since Type-I and eType-II yields very similar throughput performance for low/medium/high correlations.

### UE speed of 30 km/h

Figure 4: TDL300-100 low correlation with link adaptation

Figure 5: TDL300-100 medium correlation with link adaptation

Figure 6: TDL300-100 high correlation with link adaptation

Observation 3: With LA, as expected Type-I is more robust to mobility than eType-II as well as the Eigen-beamforming precoder for TDL@30km/h: the more detailed the CSI, the more sensitive to imperfect channel knowledge (due to mobility, PMI Delay…).

Observation 4: With LA, the Type-I codebook by selecting a unique direction with co-phasing per spatial layer is sufficiently coarse to yield relatively good performance for TDL@30km/h and 6.5ms PMI delay.

## Without Link Adaptation

The simulations parameters are given in Annex B. The precoder selection algorithm is given in [5].

### UE speed of 3km/h

Figure 7: TDLC300-10 low correlation with MCS 17 from Table 2 for PDSCH and rank 4

Figure 8: TDLC300-10 medium correlation with MCS 17 from Table 2 for PDSCH and rank 4

Figure 9: TDLC300-10 high correlation with MCS 17 from Table 2 for PDSCH and rank 4

Using the TDLC300-10 channel model with MCS 17 and rank 4, the differences in terms of SNR at a fixed efficiency of 15 bpcu for eType-II are given by 10.75 dB, 8 dB, and 4.25 dB at low, medium, and high correlation, respectively. While for Type-I, the SNR gaps are of 12.25 dB, 8.5 dB, and 6.5 dB, in the same order. Moreover, the difference between Type-I and the random Type-I bound is about 1 dB at low correlation.

Observation 5:  For MCS 17 and rank 4, the performance of codebook Type-I and eType-II are far from the Eigen-beamforming bound for low/medium correlation which confirms that RAN1 has not designed these codebooks for TDL@3km/h.

Observation 6:  For MCS 17 and rank 4, the Type-I performance is very close to the random PMI bound for the low correlation case which means that the PMI knowledge does not bring substantial gain even for TDL@3km/h.

Observation 7: For MCS 17 and rank 4, in the case of high correlation the random PMI bounds yields very bad performance for TDL@3km/h since there is a stable unique direction that can be exploited by Type-I and eType-II.

### UE speed of 30 km/h

Figure 10: TDLC300-100 low correlation with MCS 17 from Table 2 for PDSCH and rank 4

Figure 11: TDLC300-100 medium correlation with MCS 17 from Table 2 for PDSCH and rank 4

Figure 12: TDLC300-100 high correlation with MCS 17 from Table 2 for PDSCH and rank 4

Observation 8: For MCS 17 and rank 4, in the case of low/medium correlation Type-I performs better than Type-II for TDL@30km/h

Observation 9: For MCS 17 and rank 4, Type-I with optimized PMI selection is equivalent to random PMI selection for the low correlation case for TDL@30km/h

# CDL channel model

The 901 CDL channel was implemented following [7], with antenna port to antenna element virtualization as described in Annex C.

## With Link Adaptation

The simulations parameters are given in Annex C. The link adaptation as well as the precoder selection algorithms are given in [5].

### UE speed of 3km/h

Figure 13: CDL-C 363 ns with a UE speed of 3 km/h and link adaptation

Using the CDL-C 363 ns channel model at 3 km/h with link adaptation, the horizontal gap with respect to the Eigen-beamforming precoder is about 3 dB for eType-II and 8 dB for Type-I at a fixed efficiency of 15 bpcu.

Observation 10:  With LA, the throughput performance of codebook eType-II is closer to the Eigen-beamforming for the CDL@3km/h which confirms that RAN1 has designed this codebook with CDL channel in mind.

Observation 11:  With LA, the throughput performance of codebook eType-II is significantly better than Type-I for CDL@3km/h which means that there is no need to go to a MU-MIMO setting for CSI reporting requirements if RAN4 adopt the right channel model

### UE speed of 30 km/h

Figure 14: CDL-C 363 ns with a UE speed of 30 km/h and link adaptation

Observation 12: With LA, eType-II needs to be enhanced to cope with mobility (eType-II Doppler) for CDL@30km/h

Observation 13: With LA, the Type-I codebook by selecting a unique direction with co-phasing per spatial layer is sufficiently coarse to yield relatively good performance for CDL@30km/h

## Without Link Adaptation

The simulations parameters are given in Annex C. The precoder selection algorithm is given in [5].

### UE speed of 3km/h

Figure 15: CDL-C 363 ns with a UE speed of 3 km/h, MCS 17 from Table 2 for PDSCH and rank 4

Using the CDL-C 363 ns channel model at 3 km/h with MCS 17 and rank 4, the SNR gap between Type-I and the Type-I random PMI bound is about 6 dB at an efficiency of 15 bpcu. Moreover, this horizontal difference is given by 8 dB between Type-I and eType-II.

Observation 14:  For MCS 17 and rank 4, the performance of codebook eType-II is closer to the Eigen-beamforming bound for the CDL@3km/h which confirms that RAN1 has designed this codebook with CDL channel in mind.

Observation 15: For MCS 17 and rank 4, Type-I performance is far from the random PMI bound in the case of CDL@3km/h.

Observation 16: For MCS 17 and rank 4, eType-II brings substantial performance gain with respect to Type-I in the case of CDL@3km/h.

### UE speed of 30 km/h

Figure 16: CDL-C 363 ns with a UE speed of 30 km/h, MCS 17 from Table 2 for PDSCH and rank 4

Observation 17: For MCS 17 and rank 4, the Type-I codebook by selecting a unique direction with co-phasing per spatial layer is sufficiently coarse to yield relatively good performance for CDL@30km/h

Observation 18: For MCS 17 and rank 4, the eType-II codebook still outperforms Type-I for CDL@30km/h.

# Comparison of TDL and CDL

Following the simulation results presented in the preceding sections, we notice a few common trends in the observations:

- When using TDL channel models, be it with or without legacy RAN4 antenna correlation modelling, the performance of Type-I and eType-II CBs is nearly identical. This raises serious concerns on whether performance requirements based on such channel models can set a reasonable minimum performance level, both from an implementation correctness and from the perspective of absolute values for the performance requirements of different MIMO techniques. Similarly, the performance difference between precoding performance bounds and derived requirements are seen to be unreasonably large, again casting doubt on the quality of the requirements.

- When using CDL channel models (at least the 38.901 version), the comparison between Type-I and eType-II CBs and PMI selection algorithms is as intended by 3GPP, when designing the features. Indeed, eType-II has higher spatial resolution, therefore it can exploit several spatial components at the same time and mitigate efficiently spatial layer interference at the expense of potentially worse performance when using it in inappropriate circumstances and with suboptimal PMI selection algorithms.

Thus, we make the following proposals to steer the continuation of the SCM in demod study:

Proposal 1: RAN4 shall not consider TDL-derived channel models for the purpose of SCM.

- Given prior alignment on 38.827 CDL, it shall be used as a starting point to meet the SI completion timeframe.

- If needed, other CDL models are not precluded.

Proposal 2: RAN4 shall, at least, derive and compare PMI requirements with Type-I and eType-II PMI using CDL based SCM, for both single user and multi-user scenarios.

# Conclusions

In this contribution, we provide our insights regarding the use of spatial channel model for demodulation performance.

Observation 1:  With LA, the throughput performance of codebook Type-I and eType-II in the case of low correlation are far from the Eigen-beamforming bound which confirms that RAN1 has not designed these codebooks for TDL@3km/h.

Observation 2: With LA, TDL@3km/h is not adapted for CSI reporting requirements since Type-I and eType-II yields very similar throughput performance for low/medium/high correlations.

Observation 3: With LA, as expected Type-I is more robust to mobility than eType-II as well as the Eigen-beamforming precoder for TDL@30km/h: the more detailed the CSI, the more sensitive to imperfect channel knowledge (due to mobility, PMI Delay…).

Observation 4: With LA, the Type-I codebook by selecting a unique direction with co-phasing per spatial layer is sufficiently coarse to yield relatively good performance for TDL@30km/h and 6.5ms PMI delay.

Observation 5:  For MCS 17 and rank 4, the performance of codebook Type-I and eType-II are far from the Eigen-beamforming bound for low/medium correlation which confirms that RAN1 has not designed these codebooks for TDL@3km/h.

Observation 6:  For MCS 17 and rank 4, the Type-I performance is very close to the random PMI bound for the low correlation case which means that the PMI knowledge does not bring substantial gain even for TDL@3km/h.

Observation 7: For MCS 17 and rank 4, in the case of high correlation the random PMI bounds yields very bad performance for TDL@3km/h since there is a stable unique direction that can be exploited by Type-I and eType-II.

Observation 8: For MCS 17 and rank 4, in the case of low/medium correlation Type-I performs better than Type-II for TDL@30km/h

Observation 9: For MCS 17 and rank 4, Type-I with optimized PMI selection is equivalent to random PMI selection for the low correlation case for TDL@30km/h

Observation 10:  With LA, the throughput performance of codebook eType-II is closer to the Eigen-beamforming for the CDL@3km/h which confirms that RAN1 has designed this codebook with CDL channel in mind.

Observation 11:  With LA, the throughput performance of codebook eType-II is significantly better than Type-I for CDL@3km/h which means that there is no need to go to a MU-MIMO setting for CSI reporting requirements if RAN4 adopt the right channel model

Observation 12: With LA, eType-II needs to be enhanced to cope with mobility (eType-II Doppler) for CDL@30km/h

Observation 13: With LA, the Type-I codebook by selecting a unique direction with co-phasing per spatial layer is sufficiently coarse to yield relatively good performance for CDL@30km/h

Observation 14:  For MCS 17 and rank 4, the performance of codebook eType-II is closer to the Eigen-beamforming bound for the CDL@3km/h which confirms that RAN1 has designed this codebook with CDL channel in mind.

Observation 15: For MCS 17 and rank 4, Type-I performance is far from the random PMI bound in the case of CDL@3km/h.

Observation 16: For MCS 17 and rank 4, eType-II brings substantial performance gain with respect to Type-I in the case of CDL@3km/h.

Observation 17: For MCS 17 and rank 4, the Type-I codebook by selecting a unique direction with co-phasing per spatial layer is sufficiently coarse to yield relatively good performance for CDL@30km/h

Observation 18: For MCS 17 and rank 4, the eType-II codebook still outperforms Type-I for CDL@30km/h.

Proposal 1: RAN4 shall not consider TDL derived channel models for the purpose of SCM.

- Given prior alignment on 38.827 CDL, it shall be used as a starting point to meet the SI completion timeframe.

- If needed, other CDL models are not precluded.

Proposal 2: RAN4 shall, at least, derive and compare PMI requirements with Type-I and eType-II PMI using CDL based SCM, for both single user and multi-user scenarios.

### Acknowledgements

We would like to thank Lynda Berrah, who is working toward her Ph.D. within Orange, for having dedicated a lot of time and efforts to provide the simulations needed for this contribution.

# References

- RP-241610, New SID: Study on spatial channel model for demodulation performance requirements,” 3GPP RAN#104, June, 2024.

- R4-2413606, “Way forward for FS_NR_demod_SCM,”  Nokia , 3GPP RAN4#112, Aug. , 2024.

- R4-2411300, “Discussion on Spatial Channel Model for Demodulation Performance Requirements,” Nokia, 3GPP RAN4#112, Aug., 2024.

- R4-2016102, “Evaluations of Rel-16 Type II PMI testing”, Ericsson, 3GPP RAN4#97-e, Nov. 2020.

- X. Fu, D. Le Ruyet, R. Visoz, V. Ramireddy,  M. Grossman, M. Landmann, W. Quiroga , "A Tutorial on Downlink Precoder Selection Strategies for 3GPP MIMO Codebooks," in IEEE Access, vol. 11, pp. 138897-138922, 2023 (URL).

- R1-2408590, “CSI Acquisition Improvement for Type-II codebook,” Orange, ZTE, BT, 3GPP RAN1#118b, Oct. 2024.

- 3GPP TR 38.901 "Study on channel model for frequencies from 0.5 to 100 GHz", v15.0.0, 2018.

# Annex A: SNR computation

The receive signal per RE without precoding (for a given subcarrier f∈{0,…,NFFT-1} ) is

y(f)=H(f)x(f)+n(f)

where H(f)∈CNR×NT is the MIMO channel at subcarrier f assuming NT=2N1N2 transmit and NR receive antennas.

For notation simplicity, we omit the frequency dependance when it is obvious. We suppose that the transmit power PT=Exx†=1 or Ext2=12N1N2. The noise is circularly symmetric with covariance K=Enn†=σ2INR .

The receive power by definition is

PR=Ex†H†Hx
=Et,t'xt*H†Ht,t'xt' .
Since the symbols belonging to different spatial layers are independent, it yields

PR=Etxt2H†Ht,t 
=12N1N2t,rEhr,t2,

where hr,t is the (r, t)-th element of H . Since we use the channel power normalization
σh2=12N1N2NRt,rEhr,t2=1,

it comes that the received power per RE is PR=NR. Finally, the receive power per receive antenna is P=1. The SNR defined by 1σ2  where σ2 is the noise variance corresponds to the SNR per receive antenna per RE which is what RAN4 has defined.

Let the time domain channel with τ taps be hr,t,ii=0τ-1  , then hr,t(f)=i=0τ-1hr,t,ie-j2πif/NFFT  , it yields that Ehr,tf2=i=0τ-1E{hr,t,i 2}  (independence between taps)

In the time domain we have

12N1N2NRt,rEhr,tf2=12N1N2NRt,ri=0τ-1E{hr,t,i 2}=1

The CDL/TDL channel is normalized in the time domain such that hr,t,i  is normalized to 1Chr,t,i with

C=12N1N2NRt,ri=0τ-1E{hr,t,i 2}

Numerical evidence confirms that σh2f=1 ∀f∈{0,…,NFFT-1} .

# Annex B:  TDL simulation parameters

In this study the TDL300-10/100 (resp., v=3km/h and v=30km/h) channel model is considered

| BS antenna configuration 16 ports | (N1=4, N2=2, P=2) |
| --- | --- |
| UE antenna configuration 4 ports | (N1=2, N2=1, P=2) |
| RAN4 spatial correlation | Low, medium, and high |
| UE Speed | 3km/h or 30km/h |
| MCS/PMI/RI delay | 6.5 ms |
| Subcarrier Spacing | SCS=30kHz |
| Number of SubBands (SB) | N3=13 |
| Number of PRBs per SB | NSB=4 |
| Total number of PRBs (20MHz, SCS=30kHz) | TBW=51 |
| Rank | With LA: up to rank 4 Without LA: fixed rank 4 |
| Modulation and coding scheme (MCS) | With LA: MCS from Table 2 for PDSCH Without LA: MCS 17 from Table 2 for PDSCH |
| Retransmission type | Chase |
| Max. number of retransmissions | NRTmax=3 |
| Channel model | 3GPP TDLC300-10/100 RMS Delay Spread=300 ns (Speed v=3km/h and v=30km/h) |
| Detector at the receiver | LMMSE |
| eType-II codebook parameters | paramCombination-r16=5 (L=4, pν=1/4, β=3/4) |
| OLLA | ideal (the UE feedbacks directly the MCS) |
| Channel Estimation | perfect |

# Annex C: 901 CDL channel model parameters

| BS antenna configuration 16 ports | (M, N, P, Mg, Ng; Mp, Np) = (12,4,2,1,1,2,4), (dH,dV) = (0.5, 0.5)λ |
| --- | --- |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1,2,2,1,1) (dH,dV)= (0.5,-)λ |
| BS height | 25m |
| UE height | 1.5m |
| Mechanical Downtilt | β=8.1° |
| UE antenna orientation (BS and UE antenna panels are facing each other) | (α=-180°, β=-8.1°) |
| UE Speed | 3km/h or 30km/h |
| MCS/RI/PMI delay | 6.5ms |
| Subcarrier Spacing | μ=30kHz |
| Number of SubBands (SB) | N3=13 |
| Number of PRBs per SB | NSB=4 |
| Total number of PRBs (20MHz, SCS=30kHz) | TBW=51 |
| Rank | With LA: up to rank 4 Without LA: fixed rank 4 |
| Modulation and coding scheme (MCS) | With LA: MCS from Table 2 for PDSCH Without LA: MCS 17 from Table 2 for PDSCH |
| Retransmission type | Chase |
| Max. number of retransmissions | NRTmax=3 |
| Channel model | 3GPP 38.901 CDL-C RMS Delay Spread=363ns |
| Detector at the receiver | LMMSE |
| eType-II codebook parameters | paramCombination-r16=5 (L=4, pν=1/4, β=3/4) |
| OLLA | ideal (the UE feedbacks directly the MCS) |
| Channel Estimation | perfect |

## 901 CDL antenna port to antenna element virtualization

Since we based our analysis on TR 38.901 CDL, each antenna port was configured to map to 6 antenna elements in the vertical dimension with connection weights 1 1 1 1 1 1T, this results in an (2x4x2) array assumption.

Antenna port configuration for 901 CDL channel

# Annex D:  SNR gap analysis at fixed efficiency (15 bpcu)

This appendix provides the numerical differences in terms of SNR of the Type-I, Type-I Random PMI, and eType-II compared to the Eigen-beamforming precoders at a fixed spectral efficiency of 15 bpcu. Simulation results are given with and without LA at the speed of 3km/h and 30km/h for both TDLC300 and CDL-C363 ns channel models.

## D.1. TDL channel model

### D.1.1. With link adaptation

#### D.1.1.1. UE speed of 3 km/h

|  | Low correlation |  | Medium correlation |  | High correlation |  |
| --- | --- | --- | --- | --- | --- | --- |
| Precoder | eType-II | Type-I | eType-II | Type-I | eType-II | Type-I |
| ΔSNR [dB] | 8.25 | 9.75 | 3 | 3.75 | 2 | 3 |

#### D.1.1.2. UE speed of 30 km/h

|  | Low correlation |  | Medium correlation |  | High correlation |  |
| --- | --- | --- | --- | --- | --- | --- |
| Precoder | eType-II | Type-I | eType-II | Type-I | eType-II | Type-I |
| ΔSNR [dB] | -3 | -11.75 | / | / | / | / |

### D.1.2. Without link adaptation

#### D.1.2.1. UE speed of 3 km/h

|  | Low correlation |  |  | Medium correlation |  |  | High correlation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Precoder | eType-II | Type-I | Type-I random | eType-II | Type-I | Type-I random | eType-II | Type-I | Type-I random |
| ΔSNR [dB] | 10.75 | 12.25 | 13.25 | 8 | 8.5 | 15.25 | 4.25 | 6.5 | >>6.5 |

#### D.1.2.2. UE speed of 30 km/h

|  | Low correlation |  |  | Medium correlation |  |  | High correlation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Precoder | eType-II | Type-I | Type-I random | eType-II | Type-I | Type-I random | eType-II | Type-I | Type-I random |
| ΔSNR [dB] | 3.75 | 2.5 | 2.5 | 2.75 | 1 | 8 | 1.5 | 1.75 | >>1.75 |

## D.2. CDL channel model

### D.2.1. With link adaptation

#### D.2.1.1. UE speed of 3 km/h

| Precoder | eType-II | Type-I |
| --- | --- | --- |
| ΔSNR [dB] | 3 | 8 |

#### D.2.1.2. UE speed of 30 km/h

| Precoder | eType-II | Type-I |
| --- | --- | --- |
| ΔSNR [dB] | 2.5 | -2 |

### D.2.2. Without link adaptation

#### D.2.2.1. UE speed of 3 km/h

| Precoder | eType-II | Type-I | Type-I random |
| --- | --- | --- | --- |
| ΔSNR [dB] | 4.5 | 12.5 | 18.5 |

#### D.2.2.2. UE speed of 30 km/h

| Precoder | eType-II | Type-I | Type-I random |
| --- | --- | --- | --- |
| ΔSNR [dB] | 2.25 | 5.25 | 12 |

There is a direct mapping from antenna port to antenna element at the UE