3GPP TSG-RAN WG4 #112bis	R4-2415304

Hefei, China, October 14-18, 2024

Agenda item:	6.22.2.2		Adaptation of Common Signal/Channel Transmission

Source: 	Qualcomm Incorporated

Title: 	On RRM requirements for adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started. The work plan and the way forward have been agreed in [1] and [2], respectively. According to the work plan, the first focus for common signal/channel adaptation is on SSB adaptation in time domain.

The following additional agreements have been made for SSB adaptation:

- Sub-topic 3-1: SSB adaptation requirements

  - Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode

    - Agreement

      - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode

      - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress

  - Issue 3-1-2: SSB adaptation impact in RRM requirement

    - Agreement

      - RAN4 to discuss the L1/L3 measurement requirement impact as starting point

    - Way forward

      - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point

        - Other SSB adaptation mechanism(s) and requirements are not precluded depending on RAN1 progress

In this paper, we address the issues above of the way forward as defined in [2].

# Discussion

In RAN1 #118, a few additional agreements have been made for SSB adaptation in time domain. The first agreement is related for which scenarios SSB adaptation in time domain is applicable.

Agreement

For adaptation mechanism(s) of SSB in time-domain,

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

  - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)

  - FFS: Adaptation for SSB not on sync raster is supported (A3)

- For Rel-19 NES-capable UE’s SCell

  - Adaptation of SSB configured for the SCell is supported for the following cases

    - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell

    - Adaptation for SSB that is not CD-SSB on sync raster (B2’)

    - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

The first main bullet deals with the support of SSB adaprarion in time domain. It says that time domain adaption of a SSB that is cell defining is not supported on the sync raster. The other two sub-bullets are still FFS whether time domain SSB adaptation for the PCell for an NCD-SSB (A2) or outside the sync raster (A3) are supported. Hence, RAN1 did not yet conclude that SSB adaption is applicable for a PCell at all. Therefore, we propose that RAN4 work on SSB adaptation for the PCell is postponed till RAN1 made further progress.

Observation 1: RAN1 has not yet agreed whether SSB adaption in time domain is applicable for a PCell.

Proposal 1: RAN4 should postpone all work on SSB adaptation related to the PCell till RAN1 has decided that SSB adaption is applicable to the PCell.

The second main bullet confirms that SSB adaptation is applicable to SCell. It confirms that SSB adaptation is supported for the SCell on the sync raster (B2’) and outside the sync raster (B3’) if the SSB is an NCD-SSB. SSB adaption for SCell if the SSB is an CD-SSB is not yet agreed in RAN1.

Observation 2: RAN1 confirmed that SSB adaptation is supported for the SCell on the sync raster (B2’) and outside the sync raster (B3’) if the SSB is an NCD-SSB

Observation 3: SSB adaption for SCell if the SSB is an CD-SSB is not yet agreed in RAN1.

Hence, we repeat our proposal form RAN4 #112 [3]:

Proposal 2: RAN4 should start investigating requirements for SSB adaptation for UEs in connected mode that do not imply that the SSB is a CD-SSB.

RAN4 already defines minimum requirements for the measurement procedures at transitions, e.g., in case that the intra-frequency measurement object transits from measurements outside gaps to measurements inside gaps or when the DRX cycle periodicity changes, see section 9.1.6 in [4]. Change of the periodicity of SSB is a similar transition. Hence, RAN4 should investigate what requirements are impacted by those periodicity transitions by SSB and PRACH adaptation in time domain, e.g., the measurement procedure for SSB based measurements.

Observation 4: RAN4 already defines minimum requirements for the measurement procedures at transitions in the configurations. Change of the periodicity of SSB is a similar transition.

Proposal 3: RAN4 should investigate what requirements are impacted by those periodicity transitions by SSB adaptation in time domain.

Figure 2-1 illustrates the impact of SSB periodicity adaption for the example of a measurement period. Before and after the transition, the measurement period depends on the periodicity P1 and P2 and is longer the longer the periodicity is. If the SSB adaptation happens in between a measurement period, two periods have to be taken into account.

Figure 2-1: Measurement Periods with and without Transitions

If the measurements are combined across multiple SSB instances during the measurement period, a measurement period with transition implies that measurements with different periodicities need to be combined, e.g., filter coefficients in a filter may need to be adapted to take the different time gaps into account. This implies a certain impact to the implementation and a certain additional complexity. Maybe also defining new accuracy requirements need to be considered.

Alternatively, the start of the measurement period could be shifted till after the transitions has happened and a full measurement periodicity with the new periodicity P2 is available again. Instead of requesting the UE to combine measurements with different periodicities, the measurement cycle could also be defined based on maximum of the two periodicities P = max{P1, P2}.

RAN4 should further discuss which of the two options is more feasible and trade-off benefits versus impacts. In current specification for transition [4], the second approach has been followed. We propose to align with this approach. Only if clear benefits of measurement combining can be shown, RAN4 should further analyze how such requirements for measurement combining could be defined.

Proposal 4: Only if clear benefits of measurement combining can be shown, RAN4 should further analyze how such requirements for measurement combining could be defined.

Observation 5: A measurement period with transition in periodicity of the SSB implies that measurements with different periodicities need to be combined

Observation 6: Instead of requesting the UE to combine measurements with different periodicities, the measurement cycle could also be defined based on maximum of the two periodicities P = max{P1, P2}.

Proposal 5: RAN4 should further discuss which of the two options, i.e., combing of measurements or applying the maximum of the two periodicities, is more feasible and trade-off benefits versus impacts.

The second discussion in RAN1 #118 is related to Cell DTX extension to SSBs on non-sync raster. Several options have been under discussion, but no clear agreement has yet been made in RAN1. Hence, we propose to delay any discussion in RAN4 related to Cell DTX to a later point in time.

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options

- Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period

- Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period

- Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

Observation 7: No clear agreement has yet been made in RAN1 how Cell DTX relates to adaptation of common signals / channels.

Proposal 6: RAN4 should delay any discussion related to Cell DTX to a later point in time.

# Conclusions

In this paper, we provided our views on several of the open issues identified in RAN4 #112. Our observations and proposals are summarized as follows:

Observation 1: RAN1 has not yet agreed whether SSB adaption in time domain is applicable for a PCell.

Proposal 1: RAN4 should postpone all work on SSB adaptation related to the PCell till RAN1 has decided that SSB adaption is applicable to the PCell.

Observation 2: RAN1 confirmed that SSB adaptation is supported for the SCell on the sync raster (B2’) and outside the sync raster (B3’) if the SSB is an NCD-SSB

Observation 3: SSB adaption for SCell if the SSB is an CD-SSB is not yet agreed in RAN1.

Proposal 2: RAN4 should start investigating requirements for SSB adaptation for UEs in connected mode that do not imply that the SSB is a CD-SSB.

Observation 4: RAN4 already defines minimum requirements for the measurement procedures at transitions in the configurations. Change of the periodicity of SSB is a similar transition.

Proposal 3: RAN4 should investigate what requirements are impacted by those periodicity transitions by SSB adaptation in time domain.

Proposal 4: Only if clear benefits of measurement combining can be shown, RAN4 should further analyze how such requirements for measurement combining could be defined.

Observation 5: A measurement period with transition in periodicity of the SSB implies that measurements with different periodicities need to be combined

Observation 6: Instead of requesting the UE to combine measurements with different periodicities, the measurement cycle could also be defined based on maximum of the two periodicities P = max{P1, P2}.

Proposal 5: RAN4 should further discuss which of the two options, i.e., combing of measurements or applying the maximum of the two periodicities, is more feasible and trade-off benefits versus impacts.

Observation 7: No clear agreement has yet been made in RAN1 how Cell DTX relates to adaptation of common signals / channels.

Proposal 6: RAN4 should delay any discussion related to Cell DTX to a later point in time.

# References

- R4-2412508, Work plan for R19 NES, RAN4 #112, August 2024

- R4-2413885, WF on Netw_Energy_NR_enh, RAN4 #112, August 2024

- R4-211824, On the RRM impact of enhanced network energy saving, Qualcomm, RAN4 #112, August 2024

- TS 38.133, Requirements for support of radio resource management, v18.6.0, June 2024