# 3GPP TSG-RAN WG4 Meeting #112bis	          R4-2415305

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.21.2.1

Source: 		Qualcomm Incorporated

Title: 	Discussion on AIoT deployment scenarios and spectrum usage

Document for:	Approval

# Introduction

In RAN4#112, the agreements on deployment scenarios and spectrum usage were captured in [1]. In this paper, we provide further views on AIoT deployment scenarios and spectrum.

# 2. Discussion

In RAN4#112, the following deployment scenario and spectrum usage is considered as an optional case in [1] to solve the coexistence issue from reader to NR UL when DL spectrum is used for R2D.

| No. | Deployment scenario and topology | Spectrum | (Aggressor -> Victim) | Note | Coexistence summary |
| --- | --- | --- | --- | --- | --- |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R | Same with scenario 1-2, Marginal |
|  |  |  | NR DL->reader | D2R | Same with scenario 1-2, SINR degradation @5-ile is 2dB, 50-ile is 0.8dB with 140dB CW cancellation capability |
|  |  |  | Reader->NR UL | R2D | 8.73% mean T-put loss, cell edge T-put is N/A due to outage |
|  |  |  | NR UL->device | R2D | 1.78dB for 5-ile SINR degradation, 1.22dB for 50-ile SINR degradation |

Based on the coexistence results in above table, using FDD UL spectrum for R2D can avoid serious interference from R2D to NR UE receiver. Although R2D on UL spectrum will lead to NR UL interfering device and reader interfering NR UL, the coexistence problem is controllable according to the simulation results in [2]. Therefore, to solve the interference coexistence issue caused by deploying NR UE indoor, RAN4 should consider using UL spectrum for R2D in D1T1 as a potential deployment solution.

Proposal 1: RAN4 should consider using UL spectrum for R2D as a potential deployment solution to solve the interference coexistence issue in D1T1 with legacy UE indoor.

In [1], it was agreed to consider two active readers for D1T1 and D2T2 in the simulation.

Issue 4-2-1: Active rate of reader for D1T1 and D2T2 Agreement: - Minimum distance between active readers: 60m as baseline, other values can be reported by other companies. - 2 readers are activated in one drop as baseline

Since only two readers can be activated at the same time (60m apart), when the readers are activated, the device access to the reader is randomly selected from candidate devices whose R2D receiving power meets the -36dBm Device receiver sensitivity. As can be seen from the Figure 1 and 2 below, the maximum distance distribution between the device and the reader can reach 60m. And outage probability for R2D and D2R are 30% and 50% respectively assuming 10dB required SNR for R2D and D2R.

Observation 1: Outage probability for R2D and D2R are 30% and 50% respectively assuming 10dB required SNR for R2D and D2R.

Figure 1: D1T1, Outdoor NR DL to R2D, maximum distance between reader and device is 60m

Figure 2: D1T1, Outdoor NR DL to D2R, maximum distance between reader and device is 60m

However, if a more advanced access control algorithm is considered when a device accesses a reader, such as allowing the device to access a reader that is closer to it at a certain time in the future, the performance of the system can be improved as shown in Figure 3 and 4.

Figure 3: D1T1, Outdoor NR DL to R2D, maximum distance between reader and device is 15m

Figure 4: D1T1, Outdoor NR DL to D2R, maximum distance between reader and device is 15m

Observation 2: The device access control algorithms have a significant impact on system performance in AIoT coexistence study.

Proposal 2: The device access control algorithms should be studied and confirmed to ensure that coexistence simulations match AIoT real deployments.

# 3.	Conclusion

In this paper, we provide our views on AIoT deployment scenarios and spectrum usage. The following observations and proposals are made:

Proposal 1: RAN4 should consider using UL spectrum for R2D as a potential deployment solution to solve the interference coexistence issue in D1T1 with legacy UE indoor.

Observation 1: Outage probability for R2D and D2R are 30% and 50% respectively assuming 10dB required SNR for R2D and D2R.

Observation 2: The device access control algorithms have a significant impact on system performance in AIoT coexistence study.

Proposal 2: The device access control algorithms should be studied and confirmed to ensure that coexistence simulations match AIoT real deployments.

# 4.	References

- R4-2414304, WF on Co-existence study for ambient IoT, CMCC

- R4-24xxxxx, Discussion on Ambient IoT Coexistence Evaluation in NR, Qualcomm

- TR 38.863