# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415307

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.8.2.1

Source: 		Qualcomm Incorporated

Title: 	On coexistence study for NTN HPUE

Document for:	Approval

# Introduction

The new Rel-19 WI on enhanced requirements and test methodology for NR and IoT NTN was approved in [1] in RAN #103. In RAN4#112, coexistence study for NTN HPUE was discussed and the agreements were captured in WF [2]. In this paper, we provide our further views on coexistence study for NTN HPUE.

# 2. Discussion

## 2.1 Scenario for coexistence study

In [2], we have the following agreements on co-ex scenarios.

Issue 2-2-3: Co-ex scenario # to be studied No.; Combination; Aggressor; Victim; Notes; Study Phase. 1; TN with NTN; TN DL; NTN DL; Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD.; Phase 1. 2; TN with NTN; TN UL; NTN UL; Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD.; Phase 1. 3; TN with NTN; NTN DL; TN DL; Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD.; Phase 1. 4; TN with NTN; NTN UL; TN UL; Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD.; Phase 1. 5; TN with NTN; NTN UL; TN DL; Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD.; Phase 1. 6; TN with NTN; TN DL; NTN UL; Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD.; Phase 1. Agreement: - Consider both Scenario 4 and 5 for co-existence study - Prioritize scenario 4 since it is the worst case - Do not consider Scenario 1, 2, 3 and 6

It was agreed to consider both Scenario 4 (S4) and Scenario 5 (S5) for NTN HPUE co-existence study. Scenario 4 is prioritized as it was recognized as the worst case. In Table 1, we provide the preliminary simulation results for Scenario 4 and 5 without isolation distance and meanwhile the wrap-around is enabled.

Table 1: Required ACIR for Scenario 4 and 5 without 1.5km separation, wrap-around enabled

| ACIR | 0 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S5 no iso avg-THP | 31.3 | 18.1 | 8.6 | 3.5 | 1.2 | 0.4 | 0.1 | - | - | - |
| S5 no iso edge –THP | 81.8 | 57.9 | 30.47 | 12.8 | 4.5 | 1.6 | 0.6 | - | - | - |
| S4 avg –THP - no iso | 24.9 | 19.3 | 14.5 | 10.5 | 7.5 | 5.1 | 3.3 | 2.1 | 1.3 | 0.7 |
| S4 edge –THP - no iso | 100 | 100 | 86.1 | 72.6 | 55.7 | 38.6 | 24.8 | 15.4 | 8.3 | 3.7 |

According to the simulations in Table 1, there is no issue in Scenario 5 with 20dB ACIR even without isolation distance and with wrap-around enabled. Therefore, we have the following proposal:

Proposal 1: RAN4 can confirm Scenario 4 is the worst case for the NTN HPUE coexistence study.

## 2.2 Deployment and layout

The following agreements were captured in [1] regarding the isolation distance and wrap-around functionality.

Issue 2-3-1 NTN and TN network isolation distance Agreement: - Consider isolation distance in scenario 4 and 5 for both NR-NTN HPUE coex and IoT-NTN HPUE coex. - Use the isolation distance 1.5km as the starting point - Other values for isolation distance are not precluded Issue 2-3-2 TN network topology - Agreement: Reuse existing method from TR 38.863 as baseline, and encourage companies to further evaluate any performance impact with and w/o wrap-around functionality.

To evaluate the performance impact with and without warp-around functionality, we compare the simulation results with and without wrap-around functionality for Scenario 4. Note that in the below results, we did not consider the isolation distance. If the isolation distance is further considered, the Throughput loss will be further reduced.

Table 2: Required ACIR for Scenario 4 with and without wrap-around with no isolation distance

| ACIR | 0 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S4 avg –THP - no iso, wraparound enable | 24.9 | 19.3 | 14.5 | 10.5 | 7.5 | 5.1 | 3.3 | 2.1 | 1.3 | 0.7 |
| S4 edge –THP - no iso Wraparound enable | 100 | 100 | 86.1 | 72.6 | 55.7 | 38.6 | 24.8 | 15.4 | 8.3 | 3.7 |
| S4 avg –THP - no iso, wraparound disable | 9.2 | 5.9 | 3.5 | 2 | 1.1 | 0.5 | 0.3 | - | - | - |
| S4 edge –THP - no iso Wraparound disable | 52.9 | 34.2 | 19.8 | 10.1 | 4.5 | 2.2 | 1.1 | - | - | - |

As shown in Table 2, without wrap-around functionality, the required ACIR for Scenario 4 is less than 30dB even without 1.5km isolation distance.

In summary, as we did in Scenario 1 in NTN coexistence studies Rel-17, Wrap around should be turned off. Because it doesn’t make sense that we are isolating NTN UE on TN edge cluster or assuming isolation distance and we have infinitely Network on the edges with Wrap around.

Observation 1: Wrap-around functionality has a significant impact on the NTN HPUE coexistence evaluations. From a realistic deployment perspective, the wraparound function should be disabled since we are looking at the coexistence scenario in which NTN UEs are distributed at the boundary of TN coverage.

Proposal 2: The wraparound function should be disabled in the Rel-19 NTN HPUE coexistence study.

# 3.	Conclusion

In this paper, we provide our views on the coexistence study for NTN HPUE. The following proposals and observation are made:

Proposal 1: RAN4 can confirm Scenario 4 is the worst case for the NTN HPUE coexistence study.

Observation 1: Wrap-around functionality has a significant impact on the NTN HPUE coexistence evaluations. From a realistic deployment perspective, the wraparound function should be disabled since we are looking at the coexistence scenario in which NTN UEs are distributed at the boundary of TN coverage.

Proposal 2: The wraparound function should be disabled in the Rel-19 NTN HPUE coexistence study.

# 4.	References

- RP-240857, ‘New WID: Enhanced requirements and test methodology for NR and IoT NTN’

- R4-2414452, ‘WF on UE RF requirements for NTN HPUE’

- TR 38.863, ‘Non-terrestrial networks (NTN) related RF and co-existence aspects’