# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415311

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.10.2

Source: 		Qualcomm Incorporated

Title: 	On Rel-19 ATG UE RF requirements

Document for:	Approval

# Introduction

In RAN4#112, UE RF requirements for Rel-19 ATG enhancement were discussed, and the agreements are captured in WF on Rel-19 ATG UE RF requirements [1]. In this paper, we provide our views on inter-band CA and UL MIMO.

# 2. Discussion

## 2.1 Inter-band CA

The issue of whether to allow n39 UL for CA_n3A-n39A was discussed in RAN4#112, the following agreements were captured in [1].

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A Agreement: - Whether to allow n39 UL for CA_n3A-n39A depend on companies’ input. - The filter with [X] MHz frequency gap can be considered.

The implementation solutions have been extensively investigated for NR handheld UE and the conclusion is that n39 UL is not allowed in CA_n3-n39 [2]. In [3], the exemplary RF architectures with separate antenna and common antenna were analysed respectively assuming 5MHz frequency gap is reserved between DL band n3 and band n39.

In [3], the spurious emissions of n39 are specified as -15.5dBm/5MHz at 1885-1880MHz and -33dBm/5MHz at 1805-1885MHz, respectively. We could estimate that the leakage and PA noise from n39 to n3 is about-27dBm/5MHz with full 35MHz allocation. Therefore, even with 40dB attenuation which is not practical, the desense is roughly 30dB assuming 10dB antenna isolation.

Observation 1: With full 35MHz allocation, the desense is roughly 30dB even with 40dB attenuation which is not practical.

While with 5MHz transition band, n3 DL filter is almost not attenuation anything at n39 range starting from1885MHz. Therefore, the n3 receiver will receive almost full Tx power from n39 that will block n3 LNA.

Compared with handheld UE, CPE can have more flexibility to provide better antenna isolation thanks to the big form factor. But even with 40dB antenna isolation, the desense would be over 10dB based on above analysis.

Observation 2: Even with 40dB antenna isolation supported by CPE, the desense would be over 10dB.

According to the above observations, we can find that the desense cannot be ignored even with 5MHz transition band when enabling n39 UL for CA_n3A-n39A. Therefore, we have the following proposal.

Proposal 1: The results of desense with assumptions such as filter, PA noise, antenna isolation should be studied before deciding to enable n39 UL for CA_n3A-n39A with [X] MHz frequency gap.

Since all the analysis is based on the UE form factor being CPE, the conclusions about allowing n39 UL for CA_n3A-n39A if any should only apply to ATG and will not have any impact on existing NR specification for legacy NR UE.

Proposal 2: The conclusions about allowing n39 UL for CA_n3A-n39A if any should only apply to ATG and will not have any impact on existing NR specification for legacy NR UE.

## 2.2 UL MIMO

The issues of whether ULFPTx mode for UL-MIMO is supported for ATG UE were discussed in previous meetings.

Issue 4-1-1: ULFPTx mode for UL-MIMO Agreement: - FFS on whether to apply ULFPTx mode for ATG UE

The intention of ULFPTx mode is to increase UL power with 2Tx for NR UE. While for ATG, there is no UE power class, and the maximum output power of ATG UE is indicated by signalling. This means that the maximum transmit power of ATG UE can be reported based on actual capabilities with the sum of transmit power from all the connectors regardless of with or without ULFPTx. In addition, MPR and A-MPR requirements were agreed not to introduce for ATG UE. Therefore, it is not clear what’s the additional specification impact if introducing ULFPTx mode for ATG UL-MIMO.

Proposal 3: RAN4 to analyze the benefit and whether there will be additional specification impact if ULFPTx mode for ATG is introduced.

# 3.	Conclusion

In this paper, we provide our views on UE RF requirements in ATG enhancements. The following observations and proposals are made:

Observation 1: With full 35MHz allocation, the desense is roughly 30dB even with 40dB attenuation which is not practical.

Observation 2: Even with 40dB antenna isolation supported by CPE, the desense would be over 10dB.

Proposal 1: The results of desense with assumptions such as filter, PA noise, antenna isolation should be studied before deciding to enable n39 UL for CA_n3A-n39A with [X] MHz frequency gap.

Proposal 2: The conclusions about allowing n39 UL for CA_n3A-n39A if any should only apply to ATG and will not have any impact on existing NR specification for legacy NR UE.

Proposal 3: RAN4 to analyze the benefit and whether there will be additional specification impact if ULFPTx mode for ATG is introduced.

# 4.	References

- R4-2414322, WF on Rel-19 ATG UE RF requirements, CMCC

- TS 38.101-1 v18.6.0

- R4-2412955, Discussion on Rel-19 ATG UE RF with DL inter-band CA_n3-n39, Huawei