3GPP RAN WG4 Meeting #112bis		R4-2415316

Hefei, China, 14th – 18th October 2024

Agenda item:	6.2.4.2

Source:	Apple

Title:		Further input on the UE antenna characteristics and gains

WI/SI:	FS_NR_IMT_4400_7125_14800MHz

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

The WRC-23 conference identified three frequency ranges as potential IMT candidates and tasked ITU-R WP5D to conduct the corresponding sharing and compatibility studies. In turn, the ITU-R WP5D sent the LS to a number of bodies, including 3GPP, asking to provide technical parameters for the candidate frequencies [1]. In response to that LS from WP5D and based on the technical input from RAN WG4 [2], the 3GPP RAN#103 meeting approved a new SI [3], purpose of which will be to study and provide the corresponding IMT technical parameters.

During the previous RAN WG4 meetings a set of initial discussions took place and RAN WG4 concluded that for the 15GHz frequency range both FR1- and FR2-like approaches for the UE RF architecture will be studied further. And while the FR2-like approach is assumed to leverage existing FR2 antenna models and assumptions, there was no final view on which model should be taken for the FR1-like approach. There is an obvious option of using an omni-directional antenna, but an option of having more than one Tx antennas is not excluded either. For the latter approach, however, more technical input is needed on potential gains and antenna beam patterns. Thus, in this document we present our view on how multiple Tx antennas can be implemented and which gains we can anticipate in this case.

# 2	Antenna characteristics and gains

## 2.1	Antenna and EIRP gains

As discussed during the previous RAN WG4 meeting, the FR2-like antenna array has a number of challenges at 15GHz mostly because of strict assumptions on the minimum number of elements and their spacing, which in turn results in a large antenna array physical size. On the contrary to it, the FR1-like approach gives more flexibility to the OEM vendors because even if a device implements more than one Tx, there is an additional flexibility on where Tx antennas can be placed. On the other hand, it makes it more challenging to agree upon a specific FR1-like antenna model because antenna design and its locations will naturally impact the maximum gain and resulting beam patterns.

Figure 2.1-1: Exemplary placement of two patch antennas within a mobile device.

To estimate anticipated gains and antenna characteristics in a more or less realistic mobile device form factors, we consider two Tx antennas placed within a device as presented in Figure 2.1-1: there are two patch antennas, each capable of 23dBm, at the distance corresponding to approximately three wavelengths.  Figure 2.1-2 below presents separately distribution of antenna and EIRP gains for the considered antennas, where the baseline curve is only one Tx. Gain values are obtained by probing all points in 3D space around considered antennas, which allows us to find the best beam at each point of space independent of the base station location. In other words, these figures illustrate all possible gain values, including the best one, regardless of the phone orientation with respect to the base station. As can be seen, from the antenna gain perspective, the average gain comparing to one Tx is around 2.5dB. From the EIRP perspective, the average gain comparing to one Tx is around 5.5dB, which comprises 2.5dB antenna gain and 3dB coming from the fact that there are two Tx antennas each transmitting at 23dBm.  It is worth noting that these gain values are highly correlated with results from the UL MIMO TRP/TRS study. It also worth noting that we also checked gain values for a set of different distances between two patch antennas, for which we observed only marginal variations within +/- 1dB.

Figure 2.1-2: Antenna and EIRP gain for the considered two Tx antennas.

## 2.2	Antenna radiation pattern

Another aspect for the FR1-like antenna design is the resulting radiation pattern, which is needed at least for the co-existence simulations. It is worth noting that unlike at the antenna and EIRP gain, which are not very sensitive for the actual placement of Tx antennas, the radiation pattern may vary a lot. For instance, Figure 2.2-1 below presents the resulting beam pattern from two Tx antennas physical location of which is the same as in Figure 2.1-1. For the sake of better clarity, we present resulting the resulting radiation pattern in 2D and 3D spaces. Since two Tx antennas are placed at the back side of the phone, the resulting radiation pattern has a big negative gain in the opposite direction.

Figure 2.2-1: Radiation pattern for two Tx antennas presented in Figure 2.1-1.

Nevertheless, for the given mobile device and its form factor, the resulting radiation pattern will depend a lot on the antenna placement and the relative distance between them. As an example, Figure 2.2-2 below presents normalized radiation patterns for two patch antennas at the distance of one, two, and three wave-length. So as mentioned earlier, the resulting pattern may vary a lot for two Tx, not mentioning the case of e.g. four Tx with different relative distances between antennas.

Figure 2.2-2: Radiation pattern for two Tx antennas at the distance one (left), two (center) and three (right) wave-length.

It should be acknowledged that while it was possible to agree on the anticipated distance between antenna elements in the FR2 UE antenna model, it is not likely to assume that all OEMs can assume certain minimum or maximum spacing between patch antennas. For instance, our results assume the three-wavelength distance between two Tx antennas, but other implementations can have smaller or larger distance; not evening mentioning more complex arrangements if e.g. four Tx antennas are assumed. Based on that we suggest following the approach when 3GPP agrees on a single patch antenna model and its gain, as presented in Table 2.2-1 below. The actual arrangement of patch antennas, or at least the distance between them, can be declared by a proponent of the co-existence simulations.

Table 2.2-1: Patch antenna model.

| Parameter | Values |
| --- | --- |
| Vertical patch antenna radiation pattern (dB) |  |
| Horizontal patch antenna radiation pattern (dB) |  |
| Combining method for 3D pattern (dB) |  |
| Maximum directional gain of each patch antenna GE,max | [2] dBi |

# 3	Conclusions

In this discussion paper we presented our further considerations on the practical aspects of the FR1-like antennas for the 15GHz co-existence studies. Depending on the number of Tx antennas, i.e. two in our studies, the resulting gain can be approximated accordingly, which does not depend much on the distance between antennas. On the contrary to it, the radiation pattern depends a lot on the number of Tx antennas and their relative location. As it might be quite difficult to agree on a certain distance between the antennas, the resulting FR1-like antenna architecture can be abstracted through the antenna model of the individual patch, whereupon the number of the antennas and the distance between them will be provided by a proponent of the simulation results.

Proposal 1:	The FR1-like antenna model can be abstracted by an individual patch antenna model, number of patch antennas and distance between them.

Proposal 2:	Individual patch antenna model can follow the same model as FR2 antenna array element with the gain factor of [2] dB

# 4	References

RP-240024, "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz", ITU-R WP5D

R4-2102840, "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)", RAN WG4.

RP-240765, "Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz"

3GPP TR 38.921, "Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz", v17.1.0