3GPP RAN TSG-RAN4 Meeting # 112-Bis		R4-2415324

Hefei, China, October 14 – 18, 2024

Agenda item:	6.1.1.3.3

Source:	Apple

Title:	On the values of Delta_TRxSRS for 6Rx SRS antenna switching

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting (Meeting #112, Maastricht, Netherlands, August 19 – August 23, 2024), discussions were held on the requirements for 6Rx for handheld and FWA, which are parts of the new  Release 19 WID on RF enhancements for NR FR1/FR2 and EN-DC [1]. As captured in the meeting WF [2], one of the 6Rx aspects that  were greatly debated is the SRS antenna switching and the values of ΔTRxSRS, with the discussion summary highlighted below.

## Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way Forward: RAN4 agrees to adopt the average values for Bands n41, n77, and n78. For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers. - Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS for bands whose FUL_high is lower than the FUL_low of n79. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n41, n77, n78; 4.0; 3.5; 4.5. - Option 10: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79, n104; 5.4; 4.7; 5.9. - Option 11: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79; 5.4; 4.7; 5.9. Band n104; 5.7; 5.0; 6.3.

As can be seen from the highlighted WF summary above, the proposed values for ΔTRxSRS were based on the three SRS antenna switching configurations t1r6, t2r6, and t1r6-t2r6. Those were the main antenna configurations captured in the original WID [1].

However, in the June RAN plenary meeting (3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024), the original NR_ENDC_RF_Ph4 WID was modified to add t3r6 antenna configuration as captured in the WF summary shown below [3].

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this meeting, we modified the proposal we summitted in the previous RAN4 meeting [4] by adding ΔTRxSRS requirements for t3r6 and t1r6-t3r6 antenna configurations.

# Discussion

For SRS antenna switching, RAN4 defines ΔTRxSRS as the maximum allowed relaxation of the maximum configured transmitted power PCMAX,f,c due to the additional insertion loss differences between the main TX path and a given diversity branch. As was highlighted in [3], the main UE integration factors that contribute to these insertion loss differences are the losses on antenna switches, PCB traces, and the losses of the RF antenna cable located between the RF front-end ( with the integrated PA) on the main board and the antenna connector of the ports being sounded located on a separate RF board. The insertion losses of each component within any SRS branch vary depending on the suppliers. Also, these losses are frequency-dependent. As was previously used for 8Rx, we can split the bands into two frequency ranges: Bands whose FUL_high is lower than the FUL_low of n79 (n41, n77, n78) , and bands whose FUL_high is higher than the FUL_low of n79 (n79, n104).

- Antenna switches: Depending on the antenna architecture, different RF switch topologies ( SPST, SPDT, SP3T, SP4T, DP3T, SP6T etc.) could be used on different RX diversity branches. As depicted in Figure 2.1-1 below for example, the architecture shown on the left uses an SP6T switch while the one on the right uses an SP4T and an SP3T. Typically, a switch with a higher number of throws exhibits higher insertion loss compared to the one with lower number of throws, when the same process technology is used. Table 2-1 shows the assumed worst case insertion losses on several antenna switch topologies, based on a typical high-frequency SOI process.

- PCB trace losses: PCB trace is used on the main board for the connections between the Tx signals and to antennas.  These losses will be different between the main antenna path and the Rx diversity paths, due to their relative positions within the UE. We assume a trace length of 60mm to derive the worst-case losses on the PCB traces.

- RF cable losses: An RF coaxial cable is used from main board to antennas on the sub-board. We will assume that the antennas will be fed by semi rigid coax cables with length up to 200mm worst case and with an average cable jacket diameter of 1.32mm as was assumed in [4].

Table 2-1 shown below summarizes the insertion loss assumptions that we will use to derive the values of ΔTRxSRS.

| Parameter | 3.5GHz | 4.9GHz | [7.1GHz] |
| --- | --- | --- | --- |
| SP2T IL | 0.6dB | 0.9dB | 1.3dB |
| SP3T IL | 0.7dB | 1.0dB | 1.4dB |
| SP4T IL | 0.8dB | 1.1dB | 1.5dB |
| SP6T IL | 0.9dB | 1.2dB | 1.7dB |
| DP3T IL | 0.9dB | 1.2dB | 1.5dB |
| 60-mm PCB trace IL (SRS path) | 1.8dB | 2.4dB | 3.0dB |
| RF cable loss | 0.8dB | 1.1dB | 1.4dB |

Table 2-1: IL loss assumptions for the main contributors on ΔTRxSRS

  - 1T6R SRS antenna switching configuration

Below are shown two possible antenna configurations for the 1T6R SRS antenna switching. We will consider the configuration on the right (with SP4T + SP3T) to derive the insertion loss link budget

Using the assumptions shown in Table 2-1, the ΔTRxSRS can be computed as follows for the 5 antenna ports 2, 3, 4, 5, 6:

- For bands n41, n77, and n78: ΔTRxSRS = SP3T_IL + SP2T_IL + PCB_IL  + IL_Cable = 0.7 + 0.6 + 1.8 + 0.8 = 3.9dB

- For bands n79 and n104: ΔTRxSRS = SP3T_IL + SP2T_IL + PCB_IL + IL_Cable = 1.0 + 0.9 + 2.4 + 1.1 = 5.4dB

Proposal 1: For 1T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS = 3.9dB

- For n79 and n104, ΔTRxSRS = 5.4dB

Figure 2.1-1: 1T6R antenna configurations using an SP6T (left figure) and SP4T+SP3T (right figure)

  - 2T6R SRS antenna switching configuration

The diagram shown in Figure 2.2-1 can be used for the 2T6R SRS antenna switching configuration.

Figure 2.2-1: 2T6R antenna configuration for SRS antenna switching

Using the assumptions shown in Table 2-1, the ΔTRxSRS can be computed as follows for the 5 antenna ports 2, 3, 4, 5, 6:

- For bands n41, n77, and n78: ΔTRxSRS = SP2T_IL + PCB_IL + IL_Cable = 0.6 + 1.8 + 0.8 = 3.2dB

- For bands n79 and n104: ΔTRxSRS = SP2T_IL + PCB_IL + IL_Cable = 0.9 + 2.4 + 1.1= 4.4dB

Proposal 2: For 2T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS =3.2dB

- For n79 and n104, ΔTRxSRS =4.4dB

  - 1T6R-2T6R SRS antenna switching configuration

The diagram shown in Figure 2.3-1 can be used for the 1T6R-2T6R SRS antenna switching configuration.

Figure 2.3-1: 1T6R-2T6R antenna configuration for SRS antenna switching

Using the assumptions shown in Table 2-1, the ΔTRxSRS can be computed as follows for the 5 antenna ports 2, 3, 4, 5, 6:

- For bands n41, n77, and n78: ΔTRxSRS = DP3T_IL + SP2T_IL + PCB_IL + IL_Cable = 0.7 + 0.9 + 1.8 + 0.8 = 4.2dB

- For bands n79 and n104: ΔTRxSRS = DP3T_IL + SP2T_IL + PCB_IL  + IL_Cable = 1.0 + 1.2 + 2.4 + 1.1 = 5.7dB

Proposal 3: For 1T6R-2T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS =4.2dB

- For n79 and n104, ΔTRxSRS =5.7dB

  - 3T6R SRS antenna switching configuration

The diagram shown in Figure 2.3-1 can be used for t3r6 SRS antenna switching configuration.

Figure 2.3-1: 3T6R antenna configuration for SRS antenna switching

Using the assumptions shown in Table 2-1, the ΔTRxSRS can be computed as follows for the 5 antenna ports 2, 3, 4, 5, 6:

- For bands n41, n77, and n78: ΔTRxSRS = SP2T_IL + PCB_IL + IL_Cable = 0.7 + 0.9 + 1.8 = 3.3dB

- For bands n79 and n104: ΔTRxSRS = SP2T_IL + PCB_IL  + IL_Cable = 0.9 + 1.1 + 2.4 = 4.4dB

Proposal 4: For 3T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS = 3.3dB

- For n79 and n104, ΔTRxSRS = 4.4dB

  - 1T6R-3T6R SRS antenna switching configuration

The diagram shown in Figure 2.3-1 can be used for t1r6-t3r6 SRS antenna switching configuration.

Using the assumptions shown in Table 2-1, the ΔTRxSRS can be computed as follows for the 5 antenna ports 2, 3, 4, 5, 6:

- For bands n41, n77, and n78: ΔTRxSRS = SP4T_IL+DPDT_IL+ SP2T_IL +PCB_IL + IL_Cable = 0.8 + 0.8 + 0.7+0.9+1.8 = 5.0dB

- For bands n79 and n104: ΔTRxSRS = SP4T_IL+DPDT_IL+ SP2T_IL +PCB_IL + IL_Cable = 0.9 + 0.9 + 0.8+1.1+2.4 = 6.1dB

Proposal 5: For 1T6R-3T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS = 5.0dB

- For n79 and n104, ΔTRxSRS = 6.1dB

# 4	Conclusions

Proposal 1: For 1T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS = 3.9dB

- For n79 and n104, ΔTRxSRS = 5.4dB

Proposal 2: For 2T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS =3.2dB

- For n79 and n104, ΔTRxSRS =4.4dB

Proposal 3: For 1T6R-2T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS =4.2dB

- For n79 and n104, ΔTRxSRS =5.7dB

Proposal 4: For 3T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS = 3.3dB

- For n79 and n104, ΔTRxSRS = 4.4dB

Proposal 5: For 1T6R-3T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS = 5.0dB

- For n79 and n104, ΔTRxSRS = 6.1dB

# 4	References

RP-240828 “New WID: UE RF enhancements for FR1/FR2 and EN-DC , Phase 4”, by RAN4 chair (Huawei),  3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

R4-24142895, “WF on requirements for 6Rx”, by AT&T, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, August 19-23, 2024

RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, by Huawei, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024

R4-2411151, “On the values of ΔTRxSRS for 6Rx SRS antenna switching”, by Apple, 3GPP RAN TSG-RAN4 Meeting # 112, Maastricht, NL, August 19 – 23, 2024