3GPP RAN WG4 Meeting #112bis																R4-2415326

Hefei, China, 14th – 18th October 2024

Agenda item:	6.1.1.2.1

Source:	Apple

Title:	On Rel-19 power domain enhancement for FR1

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

Power domain enhancement is a re-occurring topic and enters the 4th phase when starting the Rel-19 work. The objectives are covered in the WID [1]. This contribution focuses on the high-level aspects of MPR reduction. The WID lists those goals as:

# 2  Discussion

##### 2.1 Converting outer RB allocations into inner RB allocations

RAN4 found that one strategy to improve output power is the conversion of outer RB allocations into inner RB allocations. There are two main approaches. The first one extends the UE CBW by a certain amount. The out-of-band emission requirements would be applied with respect to the extended bandwidth. The updated requirements would allow certain allocations to increase output power as those might not be limited anymore by ACLR, SEM or spurious. If all outer RB allocations shall be improved, then a minimum extension factor of two is required. With a factor of two the UE channel needs to be symmetrically extended. Otherwise, some RB allocations remain limited by ACLR or SEM. The second approach is to apply the BS CBW to the UE. The network indicates the BS CBW and a shift within the BS RBs to the UE RBstart.

When extending the UE CBW, the total bandwidth should be an element of the set of channel sizes specified in TS 38.101-1 clause 5.3.2. Any other combined bandwidth would provide issues for correctly applying the RF spectrum emission requirements and increase implementation complexity. The RF spectrum emission requirements are defined with respect to the channel sizes specified in clause 5.3.2. As a consequence, any extension which does not lead to a regular channel size would result into undefined emission requirements.

Proposal 1: In any case, when modifying the requirements by extending CBW or applying BS CBW to the UE RAN4 should consider the following two aspects:

- The BS CBW based inner/outer method and the CBW extension method need to be aligned with existing channel bandwidth sizes from TS 38.101-1 clause 5.3.2. No intermediate extension shall be allowed as it introduces increased complexity for implementation and undefined SEM and ACLR requirements.

- All improvements need to be checked against regulatory requirements. RAN4 needs to make sure that the feature is compliant with given emission requirements.

##### 2.2 Ratio size of extended CBW between UE CBW and larger BS channel BW

Last meeting our paper [3] explored improvements to MPR by extending the channel bandwidth. The limits for ACLR and SEM were not relaxed but all the different RF spectrum emission requirements were applied with respect to the extended channel. The initial analysis showed that if extending the channel size by a factor smaller than two it is possible to achieve considerable improvements for outer RB allocations. This approach has the advantage that gains of 0.5 to 1.0dB seem possible without sacrificing coexistence by altering ACLR and SEM.

Concerns have been raised that an extension by factor of two might violate ITU requirements for spurious emission. It seems that an extension factor smaller than two might be compliant. In our initial study we extended the UE CBW by 1/3. Other proposals from last meeting considered extensions of 1/4, 2/5 and 1/2. It should be checked whether those extension factors could be a solution for the raised concern.

Observation: Extending the UE CBW by a factor smaller than two could meet the raised concerns on the ITU requirements for spurious emission. In our initial study we extended the UE CBW by 1/3. It should be checked whether this and other extension factors could be a solution for the raised concern.

Proposal 2: Consider a study which analyses the MPR improvements by extending the UE CBW by a factor smaller than two. Check whether this approach could resolve the concerns raised on the spurious requirements.

# 4	Conclusions

This contribution provides certain considerations to the discussion on the MPR reduction which his part of the power domain enhancement WID. The following observations and proposals are made:

Proposal 1: When modifying the requirements by extending CBW or applying BS CBW to the UE RAN4 should consider the following two aspects:

- The BS CBW based inner/outer method and the CBW extension method need to be aligned with existing channel bandwidth sizes from TS 38.101-1 clause 5.3.2. No intermediate extension shall be allowed as it introduces increased complexity for implementation and undefined SEM and ACLR requirements.

- All improvements need to be checked against regulatory requirements. RAN4 needs to make sure that the feature is compliant with given emission requirements.

Observation: Extending the UE CBW by a factor smaller than two could meet the raised concerns on the ITU requirements for spurious emission. In our initial study we extended the UE CBW by 1/3. It should be checked whether this and other extension factors could be a solution for the raised concern.

Proposal 2: Consider a study which analyses the MPR improvements by extending the UE CBW by a factor smaller than two. Check whether this approach could resolve the concerns raised on the spurious requirements.

# 4	References

- RP-240828, “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, 3GPP TSG RAN Meeting #103

- R4-2410749, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #111

- R4-2411153, “On Rel-19 power domain enhancement”, Apple, Maastricht, Netherlands, August 19th – 23rd, 2024