3GPP RAN WG4 Meeting #112bis																R4-2415328

Hefei, China, 14th – 18th October 2024

Agenda item:	5.11.2

Source:	Apple

Title:	On simultaneous Rx/Tx for CA_n40A-n41A

WI/SI:	LTE_NR_R19_Simult_RxTx

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

A new work item [1] on simultaneous Rx/Tx has been agreed in RAN#104. One of its objectives is to clarify and improve existing mandatory simultaneous Rx/Tx requirements. This discussion paper discusses three main topics. The first topic is about the Rel-17 agreement on FR1+FR1 FDD-TDD combinations and the missing requirement in TS 38.101-1/3. The second topic is about the correct handling and request of fallbacks. The third topic is about CA_n40A-n41A and the requirements when supporting simultaneous Rx/Tx.

# 2  Discussion

#### 2.1 Background on issue of CA_n40A-n41A

A UE is required to feature a minimum of four Rx antenna ports for band n41. According to TS38.101-1 clause 7.2  this requirement applies if n41 is configured as standalone band or if n41 is part of a band combination. Due to near proximity of n40 and n41, those bands are typically grouped together using the same antennas and switches to reduce implementation complexity and the total number of required antennas.

In case of non-simultaneous Rx-Tx it is possible to share antennas between n40 and n41 as uplink and downlink transmission never occurs at the same time for both bands. However, in case of simultaneous Rx-Tx between n40 and n41 it is challenging to share antennas for uplink and downlink as filter isolation is marginal due to the close frequency separation of the two bands. Part of this issue comes from the required protection of the close-by 2.4GHz ISM band used for WIFI. On antenna side it is challenging to optimize and achieve the required performance for all the required antennas over the entire frequency of n40 and n41. If an implementation is done for MIMO support with simultaneous Rx/Tx then separate antennas are needed for each Tx and Rx path to optimize the overall performance for the entire frequency range. Integration of six or more antennas for this frequency range might not be an issue for FWA/CPE type devices but it is a challenging task for handhelds due to the small form factor.

Dependent on the RF front-end implementation a UE might not be able to support simultaneous Rx/Tx for CA_n40A-n41A together with MIMO operation. In this case the UE might either indicate sole support of MIMO or indicate sole support of simultaneous Rx/Tx for CA_n40A-n41A but not both capabilities. For such devices it might be beneficial to introduce relaxation to enable simultaneous support of MIMO and simultaneous Rx/Tx.

#### 2.2 Relaxation for CA_n40A-n41A with simultaneous Rx/Tx

The discussion during the first meeting indicated that relaxations to the 4Rx requirement on band n41 is not preferred as there exist concerns with respect to network performance. The proposed relaxation in [2] aims to enable relaxation for handheld UE only in the case when the bands n40 and n41 are part of the same configured band combination. In all other cases (i.e. single band operation, or band n41 being part of a band combo with any band other than n40) the handheld would have to support the mandatory four Rx antenna port requirement. Last meeting various proposals were made and captured within the WF [2]. The options for further discussion are provided below:

The way to handle the simultaneous Rx-Tx for CA_n40-n41 in Rel-19. - Option 1: Keep the requirements with n41 using 4Rx in simultaneous Rx-Tx for CA_n40-n41 - Option 2: Analyse the UE architecture with n41 using 2Rx in simultaneous Rx-Tx for CA_n40-n41, and try to define the separate requirement including MSD optionally from that in Rel-18, taking the implementation difficulties into consideration. - However, the UE should support 4Rx for n41 when the UE does not indicate simultaneous Rx-Tx for CA_n40-n41, i.e single n41 or other CA combo. - Option 3: Study whether new UE signalling(s) could be introduced for a certain band combination to indicate that: - concurrent support of 4x4 MIMO and simultaneous Rx/Tx is not supported - concurrent support of UL MIMO, and/or TxD and/or SRS antenna switching and simultaneous Rx/Tx is not supported - Other options are not precluded

We understand that network performance is an important concern and solutions with minimum impact should be selected. The discussion of last meeting identified two major solutions improving the situations of handhelds supporting MIMO and simultaneous Rx/Tx.

First, we would like to discuss option 3. The general idea is to introduce a new signalling capability which allows the UE to indicate that it does not feature concurrent support of MIMO and simultaneous Rx/Tx. There might be additional restrictions indicated with respect to TxD and SRS antenna switching. This approach is simple in the sense that it does not modify given requirements but introduces a restriction on how to combine their usage. The network can decide to operate the given band combination with either MIMO or simultaneous Rx/Tx. However, this approach has two major disadvantages. The first disadvantage is that if enabling MIMO, the network is restricted in its scheduling flexibility as it needs to synchronise n40/n41 TDD pattern. The second issue is that if the network decides to use simultaneous Rx/Tx it means that downlink in n41 only consists of single layer which considerably reduces the downlink throughput.

Observation 1: Introducing a new incapability signalling to indicate that it does not feature concurrent support of MIMO and simultaneous Rx/Tx is simple in the sense that it does not modify given UE capabilities but introduces a new restriction on how to combine their usage.

Observation 2: Restricting use of MIMO and simultaneous Rx/Tx has two major disadvantages. The first disadvantage is that if enabling MIMO, the network is restricted in its scheduling flexibility as it needs to synchronise n40/n41 TDD pattern. The second issue is that if the network decides to schedule TDD patterns with simultaneous Rx/Tx it means that (due to lack of MIMO) downlink in n41 only consists of single layer which considerably reduces the downlink throughput.

With the disadvantages of option 3 we would like to encourage further discussion on an alternative approach which introduces relaxation but still allows 2-layer MIMO on band n41. Due to small form factor of handhelds the devices are likely to share antennas between Tx and Rx. In case of concurrent 4-layer MIMO and simultaneous Rx/Tx the two Rx chains sharing antennas with Tx are expected to observe considerable de-sense. The combined sensitivity degradation on n41 can be up to 28.1dB. Such a high degradation has the potential to disrupt reception of the MIMO layers. While the degradation on the antennas which are sharing Tx/Rx is high the degradation on the remaining antennas is expected to be lower as the impact from the Tx is reduced by the 10dB antenna isolation. The core proposal is to use the two antennas which are less impacted and remove the ones with high de-sense from the equation.

The core proposal is that a handheld which signals support of 4-layer MIMO and simultaneous Rx/Tx shall only be expected to handle a maximum of 2-layer MIMO with 2Rx antenna ports if both capabilities are configured concurrently. The reduction to 2Rx antenna ports is required to avoid SRS antenna switching on the highly de-sensitised antenna ports which would share Tx and Rx. The advantage of this approach is that impact on downlink throughput is minimised as 2-layer MIMO might still be possible on band n41. This approach allows the UE to support 4-layer MIMO for non-simultaneous Rx/Tx while it is possible to indicate 2-layer MIMO support for the case of simultaneous Rx/Tx being scheduled. This is a considerable advantage over option 3 where band n41 would be reduced to single layer downlink operation in case of simultaneous Rx/Tx.

Table 1: MIMO operation for handhelds indicating 4-layer MIMO support on n41 for

CA_n40A-n41A with concurrent simultaneous Rx/Tx support

| TDD pattern with simultaneous Rx/Tx | CA_n40A-n41A MIMO operation |  |
| --- | --- | --- |
|  | n40 | n41 |
| No | 2-layer / 2Rx | 4-layer / 4Rx |
| Yes | 2-layer / 2Rx | 2-layer / 2Rx |

Observation 3: Due to small form factor of handhelds the devices are likely to share antennas between Tx and Rx. In case of concurrent 4-layer MIMO and simultaneous Rx/Tx the two Rx chains sharing antennas with Tx are expected to observe considerable de-sense. The combined sensitivity degradation on n41 can be up to 28.1dB. Such a high degradation has the potential to disrupt reception of the MIMO layers. While the degradation on the antennas which are sharing Rx/Tx is high the degradation on the remaining antennas is expected to be lower as the impact from the Tx is further reduced by the antenna isolation.

Proposal: A handheld which signals support of 4-layer MIMO and simultaneous Rx/Tx for CA_n40A-n41A shall only be expected to handle a maximum of 2-layer MIMO with 2Rx antenna ports if both capabilities are configured concurrently. The reduction to 2Rx antenna ports is required to avoid SRS antenna switching on the highly de-sensitised antenna ports sharing Tx and Rx. This approach allows the handheld to support 4-layer MIMO for non-simultaneous Rx/Tx while it is possible to indicate 2-layer MIMO support for the case of TDD patterns with simultaneous Rx/Tx. It is proposed to introduce a new signalling for this purpose. This signalling option does not apply to FWA form factor devices and UEs indicating 6Rx capability.

# 4	Conclusions

This contribution provides considerations on concurrent MIMO and simultaneous Rx/Tx operation. The following observations and proposals are made:

Observation 1: Introducing a new incapability signalling to indicate that it does not feature concurrent support of MIMO and simultaneous Rx/Tx is simple in the sense that it does not modify given UE capabilities but introduces a new restriction on how to combine their usage.

Observation 2: Restricting use of MIMO and simultaneous Rx/Tx has two major disadvantages. The first disadvantage is that if enabling MIMO, the network is restricted in its scheduling flexibility as it needs to synchronise n40/n41 TDD pattern. The second issue is that if the network decides to schedule TDD patterns with simultaneous Rx/Tx it means that (due to lack of MIMO) downlink in n41 only consists of single layer which considerably reduces the downlink throughput.

Observation 3: Due to small form factor of handhelds the devices are likely to share antennas between Tx and Rx. In case of concurrent 4-layer MIMO and simultaneous Rx/Tx the two Rx chains sharing antennas with Tx are expected to observe considerable de-sense. The combined sensitivity degradation on n41 can be up to 28.1dB. Such a high degradation has the potential to disrupt reception of the MIMO layers. While the degradation on the antennas which are sharing Rx/Tx is high the degradation on the remaining antennas is expected to be lower as the impact from the Tx is further reduced by the antenna isolation.

Proposal: A handheld which signals support of 4-layer MIMO and simultaneous Rx/Tx for CA_n40A-n41A shall only be expected to handle a maximum of 2-layer MIMO with 2Rx antenna ports if both capabilities are configured concurrently. The reduction to 2Rx antenna ports is required to avoid SRS antenna switching on the highly de-sensitised antenna ports sharing Tx and Rx. This approach allows the handheld to support 4-layer MIMO for non-simultaneous Rx/Tx while it is possible to indicate 2-layer MIMO support for the case of TDD patterns with simultaneous Rx/Tx. It is proposed to introduce a new signalling for this purpose. This signalling option does not apply to FWA form factor devices and UEs indicating 6Rx capability.

Table 1: MIMO operation for handhelds indicating 4-layer MIMO support on n41 for

CA_n40A-n41A with concurrent simultaneous Rx/Tx support

| TDD pattern with simultaneous Rx/Tx | CA_n40A-n41A MIMO operation |  |
| --- | --- | --- |
|  | n40 | n41 |
| No | 2-layer / 2Rx | 4-layer / 4Rx |
| Yes | 2-layer / 2Rx | 2-layer / 2Rx |

# 4	References

- RP-242088, “Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19”, Huawei, 3GPP TSG RAN Meeting #105

- R4-2411157, “On simultaneous Rx/Tx”, Apple, Maastricht, Netherlands, 19th – 23rd August, 2024

- R4-2412606, “WF on simultaneous Rx-Tx for CA_n40-n41”, Maastricht, Netherlands, 19th – 23rd August, 2024