3GPP RAN WG4 Meeting #112bis																R4-2415329

Hefei, China, 14th – 18th October 2024

Agenda item:	5.21.2

Source:	Apple

Title:	Initial considerations on EESS protection requirements

WI/SI:	NR_mmWave_protect-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The scope of the Rel-19 WI [1] is the specification of requirements allowing to protect EESS. The regional regulations seem to require an emission level of −35 dBW/200 MHz or -5dBm/200MHz inside the frequency region from 23.6GHz to 24GHz. Such requirements are appliable for China and EU region. This contribution provides initial results for the EESS protection requirements.

# 2  Discussion

##### 2.1 Simulation assumptions

The following assumptions and requirements for the simulations are applied:

- Power Class 3

- NR Waveform

- Calibration: 0dB MPR calibration: DFT-s-OFDM QPSK 100MHz, 20RB23, SCS=120kHz

- Carrier Leakage: 25dBc

- Image: 25dBc

- Emission requirement: -5dBm/200MHz from 23.6GHz to 24GHz

##### 2.2 Simulation results

This section presents initial simulation results of a 400MHz channel placed at the lower band edge of n258. The carrier frequency is at 24.45GHz. This leaves a 250MHz gap between the lower edge of the channel and the upper edge of the protected EESS range. The initial simulation results indicate that for a 400MHz channel up to 1dB power back-off might be needed.

Figure 1: Power back-off of QPSK modulated 400MHz carrier with fc at 24.45GHz

Observation: Initial simulation of a 400MHz channel placed at the lower band edge of n258 indicates that at least 1dB power back-off might be required for a fully allocated channel. Further analysis is needed to obtain a full picture for the protection of EESS.

# 4	Conclusions

This contribution provides an initial analysis on the power back-off need for the EESS protection requirement. Further analysis is required before final decision can be made.

Observation: Initial simulation of a 400MHz channel placed at the lower band edge of n258 indicates that at least 1dB power back-off might be required for a fully allocated channel. Further analysis is needed to obtain a full picture for the protection of EESS.

# 4	References

- RP-242371, “New WID on mmWave UE spurious emission”, China Unicom, Deutsche Telekom, Vodafone, CATT, Huawei, HiSilicon, OPPO, Qualcomm, Xiaomi, ZTE, 3GPP TSG RAN Meeting #105