3GPP RAN WG4 Meeting #112bis		R4-2415330

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.1.1.1.1

Source:	Apple

Title:	On Rel-19 HPUE CA/DC SAR compliance consideration

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the additional agreements were captured in the approved “WF on HPUE for CA in TN” [2]. On the SAR compliance aspect, there were whopping 9 proposals where most of them are focusing on UL duty-cycle approach for different UL power configurations.

In this contribution, we share our views on the SAR compliance aspect for HPUE CA/DC combinations and propose not to further consider UL duty-cycle with ΔPPowerClass for SAR mitigation from Rel-19 and going forward.

# Discussion

SAR (Specific Absorption Rate) is a measure of the rate of RF energy absorption by the body from the source being measured, specifically a handheld UE from 3GPP perspective. SAR limit is a regulatory requirement which is to ensure the human exposure to RF energy is below the safety level while using the device. The term “SAR” however has never been officially defined in 3GPP specifications, instead, it is simply referred as the “applicable electromagnetic energy absorption requirements” but without explicit value specified.

Observation 1: The term “SAR” has never been officially defined in 3GPP specifications, instead, it is simply referred as the “applicable electromagnetic energy absorption requirements” but without explicit value specified.

In 3GPP, the SAR issue has been made more aware since the HPUE was first introduced in LTE for handheld devices. In nearly every HPUE related work item, SAR mitigation has been included as an objective which had seemingly led to a misconception that SAR solution is only needed for HPUE, but not for default power class (PC3) or even lower power UE. In fact, even before the introduction of HPUE, SAR mitigation has already been considered in 3GPP specifications, which is known as the P-MPR solution. In our view, the SAR issue being made more aware for HPUE is due to that a new solution, also known as “duty-cycle” approach was specifically introduced for HPUE. As 3GPP has been focusing so much on the duty-cycle solution for almost every HPUE UL configuration, the most fundamental solution of using P-MPR was often overlooked or taken for granted.

Observation 2: In 3GPP, the SAR issue has been made more aware since the HPUE was first introduced in LTE for handheld devices.

Observation 3: In fact, even before the introduction of HPUE, SAR mitigation has already been considered in 3GPP specifications, which is known as the P-MPR solution.

Despite UL duty-cycle solution has drawn so much attention in every HPUE related work item and consumed substantial RAN4 efforts in the specifications development, especially for inter-band UL CA/DC, it has been realized that the approach is rather inefficient in UL performance and the applicability is also quite limited, and yet with lots of downsides, as is further elaborated below.

Observation 4: It has been realized that the UL duty-cycle solution with ΔPPowerClass is rather inefficient in UL performance and the applicability is also quite limited, and yet with lots of downsides.

First, the UL duty-cycle solution needs to be associated with a power reduction term, called ΔPPowerClass, which can only be 3dB or 6dB. For PC2 UE, when UL duty cycle exceeds the maximum limit, let’s say 50%, the UE configured maximum output power would be reduced by 3dB. Similarly, for PC1.5 UE, when UL duty cycle exceeds the maximum limit, let’s say also 50%, the UE configured maximum output power would be reduced by 6dB. Such large step of power reduction during the UL transmission could potentially cause UL reception failure at gNB or impact the UL performance.

Observation 5: The large step of power reduction ΔPPowerClass during the UL transmission could potentially cause UL reception failure at gNB or impact the UL performance.

Second, the UL duty-cycle solution with ΔPPowerClass has been causing a misconception of the so-called “power class fallback” in the current specifications where the UE configured maximum output power reduction would be mistakenly double counted, as brought up in [3].

Observation 6: The UL duty-cycle solution with ΔPPowerClass has been causing a misconception of the so-called “power class fallback” in the current specifications where the UE configured maximum output power reduction would be mistakenly double counted.

Third, the UL duty-cycle formula for inter-band UL CA/DC were less straightforward to comprehend which had taken rather lengthy discussions in RAN4 to agree on how to specify the requirements.

Observation 7: The UL duty-cycle formula for inter-band UL CA/DC were less straightforward to comprehend which had taken rather lengthy discussions in RAN4 to agree on how to specify the requirements.

Forth, since ΔPPowerClass can only be 3dB or 6dB, the UL duty-cycle approach would not be applicable to inter-band UL combinations supporting higherPowerLimit-r17 or higherPowerLimitMRDC-r17 for which RAN4 has agreed to only use P-MPR for SAR mitigation.

Observation 8: The UL duty-cycle approach with ΔPPowerClass would not be applicable to inter-band UL combinations supporting higherPowerLimit-r17 or higherPowerLimitMRDC-r17 for which RAN4 has agreed to only use P-MPR for SAR mitigation.

Owing to the above drawbacks, it is to our understanding that the UL duty-cycle solution with ΔPPowerClass has never been practically used by any UE in the field to mitigate SAR issue. On the other hand, there is also no conformance test case to verify the requirements. As a result, in our view it would not be worth RAN4’s time and efforts to continue introducing UL duty-cycle based solution with ΔPPowerClass for any new HPUE CA/DC UL configuration in Rel-19 and going forward.

Observation 9: The UL duty-cycle solution with ΔPPowerClass has never been practically used by any UE in the field to mitigate SAR issue.

Observation 10: There is no conformance test case to verify the UL duty-cycle with ΔPPowerClass requirements.

Observation 11: It would not be worth RAN4’s time and efforts to continue introducing UL duty-cycle with ΔPPowerClass based SAR solution for any new HPUE CA/DC UL configuration in Rel-19 and going forward.

Proposal: RAN4 do not consider UL duty-cycle with ΔPPowerClass based SAR solution for any new HPUE CA/DC UL configuration in Rel-19 and going forward.

# 3	Conclusion

In this contribution, we share our views on the SAR compliance aspect for HPUE CA/DC combinations and propose not to further consider UL duty-cycle with ΔPPowerClass for SAR mitigation from Rel-19 and going forward.

Observation 1: The term “SAR” has never been officially defined in 3GPP specifications, instead, it is simply referred as the “applicable electromagnetic energy absorption requirements” but without explicit value specified.

Observation 2: In 3GPP, the SAR issue has been made more aware since the HPUE was first introduced in LTE for handheld devices.

Observation 3: In fact, even before the introduction of HPUE, SAR mitigation has already been considered in 3GPP specifications, which is known as the P-MPR solution.

Observation 4: It has been realized that the UL duty-cycle solution with ΔPPowerClass is rather inefficient in UL performance and the applicability is also quite limited, and yet with lots of downsides.

Observation 5: The large step of power reduction ΔPPowerClass during the UL transmission could potentially cause UL reception failure at gNB or impact the UL performance.

Observation 6: The UL duty-cycle solution with ΔPPowerClass has been causing a misconception of the so-called “power class fallback” in the current specifications where the UE configured maximum output power reduction would be mistakenly double counted.

Observation 7: The UL duty-cycle formula for inter-band UL CA/DC were less straightforward to comprehend which had taken rather lengthy discussions in RAN4 to agree on how to specify the requirements.

Observation 8: The UL duty-cycle approach with ΔPPowerClass would not be applicable to inter-band UL combinations supporting higherPowerLimit-r17 or higherPowerLimitMRDC-r17 for which RAN4 has agreed to only use P-MPR for SAR mitigation.

Observation 9: The UL duty-cycle solution with ΔPPowerClass has never been practically used by any UE in the field to mitigate SAR issue.

Observation 10: There is no conformance test case to verify the UL duty-cycle with ΔPPowerClass requirements.

Observation 11: It would not be worth RAN4’s time and efforts to continue introducing UL duty-cycle with ΔPPowerClass based SAR solution for any new HPUE CA/DC UL configuration in Rel-19 and going forward.

Proposal: RAN4 do not consider UL duty-cycle with ΔPPowerClass based SAR solution for any new HPUE CA/DC UL configuration in Rel-19 and going forward.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2414277 “WF on HPUE for CA in TN”, Samsung, 3GPP TSG RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024

R4-2400180 “On misconception of power class fallback”, Apple, 3GPP TSG RAN WG4 Meeting #110, Athens, Greece, February 26th – March 1st, 2024