3GPP RAN WG4 Meeting #112bis		R4-2415332

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.1.1.1.3

Source:	Apple

Title:	On Rel-19 HPUE CA/DC MSD consideration

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the additional agreements were captured in the approved “WF on HPUE for CA in TN” [2]. For HPUE inter-band UL CA/EN-DC, there was the consideration of not specifying PC2/PC1.5 MSD requirements anymore but only relying on PC3 test configurations for MSD verifications or using some equations/look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD to hopefully ease the new HPUE CA/DC combination introducing process and simplify the RAN4 specifications.

In this contribution, we share our views on the possible handling for Rel-19 HPUE CA/DC MSD requirements and propose to maintain specifications consistency when considering not specifying HPUE MSD requirements from Rel-19.

# Discussion

The MSD requirements have been introduced since the unveiling of LTE inter-band CA combination in Rel-11. The similar framework was also adopted for NR inter-band CA and EN-DC. In LTE, since only power class 3 (PC3) is supported for inter-band CA combinations including DL only CA combinations, MSD requirements are always specified at PC3. However, in NR, there have been PC3 (default power class), power class 2 (PC2), and power class 1.5 (PC1.5) specified for inter-band combinations. Owing to the concern that MSD is expected to be higher at PC2 and PC1.5 than at PC3 for the same band combination, RAN4 has elected to define MSD requirements for all supported power classes of a band combination. And that resulted in numerous MSD test configurations and requirements as summarized in Table 2-1.

Observation 1: In LTE, MSD requirements are only specified at PC3 as only PC3 is supported for inter-band CA combinations.

Observation 2: In NR, RAN4 has elected to define MSD requirements for all supported power classes of a band combination.

| MSD Mechanism | PC | Aggressor UL Power (dBm) |
| --- | --- | --- |
| UL harmonic | PC3 | 23 |
|  | PC2 | 26 |
|  | PC1.5 | 29 |
| Harmonic mixing | PC3 | 23 |
|  | PC2 | 26 |
|  | PC1.5 | 29 |
| Cross-band interference | PC3 | 23 |
|  | PC2 | 26 |
|  | PC1.5 | 29 |
| 2UL IMD | PC3 | 20 + 20 |
|  | PC2 (2Tx or 3Tx) | 23 + 23 |
|  | PC1.5 (3Tx) | 23 + 27.8 |

Table 2-1 NR MSD test configurations

Compared to LTE, the introduction of HPUE CA/DC combinations in NR has at least tripled the MSD efforts for a combination supporting up to PC1.5 where the PC2 and PC1.5 MSD requirements would not be able to leverage the analysis already performed in LTE for the same or similar band combination. Furthermore, if we would follow the existing MSD framework for HPUE CA/DC combinations, more MSD analysis could be needed with new PC1.5 UL configurations in Rel-19. For example, if PC1.5 inter-band UL combination consisting of (PC2 FDD + PC2 TDD) with 2Tx or 3Tx would be introduced, then two more MSD analysis would be added.

Observation 3: Compared to LTE, the introduction of HPUE CA/DC combinations in NR has at least tripled the MSD efforts for a combination supporting up to PC1.5.

Observation 4: If we would follow the existing MSD framework for HPUE CA/DC combinations, more MSD analysis could be needed with new PC1.5 UL configurations in Rel-19.

Considering that the current MSD analysis has been known for consuming the most time and efforts in the new band combination introduction process, the necessity of HPUE CA/DC MSD requirements has not only increased the RAN4 workload substantially, but also been hindering the HPUE support for numerous CA/DC combinations due to the slow MSD analysis process. This concern has also triggered a recent RAN task to RAN4 to investigate a solution to enable HPUE maximum transmit power in downlink CA with single configured UL [3], though the similar issue also occurs for dual configured UL.

Observation 5: The necessity of HPUE CA/DC MSD requirements has not only increased the RAN4 workload substantially, but also been hindering the HPUE support for numerous CA/DC combinations due to the slow MSD analysis process.

Owing to the above concern, RAN4 has come up with the following considerations on handling HPUE MSD in Rel-19 to hopefully ease the new HPUE CA/DC band combination introducing process and simplify the RAN4 specifications.

- PC2/PC1.5 MSD requirements are not specified. UE shall meet the Rx requirements of default power class, with default power class transmission.

- Using some equations or look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD.

In our view, the second approach of using equations or look-up tables or other means to derive PC2/PC1.5 MSD from the respective PC3 MSD does not really resolve the aforementioned issue as specifying HPUE MSD requirements is still necessitated, and many companies could have already been using equations to derive the PC2/PC1.5 MSD from the respective PC3 MSD in the current process. On the other hand, if the PC2/PC1.5 MSD would be derived from the respective PC3 MSD, verifying PC3 MSD under the default power class should be sufficient as the RF components performance would be assumed to not vary with different power classes in the MSD derivation process.

Observation 6: Using equations or look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD would not solve the fundamental issue in the current HPUE introducing process as specifying HPUE MSD requirements is still necessitated.

The first approach would have avoided the aforementioned issue if it was taken since the HPUE CA/DC was first introduced. In our view, the MSD requirements are meant to verify the PA linearity, filter isolation, as well as receiver linearity performance which can already be verified by the PC3 MSD requirements in conjunction with the Tx requirements under the HPUE PCMAX configuration. There is essentially no need to further define separate MSD requirements other than PC3 MSD test configurations.

Observation 7: MSD requirements are meant to verify the PA linearity, filter isolation, as well as receiver linearity performance which can already be verified by the PC3 MSD requirements in conjunction with the Tx requirements under the HPUE PCMAX configuration.

Observation 8: There is essentially no need to further define separate MSD requirements other than PC3 MSD test configurations.

If RAN4 would consider the first approach in Rel-19, to maintain the specifications consistency, we propose to remove all the HPUE MSD requirements from the specifications going forward instead of only applying it to the existing PC3 band combinations and new band combinations to support HPUE. Removing all the HPUE MSD requirements not only helps save the all the HPUE basket WIDs for inter-band CA/DC combinations which have accounted for substantial RAN4 workload since Rel-17, but also avoids the need to use the rather cumbersome note attributes in the band combination configuration tables to indicate the support of HPUE. It would also avoid the delay of HPUE support due to the slow or procrastinated MSD analysis process. And the support of HPUE UL configurations for any band combination can simply be referred to the power class tables as specified in Clause 6 in the technical specifications [4,5].

Observation 9: Removing all the HPUE MSD requirements not only helps save the all the HPUE basket WIDs for inter-band CA/DC combinations which have accounted for substantial RAN4 workload since Rel-17, but also avoids the need to use the rather cumbersome note attributes in the band combination configuration tables to indicate the support of HPUE.

Observation 10: Removing all the HPUE MSD requirements also avoids the delay of HPUE support due to the slow or procrastinated MSD analysis process.

Observation 11: Without HPUE MSD requirements, the support of HPUE UL configurations for any band combination can be referred to the power class tables as specified in Clause 6 in the technical specifications.

Proposal: If RAN4 would consider not specifying the HPUE MSD requirements from Rel-19, to maintain the specifications consistency, it is proposed to remove all the HPUE MSD requirements from the specifications instead of only applying it to the existing PC3 band combinations and new band combinations to support HPUE.

# 3	Conclusion

In this contribution, we share our views on the possible handling for Rel-19 HPUE CA/DC MSD requirements and propose to maintain specifications consistency when considering not specifying HPUE MSD requirements from Rel-19.

Observation 1: In LTE, MSD requirements are only specified at PC3 as only PC3 is supported for inter-band CA combinations.

Observation 2: In NR, RAN4 has elected to define MSD requirements for all supported power classes of a band combination.

Observation 3: Compared to LTE, the introduction of HPUE CA/DC combinations in NR has at least tripled the MSD efforts for a combination supporting up to PC1.5.

Observation 4: If we would follow the existing MSD framework for HPUE CA/DC combinations, more MSD analysis could be needed with new PC1.5 UL configurations in Rel-19.

Observation 5: The necessity of HPUE CA/DC MSD requirements has not only increased the RAN4 workload substantially, but also been hindering the HPUE support for numerous CA/DC combinations due to the slow MSD analysis process.

Observation 6: Using equations or look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD would not solve the fundamental issue in the current HPUE introducing process as specifying HPUE MSD requirements is still necessitated.

Observation 7: MSD requirements are meant to verify the PA linearity, filter isolation, as well as receiver linearity performance which can already be verified by the PC3 MSD requirements in conjunction with the Tx requirements under the HPUE PCMAX configuration.

Observation 8: There is essentially no need to further define separate MSD requirements other than PC3 MSD test configurations.

Observation 9: Removing all the HPUE MSD requirements not only helps save the all the HPUE basket WIDs for inter-band CA/DC combinations which have accounted for substantial RAN4 workload since Rel-17, but also avoids the need to use the rather cumbersome note attributes in the band combination configuration tables to indicate the support of HPUE.

Observation 10: Removing all the HPUE MSD requirements also avoids the delay of HPUE support due to the slow or procrastinated MSD analysis process.

Observation 11: Without HPUE MSD requirements, the support of HPUE UL configurations for any band combination can be referred to the power class tables as specified in Clause 6 in the technical specifications.

Proposal: If RAN4 would consider not specifying the HPUE MSD requirements from Rel-19, to maintain the specifications consistency, it is proposed to remove all the HPUE MSD requirements from the specifications instead of only applying it to the existing PC3 band combinations and new band combinations to support HPUE.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2414277 “WF on HPUE for CA in TN”, Samsung, 3GPP TSG RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024

RP-241687 “Moderator's summary for offline discussion on CA Power Class Support”, Moderator (AT&T), 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17th – 20th, 2024

3GPP TS 38.101-1 V18.6.0 (2024-06)

3GPP TS 38.101-3 V18.6.0 (2024-06)