3GPP RAN WG4 Meeting #112bis		R4-2415333

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.19.2

Source:	Apple

Title:	On UE RF impact of 3Tx UL MIMO

WI/SI:	NR_MIMO_Ph5-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The RAN1-led Rel-19 NR MIMO Phase 5 work item was approved in RAN #102 meeting [1]. One objective with potential RAN4 impact is to specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource where UL full power transmission mode 1 and 2 are not supported. The discussion of this objective in RAN4 was not started until last RAN4 meeting where the following aspects were discussed and captured in the approved “WF on UE RF requirements for Rel-19 MIMO enhancement” [2]

    - UE RF impact of 3Tx

    - Applicable UE types for 3Tx

    - Applicable Frequency Ranges

    - Power classes and PA configuration for 3Tx UL MIMO

    - Non ULFPTx mode

    - ULFPTx mode 0

In this contribution, we share our preliminary views on the UE RF impact of 3Tx UL MIMO which includes power classes and PA configurations and their implication on the MPR requirements. We also share our view on the ULFPTx mode with 2-layer transmission.

# Discussion

The RAN4 aspect on the 3Tx UL MIMO objective in Rel-19 NR MIMO Phase 5 work item [1] was first discussed in last RAN4 meeting where it was agreed that the applicable frequency range is for FR1 only and both non-handheld and handheld UEs will be considered for UE RF requirements, as captured in the approved “WF on UE RF requirements for Rel-19 MIMO enhancement” [2]. Following the WF on the remaining open items, in this contribution, we provide our initial views on the UE RF impact of 3Tx UL MIMO which includes power classes and PA configuration and their implication on MPR requirements.

## 2.1	UE RF impact of 3Tx UL MIMO

From UE RF specifications point of view, the introduction of 3Tx UL MIMO will impact TS 38.101-1 clause 6.2D. In addition, the recent WID revision [3] on the enhancements to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching will also have impact on TS 38.101-1 clause 6.2.4 to introduce the corresponding ∆TRxSRS requirements.

Observation 1: From UE RF specifications point of view, the introduction of 3Tx UL MIMO will impact TS 38.101-1 clause 6.2D, and enabling 3T6R/3T3R SRS antenna switching will have impact on TS 38.101-1 clause 6.2.4.

Table 2.1-1 summarizes the expected specifications changes for the impacted clauses in TS 38.101-1 to support 3Tx UL MIMO and 3T6R/3T3R antenna switching.

| Clause | Title | Expected changes |
| --- | --- | --- |
| 6.2.4 | Configured transmitted power | Add ∆TRxSRS requirements to support 3T6R and 3T3R. |
| 6.2D.1 | UE maximum output power for UL MIMO | 1. Add 3-layer UL MIMO transmission codebook matrix. 2. Add 3-layer UL MIMO configuration in Table 6.2D.1-2. 3. Add ULFPTx mode 0 for 3Tx port in Table 6.2D.1-3. |
| 6.2D.2 | UE maximum output power reduction for UL MIMO | Specify MPR requirements for PC3, PC2, and PC1.5 with 3Tx. |
| 6.2D.3 | UE additional maximum output power reduction for UL MIMO | Add “3 transmit antenna” to the text description. |
| 6.2D.4 | Configured transmitted power for UL MIMO | Add PCMAX,c tolerance in closed-loop spatial multiplexing scheme for 3Tx. |

Table 2.1-1 Expected UE RF specifications changes to support 3Tx UL MIMO

Proposal 1: RAN4 to take Table 2.1-1 summary into consideration for the specifications changes to support 3Tx UL MIMO.

## 2.2	Power classes and PA configurations for 3Tx UL MIMO

Table 2.2-1 and 2.2-2 summarize our views on the supported power classes and possible PA configurations and their implication on the MPR requirements for ULFPTx and non-ULFPTx modes respectively for 3Tx UL MIMO. Our analysis was based on the common assumption that only PC3 (23dBm) and PC2 (26dBm) PAs are available for typical UE implementation in FR1 licensed bands. On the other hand, for ULFPTx mode, the WID has set the scope restriction that only mode 0 is supported. As a result, PC1.5 cannot be supported for ULFPTx mode. For MPR requirement in the tables below, if listed as a table number, it means the existing requirement specified in the corresponding table in TS 38.101-1 can be reused, and if listed as TBD, it means the requirement needs to be evaluated.

Observation 2: PC1.5 cannot be supported for 3Tx UL MIMO with ULFPTx mode.

| Power Class | PA Configuration | MPR Requirement |
| --- | --- | --- |
| PC3 | PC3+PC3+PC3 | Table 6.2.2-1 [4] |
| PC2 | PC2+PC2+PC2 | Table 6.2.2-2 [4] |
| PC1.5 | N/A |  |

Table 2.2-1 3Tx UL MIMO supported power classes and PA configurations for ULFPTx mode

| Power Class | PA Configuration | MPR Requirement |
| --- | --- | --- |
| PC3 | PC3+PC3+PC3 | Table 6.2.2-1 [4] |
| PC2 | PC3+PC3+PC3 | Table 6.2.2-2 [4] |
|  | PC2+PC3+PC3 | Table 6.2.2-2 [4] |
|  | PC2+PC2+PC3 | Table 6.2.2-2 [4] |
|  | PC2+PC2+PC2 | Table 6.2.2-2 [4] |
| PC1.5 | PC2+PC3+PC3 | TBD |
|  | PC2+PC2+PC3 | TBD |
|  | PC2+PC2+PC2 | TBD |

Table 2.2-2 3Tx UL MIMO supported power classes and PA configurations for non-ULFPTx mode

It can be seen that for both PC3 and PC2, regardless of whether ULFPTx mode is supported or not, the existing MPR requirements specified in clause 6.2.2 [4] can be reused. As a result, no additional MPR evaluation effort is needed for PC3 and PC2 3Tx UL MIMO. On the other hand, for PC1.5, there are 3 possible PA configurations to achieve 29dBm maximum output power and all of them in our view would require new MPR evaluations.  However, among them only PC2+PC2+PC2 configuration can support balanced output power under PCMAX at each antenna port. As it is unclear whether output power at each antenna port should maintain balanced or not, clarification from RAN1 on the requirement of antenna port power balance may be needed. If all 3 PA configurations are feasible to support PC1.5 3Tx UL MIMO, in principle the MPR requirements shall be based on the least linear PA configuration of PC2+PC3+PC3. Furthermore, since 3Tx UL MIMO will be supported by both FWA and handheld UE where different antenna isolation would have perceptible impact to MPR requirements, the requirements shall be specified for FWA and handheld UE separately.

Observation 3: For PC3 and PC2 3Tx UL MIMO, the existing MPR requirements specified in TS 38.101-1 clause 6.2.2 can be reused.

Observation 4: For PC1.5 3Tx UL MIMO, MPR requirements need to be evaluated and separately specified.

Observation 5: For PC1.5 3Tx UL MIMO, only PC2+PC2+PC2 configuration can support balanced output power under PCMAX at the antenna ports.

Proposal 2: Send an LS to RAN1 to clarify whether output power at each antenna port should maintain balanced or not to support UL MIMO.

Proposal 3: For PC1.5 3Tx UL MIMO, MPR requirements shall be separately specified for FWA and handheld UE.

## 2.3	ULFPTx mode with 2-layer transmission

TS 38.101-1 defines three different modes for ULFPTx. Each mode has a dedicated purpose which represents capabilities based on architecture assumptions. Mode-0 or Mode-full power is specified for a UE which features full power amplifiers on all Tx chains. It has the capability to achieve full power with respect to the advertised power class on any chain. Therefore, it can use TPMI 0 and TPMI 1. The network can configure any TPMI allowing to optimize performance by selecting the best Tx chain. There is a mixed mode, Mode-2, which is dedicated to devices which support full power on only one of the Tx chains. The assumption is that one Tx chain features a full power amplifier while the other Tx chain deploys a half-power amplifier. This architecture is useful to provide a trade-off between taking advantage of HPUE feature and balancing implementation complexity and cost efficiency. The last mode, Mode-1, is for devices which only support the half-rated power class on all Tx chains. Simultaneous transmit on two Tx chains is utilized to achieve full power over the air. The chains are non-coherent which means that the phase relation at a base-station antenna cannot be controlled which could lead to total signal cancelation at one of the base-station receive antennas. To battle the cancellation effect, diversity techniques might be used at the UE (e.g. cyclic diversity) or at the gNB receiver (e.g. spatially distributed antennas).

During the last RAN4 meeting it was tentatively agreed to use 1-layer as baseline assumption for ULFPTx Mode-0. Another proposal was made to include 2-layer transmission which was captured for further discussion. RAN1 had an agreement on non-coherent 2-layer TPMIs and defines them as follows:

The TPMIs feature simultaneous Tx on two chains while the third chain is muted. A total of three TPMIs are defined to cover all possible uplink configurations. The new matrixes resemble the 2-layer TPMIs used for mode-2 when four Tx port operation is used together with 2-layers in the uplink. As the new 3Tx TPMIs are a better match for Mode-2 and do not fit to the fundamental concept of Mode-0, it is proposed to not define those for Mode-0. Since Mode-1 and Mode-2 are specifically excluded by the WI description, the 2-layer ULFPTx operation should not be defined with the current work item.

Observation 6: The non-coherent 2-layer TPMIs feature simultaneous Tx on two chains while the third chain is muted. A total of three TPMIs are defined to cover all possible uplink configurations. The new matrixes resemble the 2-layer TPMIs used for Mode-2 when four Tx port operation is used together with 2-layers in the uplink.

Proposal 4: As the new 3Tx 2-layer TPMIs are a better match for Mode-2 and do not fit to the fundamental concept of Mode-0, it is proposed to not define 2-layer operation for Mode-0. Since Mode-1 and Mode-2 are specifically excluded by the WI description, the 2-layer ULFPTx operation should not be defined with the current work item.

# 3	Conclusion

In this contribution, we share our preliminary views on the UE RF impact of 3Tx UL MIMO which includes power classes and PA configurations and their implication on the MPR requirements. We also share our view on the ULFPTx mode with 2-layer transmission.

Observation 1: From UE RF specifications point of view, the introduction of 3Tx UL MIMO will impact TS 38.101-1 clause 6.2D, and enabling 3T6R/3T3R SRS antenna switching will have impact on TS 38.101-1 clause 6.2.4.

Proposal 1: RAN4 to take Table 2.1-1 summary into consideration for the specifications changes to support 3Tx UL MIMO.

Observation 3: For PC3 and PC2 3Tx UL MIMO, the existing MPR requirements specified in TS 38.101-1 clause 6.2.2 can be reused.

Observation 4: For PC1.5 3Tx UL MIMO, MPR requirements need to be evaluated and separately specified.

Observation 5: For PC1.5 3Tx UL MIMO, only PC2+PC2+PC2 configuration can support balanced output power under PCMAX at the antenna ports.

Proposal 2: Send an LS to RAN1 to clarify whether output power at each antenna port should maintain balanced or not to support UL MIMO.

Proposal 3: For PC1.5 3Tx UL MIMO, MPR requirements shall be separately specified for FWA and handheld UE.

Observation 6: The non-coherent 2-layer TPMIs feature simultaneous Tx on two chains while the third chain is muted. A total of three TPMIs are defined to cover all possible uplink configurations. The new matrixes resemble the 2-layer TPMIs used for Mode-2 when four Tx port operation is used together with 2-layers in the uplink.

Proposal 4: As the new 3Tx 2-layer TPMIs are a better match for Mode-2 and do not fit to the fundamental concept of Mode-0, it is proposed to not define 2-layer operation for Mode-0. Since Mode-1 and Mode-2 are specifically excluded by the WI description, the 2-layer ULFPTx operation should not be defined with the current work item.

# 4	References

RP-234007 “New WID: NR MIMO Phase 5”, Samsung (Moderator), 3GPP TSG-RAN Meeting #102, Edinburgh, Scotland, December 11th – 15th, 2023

R4-2414310 “WF on UE RF requirements for Rel-19 MIMO enhancement”, Samsung, 3GPP TSG RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024

RP-242394 “WID revision: NR MIMO Phase 5”, Samsung (Moderator), 3GPP TSG-RAN Meeting #105, Melbourne, Australia, September 9th – 12th, 2024