3GPP RAN WG4 Meeting #112bis		R4-2415334

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.26.2.2

Source:	Apple

Title:	On NTN simultaneous operation with GNSS

WI/SI:	NR_NTN_Ph3-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

Considering that the support of RedCap devices operating in FR1 band NR-NTN networks can offer enhanced service capabilities (wideband/broadband) compared to IoT-NTN while ensuring low-complexity devices and the global coverage would clearly benefit RedCap devices, an objective on the “support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands” is included in the Rel-19 new WID on “Non-Terrestrial Networks (NTN) for NR Phase 3” led by RAN2 [1]. Under this objective, there is a precondition that GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE. However, the simultaneous GNSS and NR-NTN operation aspect has not been formally discussed in the past few RAN4 meetings. In last RAN4 meeting, the following considerations were captured in the approved “Way Forward for NR_NTN_Ph3_UE_RF” [2] to facilitate the study of NR NTN UE simultaneous operation with GNSS.

- Operating bands with UL close to GNSS frequencies (n254, n255)

- Emissions on GNSS frequencies

- Whether transmission gaps enabling successful GNSS reception are present.

- Potential impact on field performance

- Potential need for specification updates, e.g. frequency error

In this contribution, we provide our preliminary RF analysis on the worst-case in-device coexistence scenario between NR NTN L-band UL and GPS band and conclude that it would not be feasible to support simultaneous Rx/Tx between GPS receiver and L-band UL transmission at maximum output power based on the typical RF front-end HW implementation for handheld or RedCap UE.

# Discussion

Our preliminary RF analysis was based on the worst-case in-device coexistence scenario between NR NTN L-band UL, or specifically n254 UL, and GPS band centered at 1575.42 MHz where the spectrum allocations are shown in Figure 2-1.

Figure 2-1 Spectrum allocations for GPS L1 band and NR NTN band n254 UL range

Table 2-1 summarizes the related RF parameters for the analysis. From NR NTN link budget perspective, band n254 UL is assumed to always transmit at maximum output power at 23dBm (PC3) during the operation. The GPS filter to band n254 UL rejection is extracted from the filter profile as shown in Figure 2-2.

| Parameter | Value | Unit |
| --- | --- | --- |
| n254 UL power | 23 | dBm |
| GPS signal level | -135 | dBm |
| GPS signal bandwidth | 10 | MHz |
| GPS C/N0 requirement | 30 | dB-Hz |
| n254 UL filter rejection in GPS band | 40 | dB |
| GPS filter to n254 UL rejection | 8 | dB |
| NTN to GPS antenna isolation | 10 | dB |

Table 2-1 RF parameters for NTN L band and GPS in-device coexistence analysis

Table 2-2 GPS filter frequency response

There are two major interference mechanisms from n254 UL which may impact GPS reception, one is the UL side-band noise falling into GPS band, the other is the UL blocking signal leaking into GPS receiver path. Our analysis shows that the impact from side-band noise falling into GPS band would not be an issue when UL transmission bandwidth is less than 4 MHz. However, due to insufficient GPS filter rejection and finite antenna isolation to n254 UL, the blocking signal seen at GPS LNA input could be as high as 5 dBm which would fail the GPS reception, irrespective of the n254 UL transmission bandwidth.

Observation 1: There are two major interference mechanisms from n254 UL which may impact GPS reception, one is the UL side-band noise falling into GPS band, the other is the UL blocking signal leaking into GPS receiver path.

Observation 2: The impact from n254 UL side-band noise falling into GPS band would not be an issue when UL transmission bandwidth is less than 4 MHz.

Observation 3: Due to insufficient GPS filter rejection and finite antenna isolation to n254 UL, the blocking signal seen at GPS LNA input could be as high as 5 dBm which would fail the GPS reception, irrespective of the n254 UL transmission bandwidth.

Based on the above observations, we can conclude that it would not be feasible to support simultaneous Rx/Tx between GPS receiver and L-band UL transmission at maximum output power.

Observation 4: It would not be feasible to support simultaneous Rx/Tx between GPS receiver and L-band UL transmission at maximum output power.

As to our understanding the GNSS capabilities is mainly for UE to identify its own position such that it can pre-compensate for the delay of the transmission to the satellite to achieve synchronization before and during connection to an NTN cell, it is unclear whether the GNSS position fix is still required when the connection to satellite is already established. If GNSS position fix is not required during the NTN connected mode, there does not seem to have a need to find a solution for mitigating the in-device coexistence issue between NTN operation and GNSS.

Observation 5: It is unclear whether the GNSS position fix is still required when the connection to satellite is already established.

Observation 6: If GNSS position fix is not required during the NTN connected mode, there does not seem to have a need to find a solution for mitigating the in-device coexistence issue between NTN operation and GNSS.

Proposal: 3GPP to clarify whether GNSS position fix is only needed during the NTN initial access or it is also required during the NTN connected mode.

# 3	Conclusion

In this contribution, we provide our preliminary RF analysis on the worst-case in-device coexistence scenario between NR NTN L-band UL and GPS band and conclude that it would not be feasible to support simultaneous Rx/Tx between GPS receiver and L-band UL transmission at maximum output power based on the typical RF front-end HW implementation for handheld or RedCap UE.

Observation 1: There are two major interference mechanisms from n254 UL which may impact GPS reception, one is the UL side-band noise falling into GPS band, the other is the UL blocking signal leaking into GPS receiver path.

Observation 2: The impact from n254 UL side-band noise falling into GPS band would not be an issue when UL transmission bandwidth is less than 4 MHz.

Observation 3: Due to insufficient GPS filter rejection and finite antenna isolation to n254 UL, the blocking signal seen at GPS LNA input could be as high as 5 dBm which would fail the GPS reception, irrespective of the n254 UL transmission bandwidth.

Observation 4: It would not be feasible to support simultaneous Rx/Tx between GPS receiver and L-band UL transmission at maximum output power.

Observation 5: It is unclear whether the GNSS position fix is still required when the connection to satellite is already established.

Observation 6: If GNSS position fix is not required during the NTN connected mode, there does not seem to have a need to find a solution for mitigating the in-device coexistence issue between NTN operation and GNSS.

Proposal: 3GPP to clarify whether GNSS position fix is only needed during the NTN initial access or it is also required during the NTN connected mode.

# 4	References

RP-234078 “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Huawei, 3GPP TSG-RAN Meeting #102, Edinburgh, Scotland, December 11th – 15th, 2023

R4-2413527 “Way Forward for [112][311] NR_NTN_Ph3_UE_RF”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024