3GPP RAN WG4 Meeting #112bis		R4-2415337

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.2

Source:	Apple

Title:	On RAN task for CA power class support

WI/SI:	TEI

Release:	Rel-16

Document for:	Approval

# 1	Introduction

In RAN #104 meeting, RAN tasked RAN4 to investigate a solution to enable HPUE maximum transmit power in downlink CA with single configured UL [1]. The main concern is that a DL CA configured with single UL transmission may not transmit the same HPUE maximum output power as supported by the same band in single carrier operation due to that the power class for DL only CA has not been specified to support the same HPUE power class as with its single carrier counterpart, likely being delayed by the slow MSD analysis process. In last RAN4 meeting (#112), companies had shared their views on the possible solutions and the outcome of the discussions with a tentative solution was captured in the “Way forwards for RAN task on HPUE with 1UL and DL CA” [2] which however was not approved. In this contribution, we share our view that the concerned issue of the RAN task is not specific to DL CA with single UL transmission only, it also occurs to 2UL CA and EN-DC with 2-band or 3-band DL combinations where the HPUE MSD for the self DL band(s) or the 3rd DL band has not been specified. And for that reason, we propose to merge the RAN task discussions with the ongoing Rel-19 HPUE MSD rules discussions and seek for a common solution.

# Discussion

Though the issue which triggered RAN to task RAN4 to investigate a solution to enable HPUE maximum transmit power in downlink CA is for configuration with single UL transmission [1], in our view the issue is not specific to DL CA with single UL transmission only, it also occurs to 2UL CA and EN-DC with 2-band or 3-band DL combinations where the HPUE MSD for the self DL band(s) or the 3rd DL band has not been specified. For any 2-band UL combination, technically PC2 should already be supported as each of the UL bands should at least support PC3. The reason why PC2 is not introduced for those 2-band UL combinations is mainly due to that the PC2 MSD has not been specified which is mandated in the current HPUE CA/DC framework. Similarly, for any 3-band DL combinations, if the PC2 MSD for the 3rd DL band is not specified, the corresponding PC2 2-band UL configurations also cannot be supported, despite PC2 has already been specified for its fallback combinations. The same issue also applies for band combinations intended to support PC1.5.

Observation 1: The issue which triggered RAN to task RAN4 to investigate a solution to enable HPUE maximum transmit power in downlink CA is not specific to DL CA with single UL transmission only, it also occurs to 2UL CA and EN-DC with 2-band or 3-band DL combinations where the HPUE MSD for the self DL band(s) or the 3rd DL band has not been specified.

Observation 2: For any 2-band UL combination, technically PC2 should already be supported as each of the UL bands should at least support PC3.

Observation 3: The reason why PC2 is not introduced for those 2-band UL combinations is mainly due to that the PC2 MSD has not been specified which is mandated in the current HPUE CA/DC framework.

Since in Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [3], there is an on-going discussion on the MSD rules for HPUE under the objective of “HPUE for NR uplink (UL) CA in FR1 and EN-DC with NR FR1 bands” which may also cover the RAN task solution, considering that there is not much TU allocated for this RAN task, it might be better to merge this RAN task discussions with Rel-19 HPUE MSD rules discussions.

Observation 4: The on-going Rel-19 HPUE MSD rules discussions may also cover RAN task solution for DL CA with single UL transmission.

Observation 5: There is not much TU allocated for RAN task solution discussions.

Proposal: Merge the RAN task discussions with the ongoing Rel-19 HPUE MSD rules discussions and seek for a common solution.

# 3	Conclusion

In this contribution, we share our view that the concerned issue of the RAN task is not specific to DL CA with single UL transmission only, it also occurs to 2UL CA and EN-DC with 2-band or 3-band DL combinations where the HPUE MSD for the self DL band(s) or the 3rd DL band has not been specified. And for that reason, we propose to merge the RAN task discussions with the ongoing Rel-19 HPUE MSD rules discussions and seek for a common solution.

Observation 1: The issue which triggered RAN to task RAN4 to investigate a solution to enable HPUE maximum transmit power in downlink CA is not specific to DL CA with single UL transmission only, it also occurs to 2UL CA and EN-DC with 2-band or 3-band DL combinations where the HPUE MSD for the self DL band(s) or the 3rd DL band has not been specified.

Observation 2: For any 2-band UL combination, technically PC2 should already be supported as each of the UL bands should at least support PC3.

Observation 3: The reason why PC2 is not introduced for those 2-band UL combinations is mainly due to that the PC2 MSD has not been specified which is mandated in the current HPUE CA/DC framework.

Observation 4: The on-going Rel-19 HPUE MSD rules discussions may also cover RAN task solution for DL CA with single UL transmission.

Observation 5: There is not much TU allocated for RAN task solution discussions.

Proposal: Merge the RAN task discussions with the ongoing Rel-19 HPUE MSD rules discussions and seek for a common solution.

# 4	References

RP-241687 “Moderator's summary for offline discussion on CA Power Class Support”, Moderator (AT&T), 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17th – 20th, 2024

R4-2414436 “Way forwards for RAN task on HPUE with 1UL and DL CA”, Qualcomm et. al., 3GPP TSG RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024