3GPP TSG-RAN WG4 Meeting #112-bis                                                      R4-2415344

Hefei, China, Oct 14 – 18, 2024

Agenda item:          6.2.1

Title:	TP for TR 38.922 on frequency allocation in USA for 4400 – 4800 MHz, 7125 – 8400 MHz, and 14800 – 15350 MHz

WI/SI:	FS_NR_IMT_4400_7125_14800MHz

Release:	Rel-19

Source:	CableLabs

Document for:	Approval

# 1. Introduction

It was discussed in the RNA4 #112 meeting to add the frequency use cases in different regions in TR 38.922 [1]. This document presents a text proposal to capture the frequency use cases for 4400 – 4800 MHz, 7125 – 8400 MHz, and 14800 – 15 350 MHz in the USA [2].

# 2. 	Text proposal

-------------------------------------------- Beginning of TP --------------------------------------------

# 8	Frequency usage

## 8.1	ITU Region 1

### 8.1.1	OOOO

### 8.1.2	OOOO

## 8.2	ITU Region 2

### 8.2.1	United States of America

The 4400 – 4800 MHz band is currently allocated in the USA to fixed and mobile from 4400 – 4900 MHz, and fixed-satellite (space-to-earth) from 4500 – 4800 MHz. The 7125 – 8400 MHz band allocation in the USA and internationally is summarized in Figure 8.2.1-1. This band is mainly occupied by military radar satellites including both geostationary orbit (GSO) and non-GSO (NGSO), fixed service (FS), earth exploration service (EES), etc. The 14800 – 15350 MHz band is allocated for fixed, mobile, and space research. The frequency allocation for all these three bands in the USA is listed in Table 8.2.1-1 [8.2-1].

On Nov. 13, 2023, the U.S. White House and National Telecommunications and Information Administration (NTIA) published the National Spectrum Strategy [8.2-2]: “approximately 2,790 megahertz of spectrum represents a mix of bands for potential expanded governmental and non-governmental use for an array of advanced, next-generation applications and services” including 3.1-3.45 GHz, 5.03-5.091 GHz, 7.125-8.4 GHz, 18.1-18.6 GHz, and 37-37.6 GHz. Specifically, the 7.125-8.4 GHz band identified by ITU for IMT-2030 is also being considered by NTIA: “This 1,275 megahertz of spectrum will be studied for wireless broadband use (on a licensed and/or unlicensed basis), though some sub-bands eventually may be studied for other uses.”

Figure 8.2.1-1. Frequency use chart for 4400–4800 MHz, 7125–8400 MHz, and 14800 MHz–15 350 MHz in USA.

Table 8.2.1-2: Frequency use case for 4400–4800 MHz, 7125–8400 MHz, and 14800 MHz–15 350 MHz Internationally

| Frequency range (MHz) | Purpose |
| --- | --- |
| 4400-4940 | Fixed |
|  | Mobile |
| 4500-4800 | FIXED-SATELLITE (space-to-Earth) |
| 7125-7145 | Fixed |
| 7145-7190 | Fixed |
|  | SPACE RESEARCH (deep space) (Earth-to-space) |
| 7190-7235 | EARTH EXPLORATION-SATELLITE (Earth-to-space) |
|  | Fixed |
|  | SPACE RESEARCH (Earth-to-space) |
| 7235-7250 | EARTH EXPLORATION-SATELLITE (Earth-to-space) |
|  | Fixed |
| 7250-7300 | FIXED-SATELLITE (space-to-Earth) |
|  | MOBILE-SATELLITE (space-to-Earth) |
|  | Fixed |
| 7300-7375 | Fixed |
|  | FIXED-SATELLITE (space-to-Earth) |
|  | Mobile-satellite (space-to-Earth) |
| 7375-7450 | Fixed |
|  | FIXED-SATELLITE (space-to-Earth) |
|  | MOBILE except aeronautical mobile |
|  | MARITIME MOBILE-SATELLITE (space-to-Earth) |
|  | Mobile-satellite except maritime mobile-satellite (space-to-Earth) |
| 7450-7550 | Fixed |
|  | FIXED-SATELLITE (space-to-Earth) |
|  | METEOROLOGICAL-SATELLITE (space-to-Earth) |
|  | MARITIME MOBILE-SATELLITE (space-to-Earth) |
|  | Mobile-satellite except maritime mobile-satellite (space-to-Earth) |
| 7550-7750 | Fixed |
|  | FIXED-SATELLITE (space-to-Earth) |
|  | MARITIME MOBILE-SATELLITE (space-to-Earth) |
|  | Mobile-satellite except maritime mobile-satellite (space-to-Earth) |
| 7750-7900 | Fixed |
|  | METEOROLOGICAL-SATELLITE (space-to-Earth) |
| 7900-8025 | FIXED-SATELLITE (Earth-to-space) |
|  | MOBILE-SATELLITE (Earth-to-space) |
|  | Fixed |
| 8025-8175 | EARTH EXPLORATION-SATELLITE (space-to-Earth) |
|  | Fixed |
|  | FIXED-SATELLITE (Earth-to-space) |
|  | Mobile-satellite (Earth-to-space)(no airborne transmissions) |
| 8175-8215 | EARTH EXPLORATION-SATELLITE (space-to-Earth) |
|  | Fixed |
|  | FIXED-SATELLITE (Earth-to-space) |
|  | METEOROLOGICAL-SATELLITE (Earth-to-space) |
|  | Mobile-satellite (Earth-to-space)(no airborne transmissions) |
| 8215-8400 | EARTH EXPLORATION-SATELLITE (space-to-Earth) |
|  | Fixed |
|  | FIXED-SATELLITE (Earth-to-space) |
|  | Mobile-satellite (Earth-to-space)(no airborne transmissions) |
| 14800-15350 | Mobile |
|  | SPACE RESEARCH |
|  | Fixed |

## 8.3	ITU Region 3

References:

[8.2-1] FCC, “Radio Spectrum Allocation,” Online Available: [https://www.fcc.gov/engineering-technology/policy-and-rules-division/radio-spectrum-allocation/general/table-frequency], Sept. 25, 2024.

[8.2-2] The White House, “National Spectrum Strategy,” Online Available: [https://www.ntia.gov/sites/default/files/publications/national_spectrum_strategy_final.pdf], Nov. 13, 2023.

-------------------------------------------- End of the TP --------------------------------------------

# 4. Reference

[1] R4-24xxxxx, “RAN4 12 Main Session report v09 after post meeting.”

[2] R4-2415343, “Add a section in TR 38.922 on frequency allocation in the IMT-2030 bands,” CableLabs, Oct. 2024.