3GPP TSG-RAN WG4 Meeting #112-bis                     R4-2415358

Hefei, CN, 14th – 18th October, 2024

Agenda Item:

Source:	Southeast University (SEU)

Title:	Discussion on testability and interoperability of AI for CSI feedback enhancement

Document for:	Discussion

# 1. Introduction

In 3GPP RAN #102, for CSI enhancement, it is agreed to further study the testability and interoperability issues in R19. In this contribution, we will focus on the testing options for CSI prediction case.

2. Discussion on CSI prediction

In RAN4#110, the following agreement was achieved on CSI prediction accuracy metrics [1]:

Issue 4-1: CSI Prediction Accuracy metrics - Proposals - Option 1: Prediction accuracy can be used as KPI/metric - Option 2: Prediction accuracy cannot be used because the “correct” value is not available - Option 3: Throughput should be the default metric, others should be discussed only if throughput is not feasible - Option 4: Others Agreement: - Agree option 3 for inference only. TBD whether we use relative or absolute throughput. - Monitoring will be discussed separately.

The following agreements for performance monitoring for CSI prediction were achieved in the previous meetings:

Agreement For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured. Observation For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

In current studies, the prediction model is primarily trained under the i.i.d. assumption, yet the performance of AI models is intimately tied to the degree of similarity between the training and testing datasets, leading to the out-of-distribution problem. The question of how to accurately evaluate this similarity degree and its subsequent impact on model performance remains an open challenge. Moreover, current studies on performance monitoring mainly focus on evaluating the prediction model based on throughput model output, with inadequate attention paid to the metrics for monitoring the model's input data.

Observation 1: Problems regarding the similarity between the training and testing datasets have not been discussed.

Observation 2: The metric for monitoring the model input data has not been fully discussed.

In the context of model-output-based performance monitoring, current studies concentrate on performance metrics and report mechanisms, while lack of comprehensive discussion surrounding the evaluation of the validity of these performance monitoring approaches. True positive rate (TPR) and true negative rate (TNR) can be used as the metrics to evaluate the validity of performance monitoring methods, which are defined as:

TPR=TP/ (TP+FN)

TNR=TN/ (TN+FP),

where ‌TP, ‌FP, ‌FN, ‌TN denotes the number of true positive, false positive, false negative, and true negative prediction outcomes, respectively.

When considering the potential impacts on downlink transmission performance, false positive outcomes often pose a more significant threat than false negative outcomes, because false positives can significantly degrade the downlink transmission performance. Conversely, while false negatives are certainly undesirable, they tend to have a less pronounced impact on downlink performance. Given this, the TNR, which measures the ability of identifying unqualified prediction outcomes, should be given greater emphasis in performance monitoring.

Proposal 1: The evaluation metrics regarding the performance of performance monitoring should also be considered.

Proposal 2: True positive rate and true negative rate can be used as metrics to evaluate the validity of performance monitoring methods.

Proposal 3: The ability of identifying unqualified prediction outcomes should be given greater emphasis in performance monitoring.

3. Discussion on Generalization

In the RAN4#110 meeting, the agreements for generalization are achieved:

Issue 1-1: Generalization update Agreement: - For AI/ML generalization [tests/requirements] - RAN4 should discuss it and decide the requirements/tests for each AI feature in the case-by-case manner

In our view, the high-performance achieved by modern AI models, particularly those utilizing deep learning techniques, is often attributed to their remarkable capacity to capture intricate patterns within a specific dataset through overfitting to key parameters. While this approach yields impressive results within the training domain, it can also lead to a narrow focus, limiting the model's ability to generalize to new or unseen situations. Achieving a delicate balance between maximizing performance within the known domain and ensuring sufficient generalization for future scenarios is thus a fundamental challenge in AI research and development. Cell-specific model can mitigate the tension between performance and generalization.

Observation 3: The trade-off between the accuracy performance and generalization has not been fully considered.

Observation 4: The generalization ability of cell-specific framework should be evaluated.

Moreover, the concept of generalization must be expanded to encompass not only spatial variations across different scenarios but also temporal dynamics within those scenarios. Real-world applications are inherently dynamic, with data distributions shifting over time due to various factors such as seasonal changes, technological advancements, or societal shifts. Therefore, developing AI systems that can adapt to such temporal variations is crucial for ensuring long-term performance and relevance.

Proposal 4: The concept of generalization must be expanded to encompass not only spatial variations across different scenarios but also temporal dynamics within those scenarios.

When AI models encounter out-of-distribution (OOD) samples, their performance can deteriorate significantly, posing a significant risk to the reliability and safety of the systems they power. This underscores the importance of implementing robust performance monitoring mechanisms that can detect and respond to such degradations in a timely manner. To ensure comprehensive coverage, evaluations of these monitoring systems must consider both in-distribution and out-of-distribution cases, allowing for a more accurate assessment of the model's overall robustness and resilience.

Proposal 5: Both in-distribution and out-of-distribution cases should be evaluated for performance monitoring.

Current studies usually use vanilla training methods when evaluating the generalization performance of AI. To further enhance the generalization capabilities of AI, domain generalization methods have emerged as a powerful tool. Domain generalization refers to the ability of an AI model to perform well on data from unseen domains, even when trained on only a limited set of known domains. It addresses the issue of distribution shift. The key objective is to learn representations that capture the essential information common to all domains, while ignoring domain-specific nuances that could hinder generalization. By exposing the model to a diverse range of data distributions during training, domain generalization aims to improve the model's ability to generalize to new and unseen domains. Therefore, domain generalization methods could be considered when evaluating the generalization of CSI feedback enhancement frameworks.

4. Evaluation results of performance monitoring methods for gNB-side CSI prediction

Current research primarily concentrates on UE-side prediction, with limited exploration and evaluation of performance monitoring methods for gNB-side prediction. This section assesses two strategies for monitoring gNB-side prediction performance.

It should be noted that due to the mechanism of SVD-based precoding, the UE needs to process the received signal using the left singular matrix. Therefore, even when employing gNB-side prediction, estimation at the UE may still be necessary during the prediction intervals.

The first baseline strategy involves intermediate-KPI-based monitoring, where the UE periodically performs CSI estimation and transmits non-quantized estimated CSI to the gNB at the prediction interval. Drawing inspiration from proxy-based monitoring methods used in two-side CSI compression, we propose the second strategy. This strategy employs a proxy located at the UE, which is trained using the gNB-side predictor's output to mimic its behavior. During the prediction phase, the UE inputs stored historical CSI into the proxy to obtain an imitated predicted CSI. Subsequently, the UE estimates the CSI at the prediction interval and calculates the intermediate KPI based on the difference between the imitated and estimated CSI. This KPI value is then used for decision-making.

Fig. 1 Illustration of the proxy-based performance monitoring method for gNB-side prediction

Evaluations of performance monitoring methods for gNB-side CSI prediction is conducted as following. The data is generated by QuaDRiGa.

Table 1. Parameters of test of performance monitoring methods.

| Scenario | UMi_NLOS |
| --- | --- |
| Carrier frequency | 2.1GHz |
| Subcarrier spacing | 15KHz |
| Frequency resources | 48 RBs, 12 subbands for 4RBs per subband |
| Rank | 1 |
| gNB antenna | 32 antenna ports |
| UE antenna | 4 antenna ports |
| UE speed | 36km/h |

The predictor and proxy’s backbone are both transformer block. We assume that the predictor utilizes 3 historical CSI to predict 1 future CSI, and the UE transmits non-quantized estimated CSI to the gNB every 5 prediction intervals. The intermediate KPI used for monitoring is SGCS. We set different monitoring thresholds, and predictions are only considered valid when the SGCS exceeds the set threshold. The accuracy of two methods are presented in Table 2.

Table 2. Evaluation results of performance monitoring methods with different monitoring thresholds.

| Methods | Monitoring threshold (SGCS) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0.5 | 0.55 | 0.6 | 0.65 | 0.7 | 0.75 |
| Periodical estimation (baseline) | 77.3% | 77.7% | 77.7% | 77.8% | 78.0% | 79.1% |
| Proxy | 92.7% | 92.8% | 92.9% | 93.6% | 94.7% | 95.9% |

Across all thresholds, the proposed proxy-based monitoring method consistently achieves over 90% accuracy and significantly outperforms the baseline method. This underscores the efficacy of the proposed proxy-based performance monitoring approach.

Conclusion

In this contribution, we provide the following proposals:

Observation 1: Problems regarding the similarity between the training and testing datasets have not been discussed.

Observation 2: The metric for monitoring the model input data has not been fully discussed.

Observation 3: The trade-off between the accuracy performance and generalization has not been fully considered.

Observation 4: The generalization ability of cell-specific framework should be evaluated.

Proposal 1: The evaluation metrics regarding the performance of performance monitoring should also be considered.

Proposal 2: True positive rate and true negative rate can be used as metrics to evaluate the validity of performance monitoring methods.

Proposal 3: The ability of identifying unqualified prediction outcomes should be given greater emphasis in performance monitoring.

Proposal 4: The concept of generalization must be expanded to encompass not only spatial variations across different scenarios but also temporal dynamics within those scenarios.

Proposal 5: Both in-distribution and out-of-distribution cases should be evaluated for performance monitoring.

Reference

[1] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.