3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415362

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for NR duplex operation

Agenda item:	6.20.3

Document for:	Discussion

Introduction

In the last RAN4#112 meeting, RAN4 started discussion about the NR duplex operation WI. During the first meeting, many non-controversial issues were agreed such like work plan, scope and co-existence with legacy UE related issues.

In this contribution, we provide our views on the remain open issues described in WF[1].

Discussion

### Sub-topic 2-1: RRM impacts of UE-to-UE CLI handling

Issue 2-1-2: Measurement quantities Recommended WF - RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. This can be revisited based on further RAN1 agreement. - FFS if other measurement quantities to be considered in the core requirements based on further RAN1 agreement.

In the last RAN1#118 meeting the two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} are agreed to add to the higher layer parameter reportQuantity. So, we think that RAN4 should define core requirements for L1-CLI-RSSI and L1-SRS-RSRP. And based on further RAN1 agreement, RAN4 can consider additional core requirements.

Proposal 1: RAN4 to define core requirements for L1-CLI-RSSI and L1-SRS-RSRP. And based on further RAN1 agreement, RAN4 can consider additional core requirements.

Issue 2-1-3: Baseline for defining requirements Recommended WF - Both R16 CLI measurement requirements and L1-RSRP measurement requirements are to be considered for defining requirements for L1 based UE-to-UE CLI measurement. - RAN4 can directly discuss the requirements for L1 based UE-to-UE CLI measurement in next meeting.

For L1 based UE-to-UE CLI measurement, both R16 CLI measurement requirements and L1-RSRP measurement requirements can be baseline. Because there is no big difference between them except the frequency domain range. The L1 based UE-to-UE CLI measurement for SBFD should consider only DL sub-band for L1-CLI-RSSI and UL sub-band for L1-SRS-RSRP respectively rather than DL active BWP which can depend on RAN1’s agreement.

Proposal 2: For L1 based UE-to-UE CLI measurement, both R16 CLI measurement requirements and L1-RSRP measurement requirements can be baseline.

Proposal 3: The additional applicability for L1 based UE-to-UE CLI measurement need to be considered as follow:

L1 based UE-to-UE CLI measurement for SBFD are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of UL sub-band - when CLI-RSSI measurement resource is configured within DL sub-band

Issue 2-1-4: Measurement methods Recommended WF - RAN4 to wait for RAN1 conclusion on the supported methods of L1 based UE-to-UE CLI measurement. - Meanwhile, RAN4 to discuss the impacts of different measurement methods on the requirements.

The UE to support L1 based UE-to-UE CLI measurement has to capable to measure the CLI measurement with sub-band unit rather than DL-active BWP unit.

Proposal 4: The UE to support L1 based UE-to-UE CLI measurement has to capable to measure the CLI measurement with sub-band unit rather than DL-active BWP unit.

Issue 2-1-5: Rx beam Recommended WF - RAN4 to wait for RAN1 conclusion on the Rx beam for L1 based UE-to-UE CLI measurement. - Meanwhile, RAN4 to discuss the impacts of Rx beam configuration/determination on the requirements.

Regarding the Rx beam configuration/determination for L1 based UE-to-UE CLI measurement, we think that the UE should not change the configuration of the Rx beam for L1 based UE-to-UE CLI measurement rather the UE should remain in the same configuration with DL data communication.

Proposal 5: The UE should not change the configuration of the Rx beam for L1 based UE-to-UE CLI measurement rather the UE should remain in the same configuration with DL data communication.

Issue 2-1-6: Measurement resources Recommended WF - RAN4 to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

Proposal 6: RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

Issue 2-1-7: Side condition Options for further discussion and down-selection (for information purpose): - Time offset between DL timing and SRS arrival timing - Option 1: Rel-16 CLI SRS RSRP assumption - Option 2: Remove cell phase error from option 1 - Other options are not precluded, pending on RAN1 progress. - SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP - Option 1: Rel-16 CLI SRS RSRP assumption - Other options are not precluded.

Different from Rel-16 CLI SRS RSRP measurement, the Rel-19 SBFD SRS RSRP measurement is the measurement CLI caused by the UL sub-band signal from same cell. So, for the time offset between DL timing and SRS arrival timing, Rel-16 CLI SRS RSRP assumption can be considered with removing cell phase error. And regarding SRS Es/Iot, Rel-16 CLI SRS RSRP assumption can be reused.

Proposal 7: For the time offset between DL timing and SRS arrival timing, Rel-16 CLI SRS RSRP assumption can be considered with removing cell phase error. And regarding SRS Es/Iot, Rel-16 CLI SRS RSRP assumption can be reused.

### Sub-topic 2-3: RRM impacts of SBFD operation

Issue 2-3-8: Requirements for UL resource muting Recommended WF - RAN4 further to discuss whether additional RRM requirements would be defined for UL resource muting for PUSCH feature.

According to the WID[1], some features are introduced to support CLI handling. Among them 2 features are considered to be related to RAN4 work. First feature is UL resource muting for PUSCH. UE can measure CLI at muted RE. Currently RAN1 assuming comb-2 muting and up to 2 OFDM symbols in time domain. For keeping the time domain power level, the transmit power of PUSCH REs in the same OFDM symbols are considered 3dB power boosting.

RF part need to check the impact on transmit signal quality/MPR requirement. But RRM perspective, there is no spec. impact.

Proposal 8: RAN4 RRM does not need to consider the impact by UL resource muting for PUSCH feature.

Summary

Proposal 1: RAN4 to define core requirements for L1-CLI-RSSI and L1-SRS-RSRP. And based on further RAN1 agreement, RAN4 can consider additional core requirements.

Proposal 2: For L1 based UE-to-UE CLI measurement, both R16 CLI measurement requirements and L1-RSRP measurement requirements can be baseline.

Proposal 3: The additional applicability for L1 based UE-to-UE CLI measurement need to be considered as follow:

L1 based UE-to-UE CLI measurement for SBFD are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of UL sub-band - when CLI-RSSI measurement resource is configured within DL sub-band

Proposal 4: The UE to support L1 based UE-to-UE CLI measurement has to capable to measure the CLI measurement with sub-band unit rather than DL-active BWP unit.

Proposal 5: The UE should not change the configuration of the Rx beam for L1 based UE-to-UE CLI measurement rather the UE should remain in the same configuration with DL data communication.

Proposal 6: RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

Proposal 7: For the time offset between DL timing and SRS arrival timing, Rel-16 CLI SRS RSRP assumption can be considered with removing cell phase error. And regarding SRS Es/Iot, Rel-16 CLI SRS RSRP assumption can be reused.

Proposal 8: RAN4 RRM does not need to consider the impact by UL resource muting for PUSCH feature.

Reference

[1] R4-2413900, “WF on SBFD RRM”, Huawei