3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415364

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Agenda item:	6.23.4.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues of RRM core requirements for NR LP-WUS/WUR based on listed issues in approved WF [1].

Discussion

- Topic #1: RRM core requirements for LP-WUS/WUR

General aspects

### Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

In the last meeting, RAN4 agreed as below:

Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

Regarding serving cell measurement, MR and WUR are different receiver architecture and they measure different transmitted signals which could have different transmission power. So, we think MR and WUR should work independently including the results.

Proposal 1: Support option 2. From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR.

For the case #2~#4, depending on the signal quality os serving cell based on MR or LR, RRM measurement relaxation could be considered. If the signal quality is high enough, then fully offloading case can be considered as #1. if LR measurement is performed in center or middle of cell coverage, MR measurement for serving and/or neighbour cell could be relaxed comparing legacy measurement requirements as #2 and #3. The UE is in cell edge area, then the UE should perform normal measurement for neigbhor cells as legacy case (#5) because the mobility is more important than power saving. However, for case#4, it is unclear under what conditions neighboring cells are measured without measuring the serving cell based on MR.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #5 Legacy case | On | On | OFF |

From the below figure, UE#1 is expected to be “Fully offloading case” and UE#2 can be “case#2 or #3” and UE#3 should be in “Legacy case”.

Proposal 2: According to the channel quality the three cases are can be necessary.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | On |
| #2 Relaxed case a | On with relaxation measurement | Off | On |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | On |
| #5 Legacy case | On | On | Off |

### Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

Based on companies’ view in the last meeting, MR RRM measurement relaxation can be considered for serving cell and/or neighbour cells. And the MR RRM measurement relaxation for serving cell and/or neigbhor cells should be considered when LP-WUR measurement is performed.

Proposal 3: Further MR based RRM measurement relaxation for serving cell and/or neighbour cells should be applied when LP-WUR measurement is performed.

Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

The entry/exit criteria for LP-WUS paging monitoring and for MR RRM relaxation or serving cell measurement offloading can be different. For example, from the figure below, if the UE is near the serving cell (orange color area), the UE can be considered that the UE is in RRM relaxation and serving cell measurement offloading case. Else if the UE is far from serving cell (light blue color area), the UE should not allowed RRM relaxation and serving cell measurement offloaging for UE’s power saving because the mobility can be more important at cell edge area. But LP-WUS paging monitoring perspective, the UE can still allowed if the LP-SS/WUS received quality is high enough. Because the purpose of the LP-WUS paging monitoring is not for mobility but for Paging monitoring. So, we need to disscuss the LP-WUS paging monitoring and MR RRM relaxation issues separately.

Proposal 4: RAN4 to discuss separately following two issues:

- On requirements for evaluating entry/exit criteria for LP-WUS paging monitoring

- On requirements for evaluating entry/exit criteria for MR RRM relaxation and serving cell measurement offloading

Issue 1-1-4-1: Entry/exit criteria evaluation when results from MR and/or LP-WUR are avaiable for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

Accoring to RAN2 agreement,:

- RAN2#127: Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ. - RAN2#127: RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

For monitoring LP-WUS, the entry/exit criterion should be based on the LR-based requirement. Considering LP-WUS entry case, there are no LR-based measurement results yet, so, initially it should be decided based on the MR results. But even though the UE met the MR-based requirement, it does not gurantee of good enough LP-WUS quality for monitoring. So, LR-based check also necessary after MR-based check which mean 2-step check is necessary. Different from LP-WUS entry case, considering LP-WUS exit case, the LR-based condition check only could be enough.

Proposal 5: RAN4 to define requirements of MR/LR based entry condition and LR based exit condition for LP-WUS monitoring.

LP-WUR measurement entry/exit criteria also should be based on the LR-based requirement like LP-WUS monitoring. But the LR-based threshold can be different from LP-WUS monitoring LR-based threshold. We think that for LP-WUS monitoring, at least the LP-WUR meausement should be activated concurrently. So, the LR-based LP-WUR measurement threshold can be less than or equal to the LR-based LP-WUS monitoring threshold.

Proposal 6: For LP-WUS monitoring, at least the LP-WUR meausement should be activated concurrently. So, the LR-based LP-WUR measurement threshold can be less than or equal to the LR-based LP-WUS monitoring threshold.

Considering entry condition for LP-WUR measurement, like LP-WUS monitoring, there is no LR-based measurement results. So, LP-WUR measurement also need to 2-step check like LP-WUS monitoring case.

Proposal 7: RAN4 to define requirements of MR/LR based entry condition and LR based exit condition for LP-WUR measurement.

For MR RRM relaxation, the MR-based requirement should be necessary. Before discuss the entry/exit criteria for MR RRM relaxation, we need to clarify about the entry/exit MR RRM relaxation. From the figure below, we need to consider two cases for entry/exit MR RRM relaxation respectively as follow:

- MR RRM relaxation Entry

  - Normal MR measurement  MR RRM relaxation

  - Serving cell measurement offloading  MR RRM relaxation

- MR RRM relaxation Exit

  - MR RRM relaxation  Normal MR measurement

  - MR RRM relaxation  Serving cell measurement offloading

So, we need to consider the criteria separately for the each cases. For simple description we named them as below:

- Scen1: The criteria from normal MR measurement to MR RRM relaxation

- Scen2: The criteria from serving cell measurement offloading to MR RRM relaxation

- Scen3: The criteria from MR RRM relaxation to normal MR measurement

- Scen4: The criteria from MR RRM relaxation to serving cell measurement offloading

Among them except Scen2 the criteria should be based on MR-based requirement. In Scen2, there is no MR-based measurement results, so in that scenario the LR-based requirement can be considered. Additionally in Scen1, the LP-WUR measurement activation also need to check. Because the MR RRM relaxation can be activated when LP-WUR measurement also activated situation. When the UE is on serving cell measurement offloading state, it can be considered that the LP-WUR measurement is activated. So, in Scen2, it is not necessary to check whether the LP-WUR measurement is activated.

Proposal 8: For simple description we named them as below:

- Scen1: The criteria from normal MR measurement to MR RRM relaxation

- Scen2: The criteria from serving cell measurement offloading to MR RRM relaxation

- Scen3: The criteria from MR RRM relaxation to normal MR measurement

- Scen4: The criteria from MR RRM relaxation to serving cell measurement offloading

The MR RRM relaxation entry/exit criteria should be based on MR-based requirement and need to check whether the LP-WUR measurement is activated. But in Scen2 case, the LR-based requirement can be considered and it is not necessary to check whether the LP-WUR measurement is activated.

Issue 1-1-4-2: Reference signals and measurement types to be used for Entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

As mentioned previous, for entry/exit criteria evalution for WUS paging monitoring, LR-based requirements should be baseline. And as the reference signal for LR-based requirement, LP-SS can be considered. Regarding the measurement metrics, following the RAN2’s decision, the UE can use (LP-)RSRP and optional (LP-)RSRQ.

Proposal 9: As the reference signal for LR-based requirement, LP-SS can be considered. Regarding the measurement metrics, following the RAN2’s decision, the UE can use (LP-)RSRP and optional (LP-)RSRQ.

According to RAN2 agreement, for entry criteria evaluation for serving cell measurement offloading, MR-based RSRP threshold can be considered. And for exit criteria evaluation for serving cell measurement offloading, LR-based RSRP only can be considered because there are no MR-based measurement results.

Different from serving cell measurement offloading, in the case of Scen2 and Scen4 from proposal 8, MR-based RSRP threshold can be considered.

The prerequisite requirement for serving cell measurement offloading or MR RRM relaxation is that LP-WUR measurement should be activated.

Proposal 10: RAN4 to define RRM requirement for MR/LR based RSRP measurement for entry criteria evaluation for serving cell measurement offloading, and LR based RSRP measurement for exit criteria evaluation for serving cell measurement offloading.

Proposal 11: RAN4 to define RRM requirement for MR based RSRP measurement for entry/exit criteria evaluation for MR RRM relaxation (Scen2 and Scen4 in proposal 8). Additionally UE mobility condition also should be considered.

Proposal 12: It is UE implementation for LP-WUR measurement at edge area.

### Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR measurement

The criteria for LP-WUR measurement and LP-WUS monitoring should be based on LR measurement results. But the role of LP-WUR measurement and LP-WUS monitoring are different from each other. So, the criteria threshold can be independent. According to the RAN2 agreement, the metrics for serving cell quality measured by MR/LR includes (LP-)RSRP and optional (LP-)RSRQ can be used as criteria metric for entry condition for LP-WUS monitoring. We think it can be used for LP-WUR measurement also. And the metrics for serving cell quality measured by LR includes (LP-)RSRP and optional (LP-)RSRQ can be used as criteria metric for exit condition for LP-WUS monitoring. We think it also can be used for LP-WUR measurement.

- RAN2#127: Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

Proposal 13: RAN4 to define requirements for the metrics for serving cell quality measured by MR/LR includes (LP-)RSRP and optional (LP-)RSRQ for entry condition, and the metrics for serving cell quality measured by LR includes (LP-)RSRP and optional (LP-)RSRQ for exit condition for LP-WUR measurement and LP-WUS monitoring independently.

### Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring

As mentioned above in Issue 1-1-5, the entry/exit condition criteria for LP-WUS monitoring is similar to the LP-WUR measurement

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

### Issue 1-2-1: Accuracy requirements

In the RAN4#110bis meeting, RAN4 discussed whther/how to define the accuracy requirements for LP-WUR in Idle/Inactive state and some agreements and information were made below.

RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: - RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples. - RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement. For information only: RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: Options for further discussion (the options are only for information purpose) - Option 1): Define the dedicated accuracy requirement in the performance section, FFS whether/how it can be verified in the test case. - Option 2): No dedicated accuracy requirement in the performance section, and reflect the accuracy performance as a margin in the core requirement. Option 2a): FFS accuracy requirement, and reflect the accuracy performance in delay requirement in core part. - Other options are not precluded.

Basically, in Idle/Inactive state, there is no measurement reporting, so core requirement for Idle/Inactive state is measurement delay based on the number of samples to meet measurement accuracy at certain side condition. Therefore, no dedicated accuracy requirement is needed.

Proposal 14: Do not define measurement accuracy requirements for LP-WUR in Idle/Inactive state in performance section.

MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

As mentioned above section 2.1, MR based measurement relaxation for serving cell and/or nighbor cells should be considered for UE supporting LP-WUR. Detailed criterion for measurement relaxation would be further discussed based on the agreed measurement relaxation scenarios, and then MR RRM relaxation factor for serving and/or neigbhor cell.

Proposal 15: RAN4 to further discuss scaling factor for MR RRM relaxation for serving and/or neigbhor cell after scenario and detailed criterion for measurement relaxation have been clarified.

LP-WUR CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

Based on the objectives for LP-WUS/WUR, RAN4 needs to clarify the overall scope of RRM core requirements. LP-WUS design commonly applicable to both IDLE/INCATIVE and CONNECTED modes, but RAN4 needs to only focus on IDLE/INACTIVE mode related core requirements.

Proposal 16: Focus on RRM core requirements only for IDLE/INACTIVE mode in Rel-19.

Conclusion

In this contribution, we provide views on issues listed in WF [1], and we propose

Proposal 1: Support option 2. From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR.

Proposal 2: According to the channel quality the three cases are can be necessary.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | On |
| #2 Relaxed case a | On with relaxation measurement | Off | On |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | On |
| #5 Legacy case | On | On | Off |

Proposal 3: Further MR based RRM measurement relaxation for serving cell and/or neighbour cells should be applied when LP-WUR measurement is performed.

Proposal 4: RAN4 to discuss separately following two issues:

- On requirements for evaluating entry/exit criteria for LP-WUS paging monitoring

- On requirements for evaluating entry/exit criteria for MR RRM relaxation and serving cell measurement offloading

Proposal 5: RAN4 to define requirements of MR/LR based entry condition and LR based exit condition for LP-WUS monitoring.

Proposal 6: For LP-WUS monitoring, at least the LP-WUR meausement should be activated concurrently. So, the LR-based LP-WUR measurement threshold can be less than or equal to the LR-based LP-WUS monitoring threshold.

Proposal 7: RAN4 to define requirements of MR/LR based entry condition and LR based exit condition for LP-WUR measurement.

Proposal 8: For simple description we named them as below:

- Scen1: The criteria from normal MR measurement to MR RRM relaxation

- Scen2: The criteria from serving cell measurement offloading to MR RRM relaxation

- Scen3: The criteria from MR RRM relaxation to normal MR measurement

- Scen4: The criteria from MR RRM relaxation to serving cell measurement offloading

The MR RRM relaxation entry/exit criteria should be based on MR-based requirement and need to check whether the LP-WUR measurement is activated. But in Scen2 case, the LR-based requirement can be considered and it is not necessary to check whether the LP-WUR measurement is activated.

Proposal 9: As the reference signal for LR-based requirement, LP-SS can be considered. Regarding the measurement metrics, following the RAN2’s decision, the UE can use (LP-)RSRP and optional (LP-)RSRQ.

Proposal 10: RAN4 to define RRM requirement for MR/LR based RSRP measurement for entry criteria evaluation for serving cell measurement offloading, and LR based RSRP measurement for exit criteria evaluation for serving cell measurement offloading.

Proposal 11: RAN4 to define RRM requirement for MR based RSRP measurement for entry/exit criteria evaluation for MR RRM relaxation (Scen2 and Scen4 in proposal 8). Additionally UE mobility condition also should be considered.

Proposal 12: It is UE implementation for LP-WUR measurement at edge area.

Proposal 13: RAN4 to define requirements for the metrics for serving cell quality measured by MR/LR includes (LP-)RSRP and optional (LP-)RSRQ for entry condition, and the metrics for serving cell quality measured by LR includes (LP-)RSRP and optional (LP-)RSRQ for exit condition for LP-WUR measurement and LP-WUS monitoring independently.

Proposal 14: Do not define measurement accuracy requirements for LP-WUR in Idle/Inactive state in performance section.

Proposal 15: RAN4 to further discuss scaling factor for MR RRM relaxation for serving and/or neigbhor cell after scenario and detailed criterion for measurement relaxation have been clarified.

Proposal 16: Focus on RRM core requirements only for IDLE/INACTIVE mode in Rel-19.

Reference

[1] R4-2413899, “WF for RRM requirements for LP-WUSWUR,” vivo