3GPP TSG-RAN WG4 Meeting #112bis	R4-2415368

Hefei, China, 14th-18th, October, 2024

Title: 	PSD imbalance impact to MPR

Agenda Item:	6.1.1.1.2

Source: 	Nokia

Document for:	Approval

# Introduction

In previous RAN4 meeting #112 there was discussions that MPR may not be sufficient in case of large PSD imbalance if it is derived with equal PSD measurements or simulations [1].

# Discussion

WF [2] from RAN4#112 had following online agreement for this topic

Online agreement:

- MPR and CANS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on TxD architecture and may account for PSD imbalance with up to 6dB.

  - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB

- PC1.5 contiguous intra-band ULCA based on dualPA architecture with two LOs is not specified in R19

- MPR studies for PC1.5 non-contiguous intra-band ULCA focusses on dualPA architecture with two LOs and may account for PSD imbalance with up to 6dB.

  - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB

- PC1.5 non-contiguous intra-band ULCA based on TxD architecture is not specified in R19

As was seen from [1] PSD imbalance can be problematic from UE emission point of view. Some of the PSD imbalance cases presented in [1] had quite big imbalance compared to 6 dB online agreement, but some seemed to be within 6 dB imbalance range. There likely will be studies presented in this meeting and some decisions need to be taken if it is shown that MPR based on equal PSD is not enough for cases of unequal PSD.

One proposed approach is to define MPR based on worst case emissions covering all possible MSD imbalance scenarios, this is not our preference as equal PSD is the likely usage scenario and bigger MPR based on unequal PSD impacts network performance.

Another proposed approach is to define baseline MPR for equal PSD assumption as RAN4 has done before when MPR is based on 1 PA architecture and define bigger MPR for cases that have unequal PSD between PAs. Similarly, as for MPR we would prefer separate sets of A-MPR instead of one, in case equal PSD case would require additional A-MPR because of un-equal PSD condition. At least CA_NS_04 seems to be of interest in this WI.

Proposal: In case un-equal PSD condition would require more MPR or A-MPR than equal PSD condition then separate sets of MPR and A-MPR are defined in-order not to degrade the performance of equal PSD case which is the typical use case.

# 3	Conclusion

In this contribution we have discussed that separate sets of MPR and A-MPR should be defined in-order not to degrade the performance of equal PSD case which is the typical use case.

Proposal: In case un-equal PSD condition would require more MPR or A-MPR than equal PSD condition then separate sets of MPR and A-MPR are defined in-order not to degrade the performance of equal PSD case which is the typical use case.

# 4	References

[1] R4-2411302, 8.1.1.1.1	Intra-band contiguous and non-contiguous UL CA with PC1.5, Skyworks, RAN4#112

[2] R4-2412819, Topic summary for [112][117] NR_ENDC_RF_Ph4_part2, Samsung, RAN4#111