3GPP TSG-RAN WG4 Meeting #112    	R4-2415371

Hefei, China, 14th – 18th Oct, 2024

Title: 	Final remarks on 6Rx for handheld and FWA REFSENS

Agenda Item:	6.1.1.3.1

Source: 	Nokia

Document for:	Approval

# 1	Introduction

In previous meeting there were good progress on 6 Rx REFSENS. Open item mentioned in WF [1] was the verification aspect for which we share our view.

# 2	Discussion

Verification aspect in WF [1]

Issue 1-1-3: Verification of 6Rx receiver requirements

Way Forward: Companies encouraged to provide their views on the ZTE proposal in Proposal 5 in R4-2411883 and any further positions on verification of 6Rx receiver requirements at future meetings given that ΔRIB,6R values are now defined.

Refers to [2] Proposal 5 and is as follows

Proposal 5. When the UE is equipped with six Rx antenna ports:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands.

We agree with this proposal with small fine tuning as below

Proposal: When the UE is equipped with six Rx antenna ports:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in all applicable operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in all applicable operating bands.

Our understanding is that ΔRIB,6R values will be defined as in table 1 and 2 below.

Table 1: ΔRIB,6R,6R for FWA

| Operating Bands | ΔRIB,6R (dB) |
| --- | --- |
| Band n41 | -3.4 |
| Band n77, n78, n79, n104 | -3.0 |

Table 2: ΔRIB,6R for FWA

| Operating bands | ΔRIB,6R for FWA |
| --- | --- |
| n77, n78, n79, [n104] | -3.4 dB |
| n41 | -4.0 dB |

# 3	Conclusion

In this contribution we have provided our view to verification aspect which was requested in [1]

Proposal: When the UE is equipped with six Rx antenna ports:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in all applicable operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in all applicable operating bands.

As the ΔRIB,6R are agreed for applicable bands and for both handheld and FWA we think that RAN4 is ready to complete the topic with formal CR in November RAN4 meeting,

# 4 References

[1] R4-2414285, WF on 6Rx UE requirements, RAN4#112

[2] R4-2411883, Further discussion on 6Rx Reference sensitivity requirements, RAN4#112