3GPP TSG-RAN4 #112-bis	R4-2415377

Hefei, Anhui, China, 14 Oct. – 18 Oct. 2024

Title:	Views on TE emulated channel model for satellite motion

Source:			Anritsu Corporation

Agenda Item:			6.8.4

Document for:			Approval

# 1.	Introduction

At the last RAN4 #112 meeting in Maastricht, the group discussed candidates of the TE emulated channel model for time varying doppler and delay shifts modelling [1]. Agreed options in the way forward are extracted below for reference.

Issue 1-2-1 Methodology for Time varying Doppler and Delay shifts modelling - Agreement - RAN4 aims to introduce TE-emulated time varying doppler shift and delay shift model for satellite motion with following candidate options: - Option 1: Eckstein-Hechler model - Option 2: Keplerian model - Option 3: Existing model from TR 38.811 5.3.4.3 - Option 4: Proposal 2 from R4-2412552 (The actual formulae described are omitted.) - Option 5: Proposal 2 from R4-2412783 (The actual formulae described are omitted.) - Other options not precluded Note: Companies are encouraged to bring more information for option 1 and option2. - Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model - FFS on the initial ephemeris information and parameters

In this contribution we discuss our views on the identified issue with the Option 1: Eckstein-Hechler model.

# 2.	Discussion

  - Gravitational potential field models

First, according to the reference article [2, p.385], the gravitational potential field is expressed in terms of spherical coordinates as follows. Note each factor in the formula is explained minimum below for simplicity of the discussion.

Vr,ϕ, θ=-Gmr-k=1∞1rk+1AkPksinϕ+j=1kPkj(sinϕ)Bkjcosjθ+ Ckjsinjθ　#2.1-1

Here -Gm/r is Keplerian motion, and so-called the zonal terms refer to terms of the form:

Pk(sinϕ)rk+1#2.1-2

and the tesseral terms refer to terms of the form:

Pkjsinϕcosjθrk+1#2.1-3

Pkjsinϕsinjθrk+1#2.1-4

Next, based on the description on GitHub [4], we can find that Eckstein-Hechler model can be expressed with zonal harmonics up to J6, and no tesseral terms.

Figure 2.1-1: Comparison of models for Earth gravity modelling on GitHub

Here Jk is obtained also from the article [2, p.388] with req being the equatorial of the Earth.

Jk=Akreqk#2.1-5

Therefore, modifying the previous formula 2.1-1, Eckstein-Hechler model can be expressed as follows.

Vr,ϕ=-Gmr+1-k=26reqrkJkPk(sinϕ)#2.1-6

And for the Earth, the associated zonal harmonics are given by

J2 = 1082.63 x 10-6

J3 = -2.52 x 10-6

J4 = -1.61 x 10-6

J5 = -0.15 x 10-6

J6 = 0.57 x 10-6

As can be seen above, the J2 harmonic is clearly the dominant harmonic and the rest of them are minor. And associated gravitational perturbation functions are described at annex A at the end of this paper.

2.2 Computation effort of test equipment for Option 1: Eckstein-Hechler model

Using the obtained gravitational perturbation function R(r) for the J2 through J6, we roughly estimated the amount of calculation by test equipment. Compared to the case that the TE computes the gravitational potential with dominant J2 zonal harmonic, Eckstein-Hechler model with up to J6 harmonics takes roughly at least 6 times for the calculation even for the one-way transmission like DL or UL between one satellite.

Observation 1: Option 1: Eckstein-Hechler model takes roughly at least 6 times than the case using J2 harmonic for the calculation even for the one-way transmission between one satellite.

If we only see periodicity of SIB19 updates which is now discussed at least 10.24 sec, it appears that the calculation with Eckstein-Hechler model has no issues, which is expected to take roughly the order of sub-milliseconds for one-way transmission by test equipment. However, due to some additional factors, we prefer the simpler model like Keplerian.

- Since bands for NTN is FDD, calculation doubles for DL and UL

- As shown in sub-clause 2.1 with actual values of zonal harmonics from J2 to J6, we expect that option 1: Eckstein-Hechler model does not show significant improvement compared to option 2: Keplerian model while option 1 has a big increase in the calculation of test equipment

- Even though SIB19 may be provided every 10.24 sec, the test equipment actually needs to update the timing advanced (TA) values internally with a slot basis, by either a calculation with a millisecond period or by an interpolation method which needs a further study with the resolution and its calculation period.

- Since the test equipment may also need to calculate orbits for multiple satellites up to 5 including 4 neighbor cells maximum in parallel to update an ephemeris information of SIB19, a total amount of calculation should be minimized

Observation 2: Even though SIB 19 may be provided every 10.24 sec, the test equipment actually needs to update the TA values internally with a slot basis in any way.

Observation 3: Considering the TA value update period, calculation of satellite motion in the test equipment may become an order of milliseconds period. In that case the TE may face an issue with the calculation with the Eckstein-Hechler model when we also take into account the previously mentioned factors such as minimum 6 times calculation increase, separate DL/UL calculations and maximum 5 satellites.

Observation 4: From the test equipment viewpoint, a total amount of calculation should be minimized as long as an accuracy of the TE-emulated channel model is sufficient compared to GMAT.

Proposal 1: Agree option 2: Keplerian model or an equivalent model with Keplerian for TE

- Conclusion

In this contribution we discussed our views on the identified issue with the Option 1: Eckstein-Hechler model.

Observation 1: Option 1: Eckstein-Hechler model takes roughly at least 6 times than the case using J2 harmonic for the calculation even for the one-way transmission between one satellite.

Observation 2: Even though SIB 19 may be provided every 10.24 sec, the test equipment actually needs to update the TA values internally with a slot basis in any way.

Observation 3: Considering the TA value update period, calculation of satellite motion in the test equipment may become an order of milliseconds period. In that case the TE may face an issue with the calculation with the Eckstein-Hechler model when we also take into account the previously mentioned factors such as minimum 6 times calculation increase, separate DL/UL calculations and maximum 5 satellites.

Observation 4: From the test equipment viewpoint, a total amount of calculation should be minimized as long as an accuracy of the TE-emulated channel model is sufficient compared to GMAT.

Proposal 1: Agree option 2: Keplerian model or an equivalent model with Keplerian for TE

# 4.	References

[1] R4-2413516, “Way Forward for [112][329] NTN_testing_NGSO_channel_model”, Samsung, RAN4 #112, Maastricht, Netherlands

[2] “Analytical Mechanics of Space Systems”, Hanspeter Schaub and John L.Junkins, 4th edition

[3] A reliable derivation of the perturbations due to any zonal and tesseral harmonics of the geopotential for nearly-circular satellite orbits, M. C. Eckstein, F. Hechler, ESRO SR-13-1970, ESA, Darmstadt, Germany, 1970.

[4] GitHub - Nosudrum/orbit-propagation-python-study: Repository started as part of a 2022 study on Python libraries for orbit propagation.

Appendix A: Gravitational perturbation function from J2 through J6 [2]

Setting m = Gm, the gravitational perturbation function R(r) for the J2 through J6 is given by the formula 2.1-6 as follows.

Computing the gradient of R(r) and making use of z/r = sin f, the perturbing acceleration aJi due to Ji is given in terms of inertial Cartesian coordinates as follows.