# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415380

# Hefei, Anhui, China, 14th – 18th October 2024

Agenda Item:	6.9.1

Source: 	Intelsat

Title: 	Regulatory Landscape and Satellite Use Cases in the Ku Band

Document for:	Discussion and Decision

# Introduction

In this contribution, the Regulatory Landscape of the Ku Band in three ITU Regions between 10.7-14.5 GHz, is described. Furthermore, several satellite use cases of the Ku Band are identified.

This an accompanying document of the NTN Ku Band WID [1]

It is noted that the ITU uses Earth Station in Motion (ESIM) to refer to an Earth station that is placed on a moving platform, such as a ship at sea, a moving train, or an aircraft in flight. ESIM provides broadband communications, including Internet connectivity, on platforms in motion. In 3GPP the ESIMs are referred to as Mobile VSATs.

The Regulatory information in this contribution is correct at the time of submission October 2024.

# Discussion

The following Table 1 illustrates the Regulatory assignment of Ku Band to the Fixed Satellite Services (FSS) across the three ITU Regions and is based on the ITU Article 5 Table of Frequency Allocation [2]. The fourth column illustrates how the spectrum is being used by the FSS industry today and highlights the major trends in the future. Today, over 107 FSS GSO satellite systems are operational and using Ku Band spectrum across the globe, with another 12 GSO FSS satellites being developed. While non-GSO today have 582 satellite constellations using this band with a further 36 being launched in the near future. Overall, there are thousands of satellites already operational or planned to use the Ku-band, with the trend expected to continue growing as more operators seek to provide global broadband coverage. From the Table 1 it can be observed that the frequency band 10.7- 14.5 GHz has been Harmonised Globally to provide FSS services in all three ITU Regions.

The allocated FSS bands can be used for GSO and non-GSO Satellites.  See also the Technical Annex in Section 6.

Table 1 Regulatory assignment for Satellite Service in the Ku Band in three ITU Regions, Current Use and Future Trends [2]

| Allocation to services |  |  |  |  |  | Satellite Current Use and Future Trends |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 |  | Region 2 |  | Region 3 |  |  |
| 10.7-10.95 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 (Earth-to-space) 5.484 MOBILE except aeronautical mobile |  | 10.7-10.95 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 MOBILE except aeronautical mobile |  |  |  | Use of this band is intensifying with more Video via FSS to remain strong in many regions. Media global market size at $86 billion by 2030 Land, Aero and maritime ESIMs to deploy. Increasing use by High Throughput Satellites, future expansion into Software Defined Satellites (SDS). NGSO systems to deploy. |
| 10.95-11.2 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B (Earth-to-space) 5.484 MOBILE except aeronautical mobile |  | 10.95-11.2 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B MOBILE except aeronautical mobile |  |  |  | The current heavy usage is expected to continue and grow and more focus on High Throughput Satellites, with increasing throughput, are expected. Video via FSS to remain strong, VSAT and other data networks important. Use of Mobility services (Aeronautical and maritime) mobile terminals well established over many years and used increasing with ESIMs. Socio-economic benefits to skyrocket from $13 billion in 2022 to $118 billion in 2030 Non-GSO satellite networks (e.g. MEO and LEO) systems to be deployed in this band |
| 11.2-11.45 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 (Earth-to-space) 5.484 MOBILE except aeronautical mobile |  | 11.2-11.45 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 MOBILE except aeronautical mobile |  |  |  | Video via FSS to remain strong in many regions. Land, maritime, Aero ESIMs to deploy. Non-GSO (MEO and LEO) systems to further be deployed in this band. Increasing use by High Throughput Satellites, future expansion into Software Defined Satellites (SDS). Market for current applications expected to grow |
| 11.45-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B (Earth-to-space) 5.484 MOBILE except aeronautical mobile |  | 11.45-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B MOBILE except aeronautical mobile |  |  |  | The current use is expected to continue and grow and more focus on High Throughput Satellites, with increasing throughput, are expected. Providing Broadband delivery for households, education, healthcare, emergency and critical services with a market size of $52 billion socio-economic benefits for 350 million people by 2030 [ref VVA study].[3] Video via FSS to remain strong in many regions, VSAT and other data networks important in some regions, Use of Aeronautical and maritime mobile terminals well established over many years and use increasing with ESIMs. |
| 11.7-12.5 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 |  | 11.7-12.1 FIXED 5.486 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.488 Mobile except aeronautical mobile 5.485 | 11.7-12.2 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 |  |  | BSS Video to remain strong in many regions. Media global market size at $86 billion by 2030 Land, Aero, maritime ESIMs to deploy using non-GSO systems. Software Defined Satellites (SDS) being developed to provide on demand Broadband, Mobility and Media services. Market for current applications expected to grow. Further expansion of High Throughput Satellite usage. |
|  |  | 12.1-12.2 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.488 |  |  |  |  |
|  |  | 5.485 5.489 | 5.487 5.487A |  |  |  |
|  |  | 12.2-12.7 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 | 12.2-12.5 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B MOBILE except aeronautical mobile BROADCASTING |  |  | BSS Video to remain strong in many regions, Aero ESIMs to deploy, non-GSO (e.g. MEO or LEO) systems to deploy. Use of High Throughput Satellites, incl. Software Defined Satellites (SDS). |
| 5.487 5.487A |  |  | 5.487 |  |  |  |
| 12.5-12.75 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B (Earth-to-space) 5.494 5.495 5.496 |  | 5.487A 5.488 5.490 | 12.5-12.75 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B MOBILE except aeronautical mobile BROADCASTING- SATELLITE 5.493 |  |  | For FSS, current use is expected to continue and grow and more focus on High Throughput Satellites, with increasing throughput, are expected. Video via FSS to remain strong in many regions, VSAT and other data networks important in some regions, Use of Aeronautical and maritime mobile terminals well established over many years and use increasing. Use of High Throughput Satellites, incl. Software Defined Satellites (SDS). |
|  |  | 12.7-12.75 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile |  |  |  |  |
| 12.75-13.25 FIXED FIXED-SATELLITE (Earth-to-space) 5.441 5.496A MOBILE Space research (deep space) (space-to-Earth) |  |  |  |  | Video via FSS to remain strong in many regions, while Mobility services such as Land, Aero and Maritime ESIM to be deployed in this band post WRC23. Expansion into High Throughput Satellites, mobile terminals (incl. Aeronautical and Maritime). Software Defined Satellites (SDS) and LEO non-NGSO (MEO and LEO) systems to further deploy |  |
| 13.25-13.4 EARTH EXPLORATION-SATELLITE (active) AERONAUTICAL RADIONAVIGATION 5.497 SPACE RESEARCH (active) 5.498A 5.499 |  |  |  |  | N/A – no commercial satellite services |  |
| 13.4-13.65 EARTH EXPLORATION-SATELLITE (active) FIXED-SATELLITE (space-to-Earth) 5.499A 5.499B RADIOLOCATION SPACE RESEARCH 5.499C 5.499D Standard frequency and time signal-satellite (Earth-to-space) 5.499E 5.500 5.501 5.501B |  | 13.4-13.65 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.499C 5.499D Standard frequency and time signal-satellite (Earth-to-space) 5.499 5.500 5.501 5.501B |  |  |  | New FSS band since 2015 – new satellites designed with this band. |
| 13.65-13.75 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.501A Standard frequency and time signal-satellite (Earth-to-space) 5.499 5.500 5.501 5.501B |  |  |  |  |  | N/A – no commercial satellite services |
| 13.75-14 FIXED-SATELLITE (Earth-to-space) 5.484A RADIOLOCATION Earth exploration-satellite Standard frequency and time signal-satellite (Earth-to-space) Space research 5.499 5.500 5.501 5.502 5.503 |  |  |  |  |  | Revise current ITU radio regulation to alleviate relax operational limitations conditions. Extend mobility applications. |
| 14-14.25 FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.484B 5.506 5.506B RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) 5.504B 5.504C 5.506A Space research 5.504A 5.505 |  |  |  |  |  | Heavy use by all FSS/MSS applications. Use of mobile terminals, including aeronautical and maritime mobile terminals, well established over many years and use is increasing. Increased use of High Throughput Satellites, incl. Software Defined Satellites (SDS). |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.484B 5.506 5.506B RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.508A Space research 5.504A 5.505 5.508 |  |  |  |  |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.484B 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radionavigation-satellite 5.504A | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.484B 5.506 5.506B Mobile-satellite (Earth-to-space) 5.506A Radionavigation-satellite 5.504A |  |  | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.484B 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radionavigation-satellite 5.504A |  |  |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.484B 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Space research (space-to-Earth) 5.504A |  |  |  |  |  |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radio astronomy 5.149 5.504A |  |  |  |  |  |  |

# Ku Band Use Cases

From the fourth column of Table 1, it can be observed that the Ku Band is used extensively for various services such as

- VSAT services

- Aero ESIM

- Maritime ESIM

- Video via Satellites

These services are supported by hundreds of satellites, including high throughput satellites, operating today in this frequency range. Future software Defined Satellite (SDS) satellites are being designed to operate in this frequency range as well.

Furthermore, there are technical and commercial benefits of using Ku Bands for IoT Services and Applications such as

- Industrial IoT which covers

  - Energy and Utilities

  - Agriculture

  - Construction Industry

- Automotive and V2X Services

- Logistics and Transport

- Smart Cities

- Communication on Mobile Platforms

  - Commercial Aero (Broadband services to Aircrafts)

  - Maritime

  - Connectivity to land vehicles, broadband to ships and to trains

- Connectivity UAV

- Emergency Services

- Mobile Edge Computing

- Cellular backhaul e.g. IAB etc.

Which are yet to be exploited.

# Conclusion

The dynamic evolution of satellite communications is transforming global connectivity. The ongoing development of multi-orbit satellite networks, inter-satellite links, and the integration of nonterrestrial networks exemplify the industry’s commitment to expanding and enhancing connectivity solutions. Most of this is being understand within Ku band frequencies.

From the brief discussion presented in this contribution, we can conclude that:

The Ku Band, which covers 10.7-14.5 GHz is harmonised and being used for FSS Services across the ITU three regions.

The band assignments as described in Table 1 are stable and in accordance to the ITU Radio Regulations, and have been used by many satellite operators across the world to offer various services.

By Standardising the allocated Ku Bands for 5G NTN Services, and by using 3GPP Open Standards a new dimension will be opened for future services.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104.

- ITU Radio Regulation Articles Vol 1 2024 Edition. Volume 1, Chapter 2, Article 5 (https://www.itu.int/pub/R-REG-RR)

- The VVA study: https://gsoasatellite.com/wp-content/uploads/GSOA-VVA_LSTelcom_Socio-Economic-Value-of-Satcom_Final_FINAL-en.pdf

# Technical Annex: Satellite Systems in the Ku Band

Satellite Systems in Ku Bands

Based on the Table 2 there are around 200 GSO Ku-band operational satellites and more than more 6500 NGSO Ku-band operational satellites worldwide.

Table 2 SATELLITE SYSTEMS USING Ku-BAND

| Satellite Operator | COVERAGE | TYPE | SATELLITES | NUMBER OF SATELLITES |
| --- | --- | --- | --- | --- |
| Intelsat | Americas | GEO | Galaxy 11, Galaxy 16, Galaxy 17, Galaxy 18, Galaxy 19, Galaxy 28, Galaxy 30, Galaxy 31, Galaxy 32, Galaxy 33, Galaxy 35, Galaxy 36, Galaxy 3C, Horizons 1, Horizons 2, Intelsat 16, Intelsat 21, Intelsat 23, Intelsat 34, Intelsat 902, Intelsat 904, Intelsat 40e | 22 in operation |
|  |  |  | Galaxy 37/Horizons 4, Intelsat 42, Intelsat 43 | 3 future (being manufactured) |
|  | Europe, Africa, Middle East | GEO | Intelsat 10-02, Intelsat 14, Intelsat 17, Intelsat 20, Intelsat 25, Intelsat 28, Intelsat 32e, Intelsat 35e, Intelsat 37e, Intelsat 38, Intelsat 39, Intelsat 901, Intelsat 905, Intelsat 906 | 14 in operation |
|  |  |  | Intelsat 41 | 1 future (being manufactured) |
|  | Asia | GEO | Horizons 3e, Intelsat 1R, Intelsat 10, Intelsat 15, Intelsat 18, Intelsat 19, Intelsat 22, Intelsat 33e | 8 in operation |
|  |  |  | Intelsat 44 | 1 future (being manufactured) |
| SES | EMEA | GEO | ASTRA 1G, ASTRA 1KR, ASTRA 1L, ASTRA 1M, ASTRA 1N, ASTRA 2E, ASTRA 2F, ASTRA 2G, ASTRA 3B, ASTRA 4A, ASTRA 5B, SES-5, Monacosat | 13 in operation |
|  | EMEA | GEO | ASTRA 1P, ASTRA 1Q | 2 future (Under procurement) |
|  | North America | GEO | AMC-1, AMC-3, AMC-6, AMC-15, AMC-21, ASTRA 2C, CIEL-2, QUETZSAT-1, SES-1, SES-2, SES-3, SES-11, SES-15 | 13 in operation |
|  | Global | GEO | ASTRA 2A, NSS-6, NSS-7, NSS-11, NSS-12, SES-4, SES-6, SES-7, SES-8, SES-9, SES-10, SES-12, SES-14, SES-17 | 14 in operation |
|  | Global | GEO | SES-24 | Launched commencing service in 2025 |
|  | Global | GEO | SES-26 | 1 future (Under procurement) |
| Eutelsat Group | Europe, Africa, Middle East | GEO | EUTELSAT 12WE, EUTELSAT 8WB, EUTELSAT 7WA, EUTELSAT 5WB, EUTELSAT 3E, EUTELSAT 7B, EUTELSAT 7C, EUTELSAT 9B, EUTELSAT 10B, HOT BIRD 13B, HOTBIRD 13C, HOTBIRD 13E, HOTBIRD 13F, HOTBIRD 13G, EUTELSAT 16A, EUTELSAT 21B, EUTELSAT 33E, EUTELSAT 36B, EUTELSAT 36C, EUTELSAT QUANTUM | 20 in operations |
|  | Asia | GEO | EUTELSAT 70B, EUTELSAT 172B | 2 in operations |
|  | Americas | GEO | EUTELSAT 65WA, EUTELSAT 113WA, EUTELSAT 115WB, EUTELSAT 117WA, EUTELSAT 117WB, EUTELSAT 133WA, EUTELSAT 139WA | 7 in operations |
|  | Global | GEO | EUTELSAT 36D, EUTELSAT FLEXSAT | 2 future satellites |
|  | Europe, Africa, Middle East | GEO | KONNECT, KONNECT VHTS, KASAT | 3 satellites having TTC in Ku band |
| Rascomstar | Africa + South Europe | GEO | RQ1R@2.9°E | 1 in operation |
| Hispasat | Europe, North Africa, Americas | GEO | HISPASAT-30W-5, HISPASAT-30W-6, HISPASAT-36W-1 | 3 in operation |
|  | Europe, Americas | GEO | AMAZONAS-2, AMAZONAS-3, AMAZONAS-5, HISPASAT-74W-1 | 4 in operation |
|  |  | GEO | AMAZONAS-NEXUS | 1 future (just launched, reaching GSO) |
| OneWeb | Global | LEO | ONEWEB-xxx Gen-1 | 618 in operation |
| Telenor | Europe | GEO | Thor 5, Thor 6, Thor 7 | 3 in operation |
| Nilesat | MENA, Africa | GEO | Nilesat 201, Nilesat 301 | 2 in operation |
| Yahsat | EMEA and Asia | GEO | Al Yah 1 | 1 in operation |
| ARABSAT | EMEA | GEO | ARABSAT-5A, -6A, BADR4, BADR5, BADR6, BADR7, HellasSat-2, HellasSat-3, & HellasSat-4 | 9 in operation |
|  | EMEA | GEO | ARABSAT-7A, BADR-8 | 2 to be launched |
| Dish | Americas | GEO | ECHOSTAR XVI, ECHOSTAR X, ECHOSTAR XI, ECHOSTAR XIV, TERRESTAR-1 | 5 in operation |
| EchoStar | Americas | GEO | ECHOSTAR IX, AMC-2, ECHOSTAR-105 | 3 in operation |
| Telesat | Americas | GEO | Anik F1, Anik F1R, Anik F2, Anik F3, Anik G1, Nimiq 4, Nimiq 5, Nimiq 6, Telstar 14R, Telstar 19 VANTAGE | 10 in operation |
|  | Americas, Europe, Middle East, Africa | GEO | Telstar 11N, Telstar 12 VANTAGE | 2 in operation |
|  | Asia | GEO | Telstar 18 VANTAGE | 1 in operation |
| Optus | Asia | GEO | Optus C1/Optus D1/Optus D2/Optus D3/Optus 10 | 5 in operation |
|  | Asia | GEO | Optus 11 | 1 future satellite |
| Chinasat | Asia | GEO | Chinasat 6D, Chinasat 9, Chinasat 9B, APSTAR 6C | 4 in operation |
|  | Asia Pacific | GEO | APSTAR 5C, APSTAR 6D, APSTAR 9 | 3 in operation |
|  | Asia, Europe | GEO | Chinasat 10 | 1 in operation |
|  | Asia, Africa, Middle East | GEO | Chinasat 11, APSTAR 7 | 2 in operation |
|  | Asia Pacific, Africa, Europe, | GEO | Chinasat 12 | 1 in operation |
|  | Africa | GEO | Chinasat 15 | 1 in operation |
|  | Asia Pacific, North America | GEO | Chinasat 19 | 1 in operation |
|  | Asia & Middle East | GEO | Chinasat 6E, Chinasat 10R, Chinasat 9C | 3 future satellites |
| AsiaSat | APAC | GEO | AsiaSat 4, AsiaSat 5, AsiaSat 7, AsiaSat 9 | 5 in operation |
|  | EMEA |  | AsiaSat 8 |  |
| JSAT/SJC | APAC | GEO | JCSAT-110A, JCSAT-110R, JCSAT-4B, JCSAT-3A, JCSAT-5B, JCSAT-9, Superbird-C2, JCSAT-1C, JCSAT-2B, Superbird-B3, JCSAT-16 | 11 in operation |
|  |  |  | Superbird-9, JSAT-31 | 2 future satellites |
| MEASAT | APAC | GEO | MEASAT-3A, MEASAT-3B, MEASAT-3D | 3 in operation |
| ARSAT | America | GEO | ARSAT-1, ARSAT-2 | 2 in operation |
| Space-X | Global | LEO | STARLINK-xxx | >6000 in operation |
| NIGCOMSAT | Africa, Europe | GEO | NIGCOMSAT-1R | 1 in operation |
| PT. Bank Rakyat Indonesia (Persero), Tbk. | Asia | GEO | BRIsat | 1 in Operation |
| Chunghwa Telecom Co. Ltd (CHT) Singapore Telecommunications Ltd. (SingTel) | Asia, Southeast Asia | GEO | ST-2 | 1 in Operation |
| TURKSAT | Europe, Africa, Middle East, Central & Southern Asia | GEO | TURKSAT-3A TURKSAT-4A TURKSAT-4B TURKSAT-5A TURKSAT-5B | 5 in operation |
|  |  |  | TURKSAT-6A | 1 Future Satellite (being manufactured) |

Coverage is a simplification, as satellites are often designed to cover oceans with part of continents on both sides.