3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415384

Hefei, China, 14th Oct 2024 – 18th Oct 2024

Agenda item:	6.17.2.1

Source:	MediaTek Inc.

Title:	Discussion on AIML RAN4 CSI Interoperability and testability

Document for:	Discussion

# Introduction

In RAN4#112, RAN4 agreed to bring further aligned simulation results in RAN4#112-bis to check the feasibility of testing for AI/ML CSI compression [1]. In this contribution, we provide our simulation results based on the agreed data generation assumptions and AI/ML model.

# Discussion

For training, validation, and testing data, we follow RAN4’s agreement to reuse system level parameters in TR38.843. Here, we consider the baseline case and ideal channel estimation.

Table 1. Parameters for system level simulation

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme -CSI feedback periodicity (full CSI feedback): 5 ms (baseline) -Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

The detailed structures of ResCNN encoder and decoder for two-sided CSI compression AI/ML model are shown in Figure 1 and 2.

| Figure 1. Detailed model diagrams of encoder | Figure 2. Detailed model diagrams of decoder |
| --- | --- |

We generate training, validation and testing data based on assumptions in Table 1 and use the above encoder and decoder pair for training with the main hyperparameters listed in Table 2. We also use ideal channel estimation.

Table 2. Hyperparameters used for ResCNN model.

| Hyperparameters |  |
| --- | --- |
| Number of samples in training dataset | 600k |
| Number of samples in validation dataset | 60k |
| Quantization | Fixed unform quantization |
| Optimizer | Adam |
| Learning rate | 0.0001 |
| Batch size | 256 |
| Max number of epochs | 150 |
| Stopping criteria | No validation loss improvement over 25 epochs |
| Loss function | 1 - SGCS |
| Drop rate | No drop out |

We provide our results for test set performance, with 336k samples, in Table 3 and 4. The SGCS of AI CSI feedback and eTypeII CSI feedback are both relative to ideal CSI. For the case of 2GHz and 1 layer, it shows that the SGCS of eTypeII and ResCNN are very similar.

Table 3. Rank 1 SGCS for eTypeII and ResCNN with 2 GHz and 15 kHz SCS

|  | Overhead (bits) | SGCS |
| --- | --- | --- |
| eTypeII PC1 | 62 bits | 0.719 |
| ResCNN model 2-bit quantized | 64 bits | 0.697 |
| ResCNN model Float32 | 32*32 bits | 0.753 |

Table 4. Rank 1 SGCS for eTypeII and ResCNN with 4 GHz and 30 kHz SCS

|  | Overhead (bits) | SGCS |
| --- | --- | --- |
| eTypeII PC1 | 62 bits | 0.716 |
| ResCNN model 2-bit quantized | 64 bits | 0.695 |
| ResCNN model Float32 | 32*32 bits | 0.744 |

Observation #1: For the case of 2GHz and 1 layer, the SGCS of quantized ResCNN model is 0.697 with 64 bits feedback. Also, the SGCS of eTypeII PC1 with 62 bits feedback is 0.719.

Observation #2: For the case of 4GHz and 1 layer, the SGCS of quantized ResCNN model is 0.695 with 64 bits feedback. Also, the SGCS of eTypeII PC1 with 62 bits feedback is 0.716.

Companies were encouraged to report the CDF of the SGCS across sub-bands and input samples. Also, If possible, report CDF for eTypeII. We provide CDFs of SGCS for ResCNN (Quantized and non-quantized) and eTypeII in 2GHz and 4GHz in Figure 3.

Figure 3. CDFs of SGCS for ResCNN and eTypeII in 2GHz and 4GHz.

Observation #3: For the case of 2GHz and 4GHz with 1 layer, the comparison of ResCNN and eTypeII CDFs of SGCS indicate that the performance losses surpass the performance gains, leading to an average decrease in overall performance.

We provide our results for model complexity in Table 5 in terms of number of parameters and FLOPs.

Table 5. Complexity of ResCNN AI/ML model

|  | Parameters [M] | FLOPs [M] |
| --- | --- | --- |
| Encoder | 0.127 | 93 |
| Decoder | 1.808 | 1480 |

Observation #4: FLOPs of ResCNN encoder is 93M and FLOPs of ResCNN decoder is 1480M.

Observation #5: Parameters of ResCNN encoder is 0.127M and parameters of ResCNN decoder is 1.808M.

# Conclusion

In this contribution, simulation results for alignment are provided. Considering ResCNN AI/ML model, we have the following observations:

Observation #1: For the case of 2GHz and 1 layer, the SGCS of quantized ResCNN model is 0.697 with 64 bits feedback. Also, the SGCS of eTypeII PC1 with 62 bits feedback is 0.719.

Observation #2: For the case of 4GHz and 1 layer, the SGCS of quantized ResCNN model is 0.695 with 64 bits feedback. Also, the SGCS of eTypeII PC1 with 62 bits feedback is 0.716.

Observation #3: For the case of 2GHz and 4GHz with 1 layer, the comparison of ResCNN and eTypeII CDFs of SGCS indicate that the performance losses surpass the performance gains, leading to an average decrease in overall performance.

Observation #4: FLOPs of ResCNN encoder is 93M and FLOPs of ResCNN decoder is 1480M.

Observation #5: Parameters of ResCNN encoder is 0.127M and parameters of ResCNN decoder is 1.808M.

# Reference

- R4-2414323, WF on AI/ML, Qualcomm