3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415385

Hefei, China, 14th Oct 2024 – 18th Oct 2024

Agenda item:	6.17.2.2

Source:	MediaTek Inc.

Title:	Discussion on CSI compression AI/ML two-sided model logistics

Document for:	Discussion

# Introduction

After RAN4#112, RAN4 had email discussion on AI/ML two-sided model logistics [1]. In this contribution, we provide our views on issues listed in email discussion summary.

# Discussion

In this Chapter, we provide our views on issues listed in email discussion summary.

2.1.1 Training data:

- Companies agree that the current assumptions and estimates for the CSI compression use case are reasonable (Training data size ~1GB up to 10GB(depends on what data will be used and the number of samples) /company (one instance)

- These assumptions are specific to the CSI compression use case in RAN4 and for the current simulation assumptions.

- Larger datasets could be needed/used in future studies depending on the use case. It would be good to maintain some flexibility

- Importance of ensuring proper data upload/download speeds and supporting breakpoint resume due to potential network interruptions – to be discussed with MCC

If we assume dataset size to be 660000 samples in total, then NumPy file size for this data is approximately 4.4GB. Dataset assumptions may change over time, and it is good to reserve enough storage margin for increased dataset sizes.

Observation #1: Dataset file size in NumPy file format for currently agreed dataset size in total is approximately 4.4GB.

Proposal #1: We propose to reserve enough storage margin for increased dataset sizes.

2.1.2 AI/ML Model Storage/Sharing:

- Companies agree that the current assumptions and estimates for the CSI compression use case are reasonable (Model size: ~7MB – 100MB, Upper bound should be 600MB per company per year -> ~10GB per year)

- These assumptions are specific to the CSI compression use case study in RAN4

- The upper bound is assuming that a company contributes in each meeting within a year (6 WG meetings)

If we assume TensorFlow model to be used with currently agreed ResCNN parameters, then file size of total encoder-decoder model is approximately 25MB. Model assumptions may change over time, and it is good to reserve enough storage margin for increased model sizes.

Observation #2: AI model file size in TensorFlow file format for currently agreed ResCNN parameters is approximately 25MB.

Proposal #2: We propose to reserve enough storage margin for increased model sizes.

2.1.3 Testing and inference data:

- Most companies commented that testing and inference data should be a subset of the training dataset so there would be no need to consider the data size separately

- Testing data set should be 10-25% of the training dataset (most companies showed 10-12.5%)

- Other details on partitioning or how to use the datasets would have to be discussed separately

We see that training, validation, and testing datasets can be combined in one dataset, and we can discuss exact split separately.

Observation #3: Training, validation, and testing datasets can be combined in one dataset.

Proposal #3: We don’t need to consider the size of training, validation or testing data sets separately for the data storage requirements. These sets can all be part of one common dataset when stored. The exact split into training, validation and testing data sets from the combined dataset can be further discussed.

2.2.1 Training/Testing/Inference data:

- Possible formats to be used for the dataset which brought up are: NumPy, Matlab and Parquet

- Formats will have to be discussed with MCC to see which ones can be used in 3GPP.

- Most companies prefer NumPy, some where ok also with Matlab

- The details of the data formatting (how data is represented, what is contained in each field, etc) will have to be discussed and agreed after the file format will be decided

We have been using NumPy file format to store training and validation data. Python and NumPy are open source and free to use and very widely used. Therefore, our preference is to use NumPy file format in 3GPP work.

Observation #4: Python and NumPy are open source and free to use and very widely used.

Proposal #4: We propose to use NumPy file format in 3GPP work.

2.2.2 AI/ML models:

- Possible formats to be used for the AI/ML models are ONNX, PyTorch and TensorFlow

- Formats will have to be discussed with MCC to see which ones can be used in 3GPP

- There are pros/cons for each format, most companies seem to favor ONNX as it is open source, however, more discussion is needed to agree on a format

- Further details on the format (contents of the files and what information needs to be provided) will have to discussed/agreed after deciding on the format

We have been using TensorFlow format for AI model. We have observed some compatibility issues between different tool versions, and it may be better to agree some specific tool versions to be used in 3GPP work. Our preference is TensorFlow format for AI models in 3GPP work, and we are fine to study ONNX if needed for better compatibility.

Observation #5: We have observed some compatibility issues between different tool versions.

Proposal #5: We propose to agree some specific tool versions to be used in 3GPP work.

Proposal #6: We propose to use TensorFlow format for AI models in 3GPP work, and we are fine to study ONNX if needed.

2.3 Other Issues

- How to document the file formats will have to be further discussed

  - Some level of documentation will be needed such that there is a common reference and anyone can process the data

- Different use cases will likely have different needs and all formats discussed here apply for the CSI compression use case

- If there will be a need to specify datasets or models, the level of documentation required might be different(more details might be needed, etc)

- The need for an API for data sharing needs to be discussed

- The “lifetime” of the data needs to be further discussed, some data used in intermediate steps in the RAN4 study might not have to be stored for a long time

We see that when datasets or models or both are shared between companies, the exact content need proper documentation or explicit definitions by 3GPP that all companies would follow, to avoid any confusions and misunderstandings.

Proposal #7: We propose all shared content need either proper documentation or explicit definition by 3GPP.

# Conclusion

In this contribution, we provide our views on issues listed in email discussion summary, and we have made the following observations and proposals:

Observation #1: Dataset file size in NumPy file format for currently agreed training and validation dataset size in total is approximately 4.4GB.

Proposal #1: We propose to reserve enough storage margin for increased dataset sizes.

Observation #2: AI model file size in Tensorflow file format for currently agreed ResCNN parameters is approximately 25MB.

Proposal #2: We propose to reserve enough storage margin for increased model sizes.

Observation #3: Training, validation, and testing datasets can be combined in one dataset.

Proposal #3: We don’t need to consider the size of training, validation or testing data sets separately for the data storage requirements. These sets can all be part of one common dataset when stored. The exact split into training, validation and testing data sets from the combined dataset can be further discussed.

Observation #4: Python and NumPy are open source and free to use and very widely used.

Proposal #4: We propose to use NumPy file format in 3GPP work.

Observation #5: We have observed some compatibility issues between different tool versions.

Proposal #5: We propose to agree some specific tool versions to be used in 3GPP work.

Proposal #6: We propose to use TensorFlow format for AI models in 3GPP work, and we are fine to study ONNX if needed.

Proposal #7: We propose all shared content need either proper documentation or explicit definition by 3GPP.

# Reference

- E-mail discussion on data sharing for AI/ML, Qualcomm