3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415390

Hefei, China, 14 – 18 October, 2024

Source:	Nokia

Title:	Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers

Agenda item:	6.6.2

Document for:	Discussion

Introduction

At RAN#104 a new study item was agreed on addressing the issues raised by operators to optimize UE receiver conditions with fragmented carriers. The latest version of the SI is found in [1].

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements for study purpose are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

This contribution discusses the aspects of methods for reducing the number of UE Rx RF chains for Fragmented Carriers.

Agreements at previous meetings

At RAN4#112 the following were captured in the WF [5]

Agreement:

- The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain

  - The two non-contiguous CCs is still under the current DL non-contiguous CA framework

      - Each fragment is treated as an individual CC

  - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz.

  - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands

    - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands.

      - Provide the details related to gap between fragmented DL blocks]

Discussion

The following will discuss what is understood by fragmented carriers and how it relates to the number of needed UE Rx RF chains. In previous discussions and contributions, multiple terms/expressions have been used such as ‘UE Rx chain’/‘RF chain’/‘Rx chain’/‘Rx RF chain’. Our preferred term and understanding of it will be elaborated further in this contribution. Finally, some suggestions for what RAN4 shall focus on when wanting to reduce the number of needed UE Rx RF chains for fragmented carriers are discussed.

Fragmented Carriers and their relation to the number of Rx RF chains

Examples to present the cases of fragmented carriers are shown in the figure below from [2].

Fig. 1 Examples of fragmented carriers [2]

Fragmented carriers are not something completely new for RAN4, since it is essentially non-contiguous intra-band carrier aggregation (NC IB CA). However, proponents of the new SI have chosen to name it Fragmented Carriers, since this is how it is perceived mainly from an operator perspective. Adding a specific name also makes sense to associate the non-contiguous intra-band carrier aggregation with the request for UEs to support downlink reception of the fragmented spectrum block with a single receiver.

Fragmented carriers are in some sense similar to non-contiguous carrier aggregation which is already specified for intra-band carrier aggregation in TS 38.101-1.

The discussion on what is meant by “Rx RF chain”, is a returning topic within RAN4. The study item has been revised to state:

UE Rx chains (e.g. from separate RF chains per CC to shared RF chains )

Which means the shared RF chains are in plural, so at least two. However, the same sentence use both UE Rx chains and RF chains adding to even more confusion.

In Note 3 it says:

This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

So the “sharing RF chain” is not with an inter-band, but now in single, meaning that for intra-band DL carriers the RF chain (No “s”) can be shared. Indicating a minimum of one receiver.

Next, we have the WF from August [5], which states:

The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain

This is more direct at indicating what an “Rx RF chain” is. It is in singular – not plural, and it is used for receiving the 2CC in non-contiguous DL.

It is not always clear within RAN4 what is meant by “Rx RF Chain”.

As also discussed in [3], a single Rx RF chain for a 2Rx UE by some have been presented to consist of one main receiver and one diversity receiver. It is understood that not simply referring to these as receivers, but choosing to refer to “a single Rx RF chain” introduces a term which refers to the main and all associated diversity receivers operating with the same center frequency and filter BW.

A simplified architecture is illustrated in Fig. 3 with the main and diversity receiver. In this picture each mixer is supported by an oscillator, but it could be the same oscillator. What is important is that the oscillator frequency is the same, when the same signal is to be received from the two antennas.

Fig. 3 Rx RF chain for a minimum 2Rx UE consisting of a main and diversity receiver (FDD).

It would be beneficial for further discussion to agree what the underlying assumptions are, as a basis for a common understanding, to align terminology which can help companies to ensure they discuss the same thing.

For a 2Rx UE, the common RAN4 understanding is that it consists of one main receiver and one diversity receiver. The combination of these receivers forms one Rx RF chain, supported by one down-conversion frequency.

Similar to the previous discussion for 2Rx UEs, it becomes even more inconsistent what is meant by a Rx RF chain for 4Rx UEs (4 antennas) when reviewing literature and previous contributions in RAN4. However, it seems the predominant interpretation is to have 1 main receiver and 3 diversity receivers. Meaning, when using Fig. 3 as a reference, there will be an addition of two diversity receivers.

For a 4Rx UE, the common RAN4 understanding is that it consists of one main receiver and three diversity receivers. The combination of these receivers forms one Rx RF chain, supported by one down-conversion frequency.

For the current UE implementation, the descriptions discussed in [3] assume one Rx RF chain per fragmented carrier which results in the limitations of the maximum number of supported CCs.

For the current UE implementation, one Rx RF chain per fragmented carrier is assumed.

This study item challenges the need for applying one Rx RF chain per fragment. In other words, the ambition is to reduce the number of needed Rx RF chains for fragmented carrier operation as also captured in the WF [5].

However, at RAN4#112 the discussion leading to the WF in [5] seemed to uncover some disagreement on what was understood by different companies to be included under the term “Rx RF Chain”. Hence, it would be beneficial for RAN4 to agree this and capture this in a WF or even better in the TR with a schematic/block diagram.

RAN4 shall define “Rx RF Chain” in relation to fragmented carrier SI and capture this in the TR for reference with a schematic/block diagram.

Based on this proposal we present our version of how to capture this definition:

Definition: One Rx RF chain is the occupation of one down-conversion frequency supporting reception from two or more antennas.

Types of UE receiver architectures for reducing the number of Rx RF chains for fragmented carriers

As previously discussed in [3] there are multiple UE receiver architectures which each have the ability to reduces its need for Rx RF chains. In relation to previous discussion these can be referred to as Type 2 and Type 3.

The differences between the Type 2 and Type 3 UE architectures of [4] may be a bit detailed to follow. Our understanding from [4] is that there is a split at the eLNA (external transceiver LNA) towards two iLNA (internal transceiver LNA) (Type2) and a split only at the DACs (Edit: They must mean ADCs, since this is at the receiver) for CC0 and CC1 (Type 3). However, it is not explaining how the UE may receive the two carriers individually.

Type 2 and Type 3 UE Rx architectures have potential for freeing up hardware resources i.e. Rx RF chains.

To extend the understanding of the different choices for transceiver implementation, we have chosen to follow the principal presented by other companies and also split into three different types with emphasis on Type 2 and Type 3 in the following. To complete this denotation Type 1 is the hardware state we want achieve by freeing up hardware resources i.e. Rx RF chains as described in the introduction.

Type 1 is the single Rx RF chain architecture the SI aims at utilizing for the fragmented carriers.

It can be noted that there is also the possibility to suppress some of the in-gap interference in the digital domain. However, this does not solve the dynamic range issues given by the near-far problem. It can mitigate some of the adjacent channel interference but only when the received signal is within proper dynamic range provided from the analog and mixed signal Rx RF chain. Hence, the focus of this work is on solutions related to the analogue Rx path.

Even though there may be a possibility to suppress some of the in-gap interference in the digital domain, the objective of the SI is to free up receivers hence the focus is on the Rx RF chains.

Type 2 – Internal Rx path split:

There are two ways that the signal can be treated by separate Rx RF chains. It may be split at the front-end components (eLNA), which we see as Type 3, and it may be split inside the transceiver, which we see as Type 2. For Type 2 we would like to show a full picture without discussing the iLNA (internal LNA) split. The UE has two Rx RF chains active, serving the same two antennas, and this allows the UE to center the LO1 and LO2 at the respective centers of each CC, so that the following filtering (analogue channel filters) may attenuate also the in-gap interferer as shown here. In the example of Fig. 4 we consider non-collocation, so the in-gap interferer is seen as a “worst case” condition. In the collocated case, the wanted to interference ratio will be less.

Fig. 4 Type 2 – Internal Rx path split. Rx RF chain shown in dashed boxes for reception of CC1 at two antennas using one down-conversion frequency at LO1.

If this UE is to transition to a single Rx RF chain state (Type 1), it will have to accept an in-gap interferer as shown in the next view of Type 2, that visualizes the “Extra Rx RF Chain” as toned-out colours:

Fig. 5 Type 2 – Internal Rx path split. Unused Rx RF chain is toned-out. Type 2 becomes Type 1.

In this state, the UE is without analogue filtering to suppress the interferer between the carriers, and RAN4 will have to discuss the relaxation of the receiver requirements for the UE to tolerate some level of in-gap interference presence. Since the requirement is based on the presence of an interfering signal, it is closer related to the ACS requirement than to the Reference sensitivity requirement for non-contiguous intra-band carrier aggregation, and a new set of requirements must capture the conditions and the relaxation for the UE state that uses a single Rx RF chain for the reception of the two fragmented carriers in the presence of an in-gap interferer.

For the case that the UE uses a single Rx RF chain for the reception of the two fragmented carriers in the presence of an in-gap interferer, a new set of requirements must capture the conditions and the relaxation.

This means that when a Type 2 implementation is using a subset of its Rx RF chains when operating in fragmented carrier mode the new set of requirements will apply.

A new set of requirements shall apply for a Type 2 implementations when the UE uses one Rx RF chain for the reception of the fragmented carries.

Type 3 – External Rx path split:

The second way that the signal can be treated by separate Rx RF chains is a split at the front-end components by multiple external LNAs (eLNA). This can either be within the main/diversity module, or at an extension from the main/diversity module that allows connection to another receive pin at the transceiver(s). For Type 3 we would like to show a full picture and without discussion of detail whether front-end modules splits at the eLNA or if there’s a separate front-end module that accompanies the main/diversity modules to extend the connection possibilities towards the transceiver(s). Some solutions also have 2 transceivers that allow split of reception in NC IB CA between the two transceivers. The full system view is that the UE has two Rx RF chains active, and this allows the UE to center the LO1 and LO2 at the respective centers of each CC, so that the following filtering (analogue channel filters) may attenuate also the in-gap interferer as shown here.

Fig. 6 Type 3 – External Rx path split. The eLNA split shown within the external modules to the transceiver.

If this UE is to transition to a single Rx RF chain state (Type 1) it will have to tolerate an in-gap interferer as shown in the next view of Type 3, that visualizes the “Extra Rx RF Chain” as toned-out colours:

Fig. 7 Type 3 – External Rx path split. Unused Rx RF chain is toned-out. Type 3 becomes Type 1.

In this state, the UE is in the same conditions as in Type 2, bringing both Type 3 and Type 2 into the state of Type 1 single Rx RF chain reception, so the relaxation of the requirements is the same for Type 2 and Type 3, and the same set of requirements can apply regardless of how the UE supports non-contiguous intra-band carrier aggregation (iLNA split or eLNA split).

For type 3 implementations the same new set of requirements as for type 2 implementations can apply.

As presented previously it has been shown that with both a Type 2 and a Type 3 transceiver implementation it is possible to separate the Rx RF chains (i.e. operate as Type 1). When separating or freeing up Rx RF chains in the Type 2 and Type 3 implementations it is possible to apply these now idle Rx RF chains for reception of other carries if new RAN4 requirements for this type of operation is introduced to the specification.

RAN4 shall focus on requirements for Type 2 (Internal Rx path split) or Type 3 (External Rx path split) implementations regardless of the UE implementation when discussion reducing the number of Rx RF chains for fragmented carriers as operated with the Type 1 implementation.

Use of “extra” Rx RF chain after reducing the number of Rx RF chains for fragmented carriers

To understand the motivation for reducing the number of Rx RF chains for fragmented carriers; the ambition is to “free up” UE’s Rx RF chains to allow the UE to support higher order inter/intra-band carrier aggregation combinations using the freed-up resources.

A motivation for the SI is to “free up” Rx RF chains as these can then be used for additional CCs in a higher order inter/intra-band combination.

Some obvious limitation becomes notable from the details of the systems shown for Type 2 and Type 3, as presented in the previous section.

For Type 2, the spare Rx RF chain will be restricted in use for inter-band combinations that are allowed for other bands to connect towards the internal split inside the transceiver. Hence, there is a finite combination possibility from the extra Rx RF chain.

For Type 3, the spare Rx RF chain will be restricted in use for inter-band combinations that can be routed from the same front-end module, which is stricter than the potential of the Type 2 architecture, which relies on LO and internal port connections among band-groups. Hence, again there is a finite combination possibility from the extra Rx RF chain.

Therefore, the outcome of this study may not be a requirement for a UE to support all expected extensions in higher order inter-band combinations, but UE designs may use this feature to expand the inter-band combinations, provided they have appropriate requirements when operated in the single Rx RF chain state for non-contiguous intra-band carrier aggregation.

There are multiple limiting factors to the availability of Rx RF chains and thereby possible number of DL CCs. A solution for fragmented carriers may depend on the specific combinations of bands.

RAN4 shall focus on the fundamental issue of fragmented spectrum and solutions to reduce the needed Rx RF chains to receive these. However, the end goal of higher order inter/intra-band combinations shall be considered as a part of any solution.

Scheduling of the fragmented carriers within a single Rx RF chain

Currently, when a UE is supporting non-contiguous intra-band carrier aggregation (NC IB CA), the configuration consists of the two or more non-contiguous component carriers (CCs). This is according to normal CA operation and example of current configurations in TS 38.101-1 is CA_n7(2A), CA_n41(4A) and CA_n77(2A).

Current DL non-contiguous intra-band carrier aggregation consists of at least two and up to four component carriers (CCs).

As seen in the examples from TS 38.101-1 for NC IB CA, it can be noted that up to four non-contiguous CCs are specified. For the SI on fragmented carriers, it is stated RAN4 shall consider a single DL band with a frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario.

As captured in the WF [5], RAN4 have agreed to consider each fragment as a single CC, similar to NC IB CA. It can be noted that the number of CCs has implications to base station scheduler considerations, placement of resource grid(s) and also the number of needed FFT procedures required by the UE.

Even though each fragment has been agreed by RAN4 to be considered a separate CC it may focus the further discussion even more if the baseband assumptions on the number of needed FFT can be agreed also. Meaning whether some companies would still consider a single FFT solution for two or more fragmented carriers.

RAN4 to agree whether a separate FFT is assumed per CC/fragment or a single FFT shall be enabled for multiple CCs/fragments.

It can be noted that a prerequisite for a single FFT covering all CCs in the UE's Rx RF chain is that the CCs use a common subcarrier grid. In general, however, this is not the case today in FDD where the 100 kHz channel raster in combination with the minimum guard band even predominantly prevents a common subcarrier grid. To achieve a common subcarrier grid, one of the two CCs would need to be upshifted by 10 kHz. If more than 2 CCs were to be covered by a single FFT, there could be cases where achieving a common subcarrier grid while fulfilling the minimum guard bands would require a channel raster of 15 or 5 kHz.

In bands with 100 kHz channel raster, using a single FFT for two non-contiguous CCs will predominantly require that one of the CCs is placed on the enhanced channel raster. For covering more than two non-contiguous CCs by a single FFT, the 10 kHz granularity may not suffice.

This discussion in RAN4 should also keep in mind that some scenarios may result in more than two spectrum fragments and a solution developed by RAN4 according to the agreed WF [5] should be applicable for this scenario also.

RAN4 shall consider solutions which also can be applicable for fragmented spectrum with more than two spectrum fragments.

Duplex modes for fragmented carrier operation

At RAN4#112 it was clarified in [5] that TDD, FDD and SDL bands are to be considered even though this was still captured in brackets. However, it is still not clear whether there can be separate solutions for TDD and FDD fragmented carriers.

RAN4 shall clarify whether a solution for fragmented carriers shall be applicable for both TDD and FDD bands or whether there can be separate solutions for TDD and FDD fragmented carriers.

While FDD operation will have dedicated uplink and downlink LOs, in TDD these may be shared between uplink and downlink operation. When the non-contiguous intra-band carriers are to be received with a single Rx RF chain, the UE must center the LO frequency at the center between the outermost edges of the DL CCs or DL BWPs received in the single Rx RF chain. This DL LO frequency will, in general, not be suitable for transmitting an UL carrier.

RAN4 shall include into TR 38.755 a note that for TDD fragmented carrier operation, the UE's LO frequency used for the joint reception of two non-contiguous CCs will not be suitable for transmitting an UL carrier.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: Fragmented carriers are in some sense similar to non-contiguous carrier aggregation which is already specified for intra-band carrier aggregation in TS 38.101-1.

Observation 2: It is not always clear within RAN4 what is meant by “Rx RF Chain”.

Proposal 1: For a 2Rx UE, the common RAN4 understanding is that it consists of one main receiver and one diversity receiver. The combination of these receivers forms one Rx RF chain, supported by one down-conversion frequency.

Proposal 2: For a 4Rx UE, the common RAN4 understanding is that it consists of one main receiver and three diversity receivers. The combination of these receivers forms one Rx RF chain, supported by one down-conversion frequency.

Observation 3: For the current UE implementation, one Rx RF chain per fragmented carrier is assumed.

Proposal 3: RAN4 shall define “Rx RF Chain” in relation to fragmented carrier SI and capture this in the TR for reference with a schematic/block diagram.

Proposal 4: Definition: One Rx RF chain is the occupation of one down-conversion frequency supporting reception from two or more antennas.

Observation 4: Type 2 and Type 3 UE Rx architectures have potential for freeing up hardware resources i.e. Rx RF chains.

Observation 5: Type 1 is the single Rx RF chain architecture the SI aims at utilizing for the fragmented carriers.

Observation 6: Even though there may be a possibility to suppress some of the in-gap interference in the digital domain, the objective of the SI is to free up receivers hence the focus is on the Rx RF chains.

Observation 7: For the case that the UE uses a single Rx RF chain for the reception of the two fragmented carriers in the presence of an in-gap interferer, a new set of requirements must capture the conditions and the relaxation.

Observation 8: A new set of requirements shall apply for a Type 2 implementations when the UE uses one Rx RF chain for the reception of the fragmented carries.

Observation 9: For type 3 implementations the same new set of requirements as for type 2 implementations can apply.

Proposal 5: RAN4 shall focus on requirements for Type 2 (Internal Rx path split) or Type 3 (External Rx path split) implementations regardless of the UE implementation when discussion reducing the number of Rx RF chains for fragmented carriers as operated with the Type 1 implementation.

Observation 10: A motivation for the SI is to “free up” Rx RF chains as these can then be used for additional CCs in a higher order inter/intra-band combination.

Observation 11: There are multiple limiting factors to the availability of Rx RF chains and thereby possible number of DL CCs. A solution for fragmented carriers may depend on the specific combinations of bands.

Proposal 6: RAN4 shall focus on the fundamental issue of fragmented spectrum and solutions to reduce the needed Rx RF chains to receive these. However, the end goal of higher order inter/intra-band combinations shall be considered as a part of any solution.

Observation 12: Current DL non-contiguous intra-band carrier aggregation consists of at least two and up to four component carriers (CCs).

Proposal 7: RAN4 to agree whether a separate FFT is assumed per CC/fragment or a single FFT shall be enabled for multiple CCs/fragments.

Observation 13: In bands with 100 kHz channel raster, using a single FFT for two non-contiguous CCs will predominantly require that one of the CCs is placed on the enhanced channel raster. For covering more than two non-contiguous CCs by a single FFT, the 10 kHz granularity may not suffice.

Observation 14: RAN4 shall consider solutions which also can be applicable for fragmented spectrum with more than two spectrum fragments.

Proposal 8: RAN4 shall clarify whether a solution for fragmented carriers shall be applicable for both TDD and FDD bands or whether there can be separate solutions for TDD and FDD fragmented carriers.

Proposal 9: RAN4 shall include into TR 38.755 a note that for TDD fragmented carrier operation, the UE's LO frequency used for the joint reception of two non-contiguous CCs will not be suitable for transmitting an UL carrier.

- References

- RP-242404, Revised SID: Study on NR FR1 DL Fragmented Carriers, MediaTek Inc.

- RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

- R4-2413339, Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, Nokia

- RP-241361, Recommended WF on Fragmented Carriers, MediaTek

- R4-2414457, WF on fragmented DL carriers study, MediaTek