3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415391

Hefei, China, 14 – 18 October, 2024

Source:	Nokia

Title:	Discussion on impacts on UE RF requirements and DL performance for Fragmented Carriers

Agenda item:	6.6.3

Document for:	Discussion

Introduction

At RAN#104 a new study item was agreed on addressing the issues raised by operators to optimize UE receiver conditions with fragmented carriers. The latest version of the SI is found in [1].

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements for study purpose are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

This contribution discusses the impacts on UE requirements and DL performance for Fragmented Carriers.

Agreements at previous meetings

At RAN4#112 the following were captured in the WF [5]

Agreement:

- The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain

  - The two non-contiguous CCs is still under the current DL non-contiguous CA framework

      - Each fragment is treated as an individual CC

  - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz.

  - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands

    - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands.

      - Provide the details related to gap between fragmented DL blocks]

Discussion

Existing requirements

Fragmented carriers are not something completely new for RAN4, since it is essentially non-contiguous intra-band carrier aggregation (NC IB CA). However, adding a specific name makes sense to associate the non-contiguous intra-band carrier aggregation with the request for UEs to support downlink reception of the fragmented spectrum block with a single Rx RF chain as defined in R4-2413339.

Fragmented carriers are in some sense similar to non-contiguous intra-band carrier aggregation (NC IB CA) which is already supported by the specification.

The reception at the UE of non-contiguous intra-band carrier aggregation in a system with a single Rx RF chain will be impacted by several sources of interference.

A first source of interference is the impact from the UE’s own UL transmission, which is reflected in the Reference sensitivity (REFSENS) requirements for non-contiguous intra-band carrier aggregation that allow an MSD in the sense of ΔRIBNC as specified in TS 38.101-1 clause 7.3A.2.2. This is however only present for FDD carriers and considers the placement of the secondary component carrier (SCC) at a distance less than the normal duplexing distance of the band of operation in single carrier configuration. Although there are existing requirements for the relaxation of the sensitivity at the SCC, they are different from what is needed for this SI. Hence, something similar may be used as a new requirement for fragmented carriers but it must be separated from the existing FDD non-contiguous intra-band carrier aggregation ΔRIBNC.

New requirements for fragmented carriers shall be separated from the existing requirements.

This means that fragmented carrier operation shall not be used as a reason for relaxing existing requirements, e.g. ΔRIBNC in FDD non-contiguous intra-band carrier aggregation.

A second source of interference is the impact an adjacent channel interference may have on the reception of the two fragmented carriers, which is reflected in the ACS requirements for non-contiguous intra-band carrier aggregation. This already today mandates a UE to include the potential in-gap interference as written in TS 38.101-1 Clause 7.5A.2:

“The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active”.

Current specification already has a requirement to the UE that it shall fulfil the requirements with an in-gap interferer of up to -25dBm.

It can be noted that the in-gap interference is not tested in the existing ACS test specifications to our understanding. This may need to be addressed within this SI if support of fragmented carrier operation is dependent on a specified level of in-gap interference. It is understood that any performance requirements can wait until a work-item phase for fragmented carriers. Nevertheless, it should be kept in mind by RAN4 that e.g. a given threshold for in-gap interference should be testable.

Testing of any new requirements, e.g. in-gap interference level, for fragmented carriers may need to be considered within this SI.

The existing requirement of being able to fulfil requirements with a –25 dBm in-gap interferer may have set the need for UE implementations to have multiple Rx RF chains that can be applied for attenuation/suppression of the in-gap interferer by having separate analogue channel filters at each Rx RF chain. This additional attenuation/suppression of the in-gap interferer by the analogue channel filters is not possible when a single Rx RF chain has to cover the fragmented carriers.

A main challenge of non-contiguous intra-band carrier aggregation with a single Rx RF chain is the additional sources of interference since the single Rx RF chain has to cover multiple component carriers (CCs) with a potential in-gap interferer.

A third source of interference was highlighted by CHTTL [6] that identifies the image interference that arises when the fragmented carriers have different bandwidths. This will lead to the image of the in-gap interferer affecting the larger of the two fragmented carriers. If the in-gap interference has a larger PSD than the fragmented carriers, the ratio between the wanted signal and the image interference will become smaller than the UE's image rejection ratio in the part of the widest CC where the image is caused by the in-gap interferer which is stronger than the wanted signal.

It must be considered if requirements to the UE can apply independently of the CCs' BW difference, or if two sets of requirements are needed, one focusing on the in-gap interference level for equal carrier bandwidths, and one focusing on the resilience for the image interference.

RAN4 shall discuss whether UE requirements for fragmented carrier operation can be applicable to both equal and unequal bandwidth of the fragment’s carrier bandwidths.

Requirements for fragmented carriers

As described in the previous section, there would be a need to address at least REFSENS and ACS requirements related to operation with fragmented carriers and a single Rx RF chain.

At least REFSENS and ACS requirements shall be discussed in relation to fragmented carriers in DL.

For REFSENS, it is expected that some relaxation may be introduced as compared to the current requirements. These relaxations would mainly be justified by the widening of the total transmission bandwidth to cover both of the two spectrum fragments as compared to a scenario with two narrower filters.

REFSENS relaxation for fragmented carrier operation can be related to the widening of the total transmission bandwidth.

The wider transmission bandwidth would need a larger guard band, as further discussed in section 2.6, as there is a higher attenuation of the RBs at the outermost edges of the CCs and thus a degradation of the sensitivity. Additionally, suppression of Tx noise may be lower in a FDD scenario, requiring additional relaxation.

The sensitivity degradation related to the widening of the transmission bandwidth is dependent on specific UE implementations, why input from UE vendors is needed to progress this discussion.

RAN4 shall encourage UE vendors to provide information on expected sensitivity degradation related to widening of the transmission bandwidth for fragmented carrier operation.

For ACS, there is a need to split the discussion into the selectivity towards a channel adjacent to the fragmented carrier combination (i.e. out-of-gap interference) and the selectivity within the gap of the fragmented carriers (i.e. in-gap interference). As a principal there should be no need for relaxation outside of the fragmented carrier combination if the relaxation can’t be justified. Given the choice of supporting fragmented carriers with a single Rx analogue filter passband, some relaxation should be expected within the fragmented carrier configuration since there is no possibility for the single Rx RF chain's analogue filters to suppress the in-gap interferer. This difference is illustrated in Fig 1.

Fig. 1 ACS comparison between single carrier and fragmented carrier operation.

The ACI attenuation is illustrated on the left-hand side of Fig. 1, where the analogue filter serves to suppress the adjacent channel interfering signal from saturating the ADC of the receiver. At the right-hand side of Fig. 1, the two CCs (wanted signals) are received with an in-gap interfering signal using a single Rx RF chain with an analogue filter bandwidth that covers both wanted carriers and the full in-gap interfering signal with no attenuation. In the case of 2 Rx RF chains, the attenuation would have been provided at the level shown on the left-hand side of Fig. 1, which means that an option is to relax the ACS requirement for fragmented carrier operation as shown at the right-hand side of Fig. 1 by the ACI attenuation on the left-hand side of Fig. 1.

The current ACS requirements for non-contiguous intra-band carrier aggregation assume the presence of analogue filters which are not available for fragmented carrier operation.

The difference in attenuation/suppression of in-gap interference between current implementations and the expected implementation of fragmented carriers can be used to develop the in-gap ACS requirement. The difference in attenuation is heavily impacted by multiple factors related to both the specific scenario of fragmented carriers and the UE implementation. Therefore, to form a basis for discussion of potential ACS relaxations, input from UE vendors is welcomed.

RAN4 shall seek input from UE vendors on the expected difference in interference attenuation between current implementation and fragmented carrier operation.

PSD imbalance of the fragmented carriers

In [3] it is discussed whether or not the power imbalance of the wanted carriers within the fragmented carriers shall be considered. While this is a valid point that there may be an imbalance of the received power of the wanted carriers, the current SID [1] implicitly limits the scenario to co-located wanted carriers within the fragmented carriers

- as it is for “the inter-operator co-located BS scenario” which implies that all relevant carriers – the non-contiguous CCs of the IB CA (i.e. wanted carriers) and any in-gap interferer – are transmitted from the same location and also

- by specifying the maximum of 6dB DL received power spectral density (PSD) imbalance requirement/prerequisite between the non-contiguous CCs.

The SID implies  co-location of the non-contiguous intra-band CA's CCs and hence, a DL received PSD imbalance of 6dB between the wanted carriers within the fragmented carriers is a reasonable limit.

This means that the DL received PSD imbalance of up to 6dB is to be considered with any relaxation RAN4 may allow to enable support of reception of fragmented carriers with a single Rx RF chain.

If power imbalance of the wanted carriers within the fragmented carriers is to be considered, it means that the level of signal-to-in-gap-interference ratio will always be towards the CC of lowest PSD. For that reason, the imbalance should not be treated as an aspect of fragmented CA requirements but treated similarly as it would for all other test cases, such as the NC IB CA FDD REFSENS test that also does not include aspects of power imbalance of the wanted component carriers. Hence, we believe that aspects of power imbalance should not be a part of this study for requirements for fragmented carrier operation.

RAN4 shall not re-discuss the power imbalance assumption of the wanted carriers further in relation to the SI on fragmented carriers.

Expected interference power level

When referring to Fig. 1, the in-gap interference is illustrated between the wanted fragmented carriers. The power level of the interferer is in the presented example significantly larger than the wanted signals mostly for illustration purpose. However, what power level of the in-gap interference a UE can tolerate also needs to be addressed by RAN4. A starting point for the discussion could be the current ACS or IBB requirements.

RAN4 shall consider a maximum level of in-gap interference fragmented carrier operation shall be able to tolerate.

Additionally, RAN4 needs to consider whether the power level of the in-gap interferer needs to be used as a determining factor for the UE to report the possibility of fragmented carrier operation, i.e. the possibility to successfully operate with semi-statically switched hardware resources freeing up Rx RF chains. If so, the need for measuring the in-gap interferer power level may need involvement of RAN4 RRM experts.

RAN4 should consider if the power level of the in-gap interferer shall be used for enabling or disabling fragmented carrier operation.

Regardless of the above, RAN4 needs to consider how the UE is expected to behave if an in-gap interferer precludes the UE to operate in fragmented carrier mode. Is the UE expected to choose one of the fragments (i.e. CCs) or drop the configuration, or parts hereof, completely? This UE fallback behaviour needs to be known by the network, maybe even controlled by the network to facilitate scheduling.

RAN4 shall discuss expected UE fallback behaviour when an in-gap interferer precludes the UE to operate in fragmented carrier mode.

Means for a UE to inform the network of CA configuration it can support with adjusted RF requirements

As mentioned multiple times, fragmented carrier operation has similarities to NC IB CA for which support is mainly indicated via the supported band combination list provided to the network from the UE. Examples of NC IB CA combinations could be CA_n7(2A), CA_n41(4A) and CA_n77(2A) where the non-contiguous part is indicated in the parentheses with the number for the number of CCs and where the letter indicates their size/type.

RAN4 specification already have an agreed syntax for indicating non-contiguous parts of a band combination.

It can be suggested that RAN4 introduces a new syntax for indicating band combinations supported also with fragmented carrier operation like CA_n7(2F) where the number indicates the number of fragments, and the “F” indicates fragmented carrier operation. By indicating the support of fragmented carrier operation directly in the band combination (i.e. CA configurations), the new type of band combinations would have to be introduced in a similar manner as now done for NC IB CA where each case is assessed individually for needed potential relaxations. Additionally, it would be possible to list all supported band combinations in the specification and a UE would be able to indicate only those which it supports to the network.

RAN4 can consider a new syntax for indicating support of fragmented carrier operation in a CA configuration.

With this it would be up for further discussion whether there is a need for other signalling introduced by RAN2 together with a new UE capability. RAN2 will not introduce the new capability before a WI phase for fragmented carriers but they may be needed to provide input on other signalling aspects during the SI phase.

RAN4 needs to further discuss whether to ask RAN2 on input on signalling to support the feature at a later stage of the SI.

DL performance impact

The starting point for introducing a new CA configuration type, in this case fragmented carrier operation, should be that there should be no performance degradation. By that meant, that in the absence of in-gap interference, the performance should be similar to current CA configurations and when there is a strong in-gap interferer, the performance should not be worse than the performance with a single carrier.

There should be no performance degradation when introducing a new CA configuration type, in this case fragmented carrier operation.

If the in-gap interference does not have a larger PSD than any of the wanted CCs, the performance should be similar to the performance of legacy NC IB CA of UEs supporting it. If there is stronger in-gap interference, the performance of a UE supporting DL fragmented carriers should not be worse than the performance of a legacy UE using the widest of the fragmented carriers.

RAN4 shall not consider requirements for fragmented carriers which will result in performance degradation as compared to legacy NC IB CA or – only in case of a strong in-gap interferer – single CC operation.

This means that aspects such as throughput and spectrum utilization should be considered. In relation to this a discussion needed in RAN4 is how the minimum guardbands and related maximum transmission bandwidth configurations should be defined when operating in fragmented carrier mode. By this meant whether the total bandwidth, including the gap(s), or something different is used as the baseline.

RAN4 shall discuss what are the minimum guardbands and related maximum transmission bandwidths when operating in fragmented carrier mode.

This has an impact on available PRBs and hence the expected DL performance.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: Fragmented carriers are in some sense similar to non-contiguous intra-band carrier aggregation (NC IB CA) which is already supported by the specification.

Proposal 1: New requirements for fragmented carriers shall be separated from the existing requirements.

Observation 2: Current specification already has a requirement to the UE that it shall fulfil the requirements with an in-gap interferer of up to -25dBm.

Observation 3: Testing of any new requirements, e.g. in-gap interference level, for fragmented carriers may need to be considered within this SI.

Observation 4: A main challenge of non-contiguous intra-band carrier aggregation with a single Rx RF chain is the additional sources of interference since the single Rx RF chain has to cover multiple component carriers (CCs) with a potential in-gap interferer.

Proposal 2: RAN4 shall discuss whether UE requirements for fragmented carrier operation can be applicable to both equal and unequal bandwidth of the fragment’s carrier bandwidths.

Proposal 3: At least REFSENS and ACS requirements shall be discussed in relation to fragmented carriers in DL.

Observation 5: REFSENS relaxation for fragmented carrier operation can be related to the widening of the total transmission bandwidth.

Proposal 4: RAN4 shall encourage UE vendors to provide information on expected sensitivity degradation related to widening of the transmission bandwidth for fragmented carrier operation.

Observation 6: The current ACS requirements for non-contiguous intra-band carrier aggregation assume the presence of analogue filters which are not available for fragmented carrier operation.

Proposal 5: RAN4 shall seek input from UE vendors on the expected difference in interference attenuation between current implementation and fragmented carrier operation.

Observation 7: The SID implies  co-location of the non-contiguous intra-band CA's CCs and hence, a DL received PSD imbalance of 6dB between the wanted carriers within the fragmented carriers is a reasonable limit.

Proposal 6: RAN4 shall not re-discuss the power imbalance assumption of the wanted carriers further in relation to the SI on fragmented carriers.

Proposal 7: RAN4 shall consider a maximum level of in-gap interference fragmented carrier operation shall be able to tolerate.

Observation 8: RAN4 should consider if the power level of the in-gap interferer shall be used for enabling or disabling fragmented carrier operation.

Proposal 8: RAN4 shall discuss expected UE fallback behaviour when an in-gap interferer precludes the UE to operate in fragmented carrier mode.

Observation 9: RAN4 specification already have an agreed syntax for indicating non-contiguous parts of a band combination.

Proposal 9: RAN4 can consider a new syntax for indicating support of fragmented carrier operation in a CA configuration.

Observation 10: RAN4 needs to further discuss whether to ask RAN2 on input on signalling to support the feature at a later stage of the SI.

Observation 11: There should be no performance degradation when introducing a new CA configuration type, in this case fragmented carrier operation.

Proposal 10: RAN4 shall not consider requirements for fragmented carriers which will result in performance degradation as compared to legacy NC IB CA or – only in case of a strong in-gap interferer – single CC operation.

Proposal 11: RAN4 shall discuss what are the minimum guardbands and related maximum transmission bandwidths when operating in fragmented carrier mode.

- References

- RP-242404, Revised SID: Study on NR FR1 DL Fragmented Carriers, MediaTek Inc.

- RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

- RP-241269, On Fragmented Carriers in Rel-19, Apple

- RP-241361, Recommended WF on Fragmented Carriers, MediaTek

- R4-2414457, WF on fragmented DL carriers study, MediaTek

- R4-2412278, Operator’s initial views on FR1 fragmented carriers study, CHTTL