3GPP TSG-RAN WG4 Meeting # 112bis								    R4-2415436

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item: 6.9.3

Source: MediaTek Inc.

Title: Discussions on system parameters of Ku Band for NR NTN

Document for: Discussion

# 1	Introduction

In RP#104, a new WI [1] for Ku band was approved with objectives asking RAN4 to discuss the system parameters, including FR1/FR2, SCS and channel BW. The relevant objectives are captured as below.

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths.

In RAN4#112 meeting, this issue was discussed with the following agreements reached in WF [2].

- Issue 3-1: Frequency range definition - Following the scope of the WID, definition of a new frequency range e.g. FR3, is not in scope. - Issue 3-2: Numerology - Decision criteria - The following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology - UL timing synchronization - Phase noise - Efficient channel utilization - Beam hopping granularity - Link budget UL/DL – as a function of SCS and channel bandwidth - Issue 3-3: Numerology – timing aspects - The impact of SCS on timing is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues. - Issue 3-4: Numerology – phase noise - The impact of SCS on susceptibility to phase noise is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues. - Issue 3-5: Numerology – efficient channel utilization - The ability to efficiently support in future the required aggregate bandwidths [list] of NTN services for different SCS choices based on existing FR1 and FR2 channel bandwidths needs to be fully studied. - Issue 3-6: Numerology– beam hopping granularity - An analysis of the impact of different SCS on performance, although not critical, can influence the decision on FR1 vs FR2 numerology.

In this paper, we provide our view from the 2 aspects: 1) UL timing accuracy requirements and 2) link budget.

# 2	UL timing accuracy requirements

The UL timing budget is calculated as the Table 1 below with the detail explanation for individual values as follows:

- Te for TN is UL Tx timing accuracy requirement for TN UE. Values can be found in Table 7.1.2-1 of TS 38.133. It includes the allowed error in UE baseband (depending on the SSB BW) and RF (according to UE implementation). We skip the details of how to derive baseband and RF error margins here since this is not relevant to the later discussions.

- TA adjustment accuracy error is the allowed error at UE side when UE adjusts its UL timing based on the NW TAC (timing adjustment command). Values can be found in Table 7.3.2.2-1 of TS 38.133.

- Error from TAC resolution is the step size of network TAC. Values can be find in clause 4.2 of TS38.213. Note that the error equals half of the step size.

- GNSS/SAT error for NTN is the additional margin introduced for NTN, compared to TN requirements. It is to accommodate the additional error including satellite position, ephemeris inaccuracy and UE GNSS error. The value can be derived from the difference between Te in TN requirements (in Table 7.1.2-1 of TS 38.133) and Te_NTN in NTN requirements (in Tables 7.1C.2-1 and 7.1C.2-2 of TS 38.133). Since in this WI we focus on VSAT type of UE, we reused the agreed values of 120KHz for Ka band, i.e.,

    - With 120KHz: Te_NTN - Te = 7.5 Ts – 3.5Ts = 4Ts

    - Note that 4Ts is already a very tight requirements in our understanding. It means the aggregated error of satellite positioning, ephemeris inaccuracy and UE positioning should be less than 40 meter.

- Total error of single UE is the sum of previous 4 error contributors.

- NW margin is the difference between 0.5CP and the total error of single UE. This is the margin provided to the implementation of NW UL reception to accommodate timing drift, channel delay spread, … etc. The reason why we use 0.5CP is to address the case when 2 different UEs are transmitting simultaneous but one may have a positive error and one may have a negative error. In this case, one UE will share only 0.5CP margin equivalently.

Table 1. UL timing budget calculation for different SCS (unit: Ts)

| SCS of SSB | Te for TN | TA adjustment accuracy error | Error from TAC resolution | GNSS/SAT error for NTN | Total error of single UE | 0.5CP | NW margin |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (A) | (B) | (C) | (D) | (E=A+B+C+D) | (F) | (G=F-E) |
| 15 kHz | 12 | 4 | 8 | 4 | 28 | 72 | 44 |
| 30 kHz | 8 | 4 | 4 | 4 | 20 | 36 | 16 |
| 120 kHz | 3.5 | 0.5 | 1 | 4 | 9 | 9 | 0 |

From above calculations, we can see that there is already zero margin left in 120KHz. Moreover, if there is a strong demand to consider mobile VSAT in the later works, we also need to further allow larger UE GNSS error, leading to negative margin for NW implementation. From this perspective, 120KHz seems not forward compatible. Based on above analysis, we suggest adapting FR1 numerology for Ku band.

Observation 1: With 120KHz SCS, there will be very little margin left for network reception to accommodate timing drift or channel delay spread, while there are still roughly half CP margins left with SCS 15KHz and 30KHz.

Proposal 1: According to the UL timing analysis, it is suggested to use FR1 SCS (15KHz and 30KHz) for Ku band to leave sufficient margin to network reception.

# 3	Link budget analysis

The link budget is calculated as the Table 2 below. We focus on DL side which seems already sufficient to explain the idea. We presented 4 scenarios, depending on whether we need to follow the regulation of power flux density (PFD, mainly in US) and the 2 different channel BWs (50MHz and 100MHz).

- Scenario A: 50MHz CBW without PFD limitation

- Scenario B: 10MHz CBW without PFD limitation

- Scenario C: 50MHz CBW with PFD limitation

- Scenario D: 10MHz CBW with PFD limitation

The whole calculation follows the same framework as clause 6.1.3 in TR 38.821. The main equation is captured below for quick reference.

CNRdB=EIRPdBW+GTdB/K-k[dBW/K/Hz]-PLFSdB-PLAdB-PLSMdB-PLSLdB-PLADdB-BdBHz

From the equation it is already obvious that larger channel BW (CBW) lead to lower CNR. Therefore, to maintain the same CNR, UE may need to be installed with more antennas. Nevertheless, we still provide some detail calculations of individual terms as below:

- Effective isotropic radiated power (EIRP): Whether to consider PFD impact the value of EIRP.

  - Without PFD, the EIRP value is up to the satellite implementation. Here, we took the typical values in RAN1 Rel-19 discussion (Maximum EIRP per Satellite beam in Tables for Set1-1 and Set1-2 in the Appendix II of [3].

  - With PFD, ITU regulation [4] for Ku band is -138dBW/m2 per 4KHz (equals to -114dBW/m2 per 1MHz). Adding a bit margin, we assume -115dBW/m2 per 1MHz in the calculation. With the PFD, we can derive the EIRP value at the satellite transmitter, i.e., 35dBW for 50MHz CBW and 38dBW for 100MHz CBW

- Antenna-gain-to-noise-temperature (G/T): It can be calculated based on equation (6.1.3.1-3) in TR 38.821. Here, we assume 256 antennas for Scenario A, 512 antennas for Scenario B and 1024 antennas for Scenarios C/D. The main intension to maintain the same final CNR. These values are for examples and have nothing to do with the requirements. Other required parameters are also listed in Table 2 for reference.

- Noise figure (NF): It is part of the calculation in G/T. Here we follow the values in Table 4.4-1 in TR 38.811. It is likely to be a different value in the later requirement discussion.

- Boltzmann constant (k): Same value is assumed as that in TR38.821.

- Free space path loss (PLFS): It is calculated based on Equation (6.6-2) in TR 38.811, where d is the distance between satellite and UE, and fc is the carrier frequency.

- Other pathloss (PLA,PLSM,PLSL,PLAD): We assume zero in the calculation. Note that there are some non-zero values in [5], but the values are typically not big.

Table 1. UL timing budget calculation for different scenarios

| Scenario | unit | without PFD |  | with PFD |  |
| --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D |
| Carrier frequency | GHz | 12 | 12 | 12 | 12 |
| distance (d) | km | 1200 | 1200 | 1200 | 1200 |
| BW (B) | MHz | 50 | 100 | 50 | 100 |
| EIPR | dBW | 41 | 41 | 35 | 38 |
|  | dBW/MHz |  |  | 18 | 18 |
| PFD limit | dB(W/(m^2)/MHz) |  |  | -115 | -115 |
| G/T | dB/K | -10 | -7 | -4 | -4 |
| Rx ANT gain | dB | 24 | 27 | 30 | 30 |
| #Rx Array Elements (Nr) | # | 256 | 512 | 1024 | 1024 |
| Rx element gain | dB | 3 | 3 | 3 | 3 |
| Pol loss | dB | 3 | 3 | 3 | 3 |
| NF | dB | 9 | 9 | 9 | 9 |
| ambient temperature (T0) | K | 290 | 290 | 290 | 290 |
| antenna temperature (Ta) | K | 290 | 290 | 290 | 290 |
| Boltzmann constant (k) | dBW/K/Hz | -228.6 | -228.6 | -228.6 | -228.6 |
| PLFS | dB | 176 | 176 | 176 | 176 |
| PLA | dB | 0 | 0 | 0 | 0 |
| PLSM | dB | 0 | 0 | 0 | 0 |
| PLSL | dB | 0 | 0 | 0 | 0 |
| PLAD | dB | 0 | 0 | 0 | 0 |
| CNR | dB | 7 | 7 | 7 | 7 |

From above calculations, we can observe:

  - Without PFD, to maintain the same CNR at the UE, more RX antennas are needed.

  - With PFD which further limits the EIRP, we need even more Rx antennas. Since PFD limitation is per MHz, further increase the CBW does not require more antenna at UE side, as long as the EIRP is not larger than 41dBW.

  - If the target CNR is higher than 7dB, more UE antennas are needed.

Considering 12GHz frequency for Ku band, the half wavelength is 1.25 centimeters (cm). Antenna number 256 and 1024 roughly mean 20x20 cm2 and 40x40 cm2 antenna arrays, respectively. These sizes may be challenging for some devices. Therefore, we suggest RAN4 to consider whether all channel BWs in Ku band are mandatory, i.e., some larger channel BW is optional. So that not all UE are mandated to implement large antenna array to accommodate the largest channel BW. This could help to lower to cost of UE and speed up to time-to-market Ku band deployment.

Observation 2: With larger channel BW, the CNR of the link will decrease correspondingly. This requires UE to implement more antennas to keep the same link CNR.

Observation 3: Without power flux density (PFD), to maintain the same CNR at the UE, more RX antennas are needed. With PFD, UE’s antenna number are expected to be more, compared to the case without PFD.

Proposal 2: RAN4 to consider whether some larger channel BW can be optional for Ku band.

# 4	Summary

In this paper, we provide our views on the system parameters for Ku band. We have the following observations and proposals.

Observation 1: With 120KHz SCS, there will be very little margin left for network reception to accommodate timing drift or channel delay spread, while there are still roughly half CP margins left with SCS 15KHz and 30KHz.

Proposal 1: According to the UL timing analysis, it is suggested to use FR1 SCS (15KHz and 30KHz) for Ku band to leave sufficient margin to network reception.

Observation 2: With larger channel BW, the CNR of the link will decrease correspondingly. This requires UE to implement more antennas to keep the same link CNR.

Observation 3: Without power flux density (PFD), to maintain the same CNR at the UE, more RX antennas are needed. With PFD, UE’s antenna number are expected to be more, compared to the case without PFD.

Proposal 2: RAN4 to consider whether some larger channel BW can be optional for Ku band.

# 5	References

RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Eutelsat

R1-2406435, FL Summary #1: NR-NTN downlink coverage enhancements, THALES

https://www.itu.int/dms_pubrec/itu-r/rec/sf/R-REC-SF.358-5-199510-W!!PDF-E.pdf

R1-1913351, Link budget results for NTN, Thales