3GPP TSG-RAN WG4 Meeting #112bis	R4-2415443

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	KDDI

Title: 	Discussion on UE Capability and BS Signaling for non-collocated Type 4a4b

Agenda Item:	6.5.2.2

Document for:	Discussion

- 1. Introduction

In RAN#105 meeting, the revised WID for intra-band non-collocated EN-DC/NR-CA is approved [1]. The WI’s core part is focusing on specifying UE RF/RRM requirements for Type-4 UE capability for FWA and clarifying the UE behaviour for Type-4 supported UE. Some discussions were organized in the last RAN4#112 meeting and the agreements were captured in [2]. We try to solve remaining issues in this contribution.

- 2. Discussion

- 2.1 Working Assumption

According to WID [1], RAN4 need to specify Type-4a/4b capability for FWA. Based on the working assumption previously agreed as WF in RAN4#106 [4], in RAN#103 we propose the following working assumption in Table-1 same as [3].

Table-1. Working assumption for UE architecture

Observation 1: The working assumption of RP-240101 can be reused in RAN4.

- 2.2 Reference UE Architecture

In RAN4#104-bis, UE reference architecture for Type 4b was proposed as Rel-18 in Figure-1 [5], and also it can be reused in Rel-19.

Figure-1. Reference UE architecture

- 2.3 New UE Capability for Type 4a/4b

Regarding UE capability, RAN4 conclude the WF[2] in the last meeting as follows. In the last meeting, only one company mentioned that Type 4a can be for both EN-DC and NR-CA, therefore RAN4 couldn’t agree the following Issue 2-3-1.

< Issue 2-3-1: New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA > Tentative agreement: - Define one capability for NR CA and one UE capability for EN-DC - FFS on how to differentiate type 4a and 4b.

In the last meeting, RAN4 discussed whether there is no need to distinguish Type 4a and Type 4b, indication of Type 4b would be enough and Type 4a can be discarded for EN-DC. On the other hand, Type 4a and 4b have already been agreed in Rel-18 for EN-DC[4], furthermore 4Rx is an option for band 42 different from n77/n78 and this also means that there is a difference of RF requirement for band 42 between 2Rx and 4Rx as shown in the following table. Therefore, we prefer to keep Type 4a for EN-DC not to exclude a possibility of UE implementation.

TS36.101 v16.20.0 Table 7.3.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |

TS36.101 v16.20.0 Table 7.3.1-1a: ΔRIB,4R

| E-UTRA Band | ΔRIB,4R [dB] |
| --- | --- |
| 1, 2, 3, 4, 7, 20, 21, 25, 30, 34, 39, 40, 41, 66 | - 2.7 |
| 42, 43, 52 | - 2.2 |

Additionally, the WID[1] is clearly stated as follows.

Note: UE types are defined in R4-2217734 (WF on NonCol_intraB_ENDC_NR_CA for New Type UE).

Our understanding is that UE types is intended for UE capability types. R4-2217734 is clearly stated that Type 4a is for EN-DC only, hence RAN4 should align to WID and doesn’t need to discuss reverting the RAN level agreement now.

Proposal 1:

- Define one capability for NR CA and one UE capability for EN-DC

  - Differentiate Type 4a and 4b by UE Capability for EN-DC

  - Only Type 4b for NR-CA, not differentiate Type 4a and 4b for NR-CA

- Specific UE Capability’s design up to RAN2

- 2.4 New BS signaling for Type 4a/4b

In Rel-18, RAN2 designed BS signaling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC in a case of maxMIMO-Layers with value less than or equal to 2. The following description on Rel-18 BS signaling and UE behavior between Type 1 and Type 2 for NR-CA is excerpted from TS 38.331 v18.2.0 section 6.3.2.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

The above Rel-18 BS signaling nonCollocatedTypeNR-CA (nonCollocatedTypeMRDC for EN-DC) to switch between Type 1 and Type 2 is not mentioned for a case with maxMIMO-layers equal to 4, but also in our understanding the case of maxMIMO-layers equal to 4 is collocated scenario Type 1.

Observation 1: Rel-18 BS signaling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC to switch between Type 1 and Type 2 is specified for the case with maxMIMO-layers less than or equal to 2.

Observation 2: In Rel-18, the case of maxMIMO-layers larger than 2 is collocated Type1.

Based on the above Rel-18 BS signaling, we can follow a same direction on Rel-19 BS signaling to switch between Type 1 and Type 4a/4b. So specifically, Rel-19 BS signaling to switch between Type 1 and Type 4a/4b is needed to specify for the case with maxMIMO-layers larger than 2 and less than or equal to 4. Additionally, the case of maxMIMO-layers larger than 4 is collocated scenario Type1.

Proposal 2: Specify Rel-19 BS signaling to switch between Type 1 and Type 4a/4b for the case with maxMIMO-layers larger than 2 and less than or equal to 4. Additionally, the case of maxMIMO-layers larger than 4 is collocated Type 1.

- 2.5 UE behavior/transition between single CC, Type 1, Type 2 and Type 4

Based on the previous observation 1, 2 and the proposal 2, in a case of NR-CA, we can show UE behaviors/transitions between single CC operation, Type 1, Type 2 and Type 4 (4b) with using Rel-19 new BS signaling and maxMIMO-layers as shown in the following table. In the previous RAN4 meetings, companies tried to solve this issue from aspects of each specific values of new/legacy BS signaling, UE capabilities and related RF requirements. On the other hand, in this contribution, firstly we try to promptly show overall from aspects of UE behaviors/transitions between each UE types as follows.

Table 1: UE behavior between single CC, Type 1, Type 2 and Type 4

| UE behavior/transition From→To |  | To |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ①Type 4_Non-colloc_2cc_4L/cc | ②Type 2_Non-colloc_2cc_2L/cc | ③Type 1_Colloc_2cc_4L/cc | ④Type 1_Colloc_2cc_2L/cc | ⑤Single 1cc_4L/cc |
| From | ①Type 4_Non-colloc_2cc_4L/cc | N/A | maxNumberMIMO-Layers=4→2 | Rel-19 new BS signaling=1 | Rel-19 new BS signaling=1 +maxNumberMIMO-Layers=4→2 | RRC Release Scell |
|  | ②Type 2_Non-colloc_2cc_2L/cc | maxNumberMIMO-Layers=2→4 | N/A | Rel-18 NonCollocatedTypeNR-CA +maxNumberMIMO-Layers=2→4 | Rel-18 NonCollocatedTypeNR-CA | RRC Release Scell +maxNumberMIMO-Layers=2→4 |
|  | ③Type 1_Colloc_2cc_4L/cc | Rel-19 new BS signaling=0 | Rel-18 NonCollocatedTypeNR-CA +maxNumberMIMO-Layers=4→2 | N/A | maxNumberMIMO-Layers=4→2 | RRC Release Scell |
|  | ④Type 1_Colloc_2cc_2L/cc | Rel-19 new BS signaling=0 +maxNumberMIMO-Layers=2→4 | Rel-18 NonCollocatedTypeNR-CA | maxNumberMIMO-Layers=2→4 | N/A | RRC Release Scell +maxNumberMIMO-Layers=2→4 |
|  | ⑤Single 1cc_4L/cc | RRC Add Scell=0 +Rel-19 new BS signaling | RRC Add Scell +Rel-18 NonCollocatedTypeNR-CA +maxNumberMIMO-Layers=4→2 | RRC Add Scell | RRC Add Scell +maxNumberMIMO-Layers=4→2 | N/A |

We try to describe details on the above table. First, regarding behaviors between Type 1(collocated) and Type 2(non-collocated), it has been already specified with using nonCollocatedTypeNR-CA and maxMIMO-layers in Rel-18.

Next, regarding behaviors between Type 2 and Type 4 (both non-collocated), we agreed in the last RAN4 meeting as follows.

Issue 2-3-3: New BS Signaling to switch between Type 4a/4b and Type 2 Agreement: - No new NW signaling needed from Type-4 to Type-2 switch and maxMIMO-layer can be used.

And then, regarding behaviors between Type 1(collocated) and Type 4(non-collocated), based on the previous proposal 2, this transition needs Rel-19 new BS signaling with/without maxMIMO-layers.

Finally, regarding behaviors between single CC and Type 4(non-collocated), new BS signaling is not needed, because Scell Add/Release via RRC can be reused same as a legacy operation. Actually, RAN4 conclude the WF[2] in the last meeting as follows.

Issue 2-3-6: Switch between Type 4a/4b and single CC operation Way Forward: - No new BS signaling needed from Type-4 to single carrier fallback.

We think that RAN4 doesn’t need to decide only most valuable UE behavior/transition path and it is enough for RAN4 to make all paths possible and under NW control. Regarding values 0/1 of Rel-19 new BS signaling in the above table, we describe details in the next section 2.6.

Proposal 3: No new BS signaling needed from Type-4 to single carrier fallback.

- 2.6 UE behavior on initial access

Based on the previous observation 1, 2, UE behavior on initial access for Rel-18 is as follows.

- The case of maxMIMO-layers larger than 2 is Type 1.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

We also think that same direction might be adopted for Rel-19 as follows.

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present is Type 1.

  - Rel-19 New BS signaling is absent is Type 4.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

On the other hand, we need to consider the following two cases raised during offline discussion after the last RAN4 meeting.

- Type 4 capable Rel-19 UE behavior operate under Rel-18 BS

- Only Type 2 capable Rel-19 UE behavior under Rel-19 BS

Especially, regarding above case A, assuming Rel-19 UE indicates both Type 4 and Type 2 capability, then Rel-18 BS only understand Type 2 and use maxMIMO-layers=4 to control UE work under collocated as Type 1 if following above Rel-18 based new BS signaling design, but Rel-19 UE would see "new Rel-19 BS signaling=absent + maxMIMO-layers=4", then this UE misthink Type4. Eventually, discrepancy between Rel-19 UE and Rel-18 BS happens.

So, we propose new BS signaling to control “present with having value 0/1” and “absent” adding to only “present/absent” of Rel-18 BS signaling as follows, so that new BS signaling can solve the discrepancy as above mentioned.

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present with value 0 is Type 4.

  - Rel-19 New BS signaling is present with value 1 is Type 1.

  - Rel-19 New BS signaling is absent is that UE follows Rel-18 specification.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

Proposal 4: Confirm the following understanding on maxMIMO-layers and Rel-18/19 BS signaling:

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present with value 0 is Type 4.

  - Rel-19 New BS signaling is present with value 1 is Type 1.

  - Rel-19 New BS signaling is absent is that UE follows Rel-18 specification.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

- 3. Conclusion

Observation 1: Rel-18 BS signaling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC to switch between Type 1 and Type 2 is specified for the case with maxMIMO-layers less than or equal to 2.

Observation 2: In Rel-18, the case of maxMIMO-layers larger than 2 is collocated Type1.

Proposal 1: Specify capabilities both Type 4a for EN-DC and Type 4b for EN-DC and NR-CA.

Proposal 2: Specify Rel-19 BS signaling to switch between Type 1 and Type 4a/4b for the case with maxMIMO-layers larger than 2 and less than or equal to 4. Additionally, the case of maxMIMO-layers larger than 4 is collocated Type 1.
Proposal 3: No new BS signaling needed from Type-4 to single carrier fallback.
Proposal 4: Confirm the following understanding on maxMIMO-layers and Rel-18/19 BS signaling:

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present with value 0 is Type 4.

  - Rel-19 New BS signaling is present with value 1 is Type 1.

  - Rel-19 New BS signaling is absent is that UE follows Rel-18 specification.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

- 4. Reference

[1] RP-230838 Revised WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), KDDI, RAN#105

[2] R4-2414453 WF on Rel-19 non-collocated scenario, KDDI, RAN4#112

[3] RP-240101 Motivation on Non-collocated ENDC,NRCA in Release 19, KDDI, Docomo, KT, LG Uplus, SoftBank, Telecom Italia, Samsung, RAN#103

[4] R4-2303696 WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN4#106

[5] R4-2215736, Views on UE RF aspect for non-collocated EN-DC, NR-CA deployment, Samsung, RAN4#104-bis-e

[6] R4-2306605 WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN4#106-bis-e