3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415445

Hefei, China, 14th – 18th Oct. 2024

Source: 	Nokia, Nokia Shanghai Bell

Title: 					n68 NS_26 and NS_36 A-MPR simulations

Agenda Item: 			5.16.1

Document for:	Information

# 1	Introduction

We simulate A-MPR for band n68 due to additional spurious emission masks signaled through NS_26 and NS_36.

# 2	Simulation assumptions and scenario

## 2.1	Simulation parameters

- Operating band n68 (UL 698 MHz – 728 MHz)

- Channel bandwidths [1]: 5 MHz, 10 MHz and 15 MHz

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional spurious emission masks from NS_26 and NS_36 are simulated. Other gating factors (ACLR, SEM, etc.) are excluded.

## 2.2	NS_26 mask

We assume the mask from LTE is also valid for NR.

Table 6.6.3.3.22-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |
| 686 ≤ f ≤ 694 | -25 | 8MHz |

## 2.3	NS_36 mask

We assume the mask from LTE is also valid for NR.

Table 6.6.3.3.28-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz and 15 MHz |  |
| 470 ≤ f ≤ 694 | -42 | 8MHz |
| NOTE: For a 5 MHz E-UTRA carrier confined within 698MHz and 703MHz, this requirement shall be met in normal conditions only. The requirement is relaxed to -30dBm in extreme conditions. |  |  |

# 3	Simulation results and discussion

In this section we provide examples of the simulation results. In appendix we provide complete sets of A-MPR triangles at chosen center frequencies.

## 3.1	NS_26

For NS_26 A-MPR is needed for all CBWs at the lower edge of the operating band. In Table 1 we present the center frequencies where A-MPR is not needed. In Figure 1 we show examples of the A-MPR triangles for 15 MHz CBW at the lower edge of the operating band.

Table 1: The center frequencies where A-MPR for NS_26 is not needed according to simulations.

| Channel Bandwidth [MHz] | Fc, noAMPR [MHz] |
| --- | --- |
| 5 | ≥ 701.5 |
| 10 | ≥ 708.6 |
| 15 | ≥ 715.1 |

Figure 1: Examples of the simulated backoff to satisfy the NS_26 additional spurious emission limit for 15 MHz CBW at the lower edge of the operating band (Fc=705.5 MHz).

Observation 1: For NS_26 A-MPR is needed at some center frequencies but not all of them.

## 3.2	NS_36

For NS_36 A-MPR is needed for all CBWs at the lower edge of the operating band. In Table 2 we present the center frequencies where A-MPR is not needed. With 15 MHz CBW A-MPR cannot be avoided anywhere in the operating band. Figure 2 shows examples of the A-MPR triangles at the lower edge of the operating band and Figure 3 at the upper edge.

Table 2: The center frequencies where A-MPR for NS_36 is not needed according to simulations. With 15 MHz CBW A-MPR cannot be avoided anywhere in the operating band.

| Channel Bandwidth [MHz] | Fc, noAMPR [MHz] |
| --- | --- |
| 5 | ≥ 708.0 |
| 10 | ≥ 721.8 |

Figure 2: Examples of the simulated backoff to satisfy the NS_36 additional spurious emission limit for 5 MHz and 15 MHz CBWs at the lower edge of the operating band (Fc=700.5 MHz and Fc=705.5 MHz).

The large A-MPR values at the lower right edges of the triangles are due to CIM3.

Figure 3: Examples of the simulated backoff to satisfy the NS_36 additional spurious emission limit for 15 MHz CBW at the upper edge of the operating band (Fc=720.5 MHz).

Observation 2: For NS_36 and CBWs of 5 MHz and 10 MHz A-MPR is needed at some center frequencies but not all of them.

Observation 3: For NS_36 and 15 MHz CBW A-MPR is needed for all center frequencies in the operating band.

# 4	Conclusions

We have performed A-MPR simulations for band n68 taking into account network signaling values NS_26 and NS_36. We have made following observations based on our simulation results.

Observation 1: For NS_26 A-MPR is needed at some center frequencies but not all of them.

Observation 2: For NS_36 and CBWs of 5 MHz and 10 MHz A-MPR is needed at some center frequencies but not all of them.

Observation 3: For NS_36 and 15 MHz CBW A-MPR is needed for all center frequencies in the operating band.

# 5	References

[1] R4-2414273, WF on introduction of NR band n68, Ericsson

# 6	Appendix

We show the complete sets of A-MPR triangles at the lower edge of the operating band and also at the upper edge in case it is needed.

## 6.1	NS_26

## 6.2	NS_36