3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415449

Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			A-IoT general overview

Agenda item:		6.21.1

Document for:		Approval

# Introduction

In this paper, we present our thoughts on general issues for A-IoT SI and the TP for system parameters.

# Discussion

The coexisting study and RF requirement study are ongoing task in RAN4. Additionally, RAN4 will discuss the CW cancellation capability.  However, there is one missing aspect which has not discussed in RAN4 which is the link budget. Though it is a RAN1 topic but the conclusion of it impacts the RF requirement discussion. For example, in our companion paper, the link budget is discussed based on most of the RAN1 assumed parameters. For purpose of the discussion, Figure 1 and Figure 2 in [1] are reproduced in this paper. It can be observed that when considering the device is harvesting the energy from RF using CW signal and the WID objective of peak power consumption of 1 µW, the minimum received power for A-IoT BS or UE as intermediate node is decided by R2D distance and therefore the legacy way of the analysis the A-IoT BS REFSENS is not necessary any more. For example, in Figure 1, when CW transmission power is 23 dBm, the device 1 with EH sensitivity of -24 dBm will set the min received power for A-IoT BS or UE as reader with -74 dBm and this is 33 dB relaxation if A-IoT BS REFSENS would be set in a traditional way. In Figure 2, the hardcoded device 1 and 2a sensitivity is marked and it also shows the received power for A-IoT BS or UE as reader can be relaxed.

The REFSENS of A-IoT BS or UE as intermediate node is limited by R2D link not D2R link for backscattered signal transmission.

Current coexisting are assuming both device 1 and 2a in the same network, so the cell range will decide by the device 1 only. This also means that with such coexisting assumption, the A-IoT BS REFSENS can be relaxed as this received power will be decided by R2D link not D2R link.

For a network with mixing the device 1 and 2a, the cell range is decided by device 1.

Additionally, the coexisting scope in RAN4 scope is limited as the intra A-IoT system with two adjacent channel is down-prioritized. Therefore, it is remaining unknow factor that whether the reader to device distance will be further limited when two different A-IoT channel would be deployed in the same area. From the current prioritized D1T1-A2, the RAN4 coexisting assumption of the A-IoT inter-BS distance is modified due to the A-IoT intra-system interference and the simultaneous operating A-IoT BS is reduced from 18 to 2.

The coexisting scope may set further limitation on the reader to device range.

From the above discussion, it can be observed that there are several factors impacting the REFSENS of A-IoT or UE as reader, these are D2R range, energy harvesting used in device and coexisting scenario. As EH is out of scope of the SI and link budget is RAN1 scope and even coexisting scope is limited in RAN4, we suggest that the factors to be considered for the REFSENS specifying could be discussed in RAN4 but leaving the decision of how to specify it in work item phase.

RAN4 discuss the impacting factors on the REFSENS of the A-IoT BS or UE as reader in study phase.

Figure 1:device 1 range considering the energy harvesting with CW transmission power of 23 dBm

Figure 2: device 1/2a range with hardcoded sensitivity and CW transmission power of 23 dBm

## system parameter

For operating band, when the same spectrum would be used for D2R and R2D signals, a TDD band should be defined and when the different spectrum is used for D2R and R2D, a FDD band should be defined according to 3GPP definition. However, as the reader can also transmit the CW signal while receiving the D2R signal, it should be discussed what is the duplex mode when transmitter and receiver are simultaneously operated. It can be argued that there is still some duplex distance between CW and D2R signal.  As the CW transmission takes place at a different frequency with the D2R signal , the duplex distance may be defined as tens of kHz between D2R and CW signal, or maybe depend on the device frequency shift capabilities. However, as the frequency distance between CW and D2R is too narrow, it will be hard to define a duplex distance in traditional ways where the duplex filter can be used to isolate the Tx and Rx signal. On top of this, the CW is transmitted within a channel bandwidth where CW RF characteristic will be verified. With this understanding, the frequency of the CW channel and D2R channel may be the same or overlap and CW transmission and D2R receiving occur at the same frequency. TDD and FDD related definition in NR definition is below in TS 38.211:

OFDM symbols in a slot in a downlink or uplink frame can be classified as 'downlink', 'flexible', or 'uplink'. Signaling of slot formats is described in clause 11.1 of [5, TS 38.213].

In a slot in a downlink frame, the UE shall assume that downlink transmissions only occur in 'downlink' or 'flexible' symbols.

In a slot in an uplink frame, the UE shall only transmit in 'uplink' or 'flexible' symbols

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by Table 4.3.2-3.

A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same cell where NTx-Rx is given by Table 4.3.2-3.

Furthermore, in 25.201, the FDD (Frequency Division Duplex) and TDD (Time Division Duplex) is defined as below

FDD:	A duplex method whereby uplink and downlink transmissions use two separated radio frequencies. In the FDD, each uplink and downlink uses the different frequency band. A pair of frequency bands which have specified separation shall be assigned for the system.

TDD:	A duplex method whereby uplink and downlink transmissions are carried over same radio frequency by using synchronised time intervals. In the TDD, time slots in a physical channel are divided into transmission and reception part. Information on uplink and downlink are transmitted reciprocally.

From the definition above, it can be observed that NR definition of the TDD and FDD relating to full-duplex communication and for definition of the full duplex, it can be referred to 25.201. For FDD and TDD, it can be observed that when CW transmission and D2R receiving at the same frequency, it seems that such operation does not qualify the FDD; similarly, as CW transmission and D2R receiving also at the same time, so it seems that such operation does not qualify the TDD. It can also be observed that the duplex definition is about “uplink or downlink transmission” in 25.201 and in uplink transmission or downlink transmission in TS 38.211 also for uplink or downlink symbols. Our understanding is that the CW signal is not designed as physical channel for control and data, therefore, there is no CW symbols defined in RAN1. So CW transmission is not in the scope of the FDD and TDD definition in NR specification. CW signal is not designed as downlink symbol or uplink symbol.

CW signal is not compatible with FDD and TDD definition.

Define either TDD or FDD bands depends on the whether same or different spectrum is used for A-IoT operation.

For channel bandwidth, RAN1 has discussed R2D and D2R bandwidth separately. The R2D related agreement is

RAN1#116:

Agreement At least the following bandwidths for R2D are defined for the purpose of the study: - Transmission bandwidth, Btx,R2D from a Reader perspective: The frequency resources used for transmitting R2D - Occupied bandwidth, Bocc,R2D from a Reader perspective: The frequency resources used for transmitting R2D, and potential guard band - Bocc,R2D ≥ Btx,R2D - FFS: Further constraint(s) e.g. Bocc,R2D = Btx,R2D. - Possible values of each bandwidth are FFS

RAN1#116bis:

Agreement For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D is ≤ [12] PRBs and is down-selected among: - Alt 1: Including 180 kHz, 360 kHz, and FFS other values - Alt 2: Integer multiple(s) of 180 kHz (FFS: what integer(s)) - Alt 3: Integer multiple(s) of the subcarrier spacing (FFS: what integer(s))

It can be observed that Btx,R2D is defined as frequency resource and less than 12 RB and this can be the same with the transmission bandwidth definition defined in Figure 5.3.1-1 in TS 38.101-1 and this is quoted as below. The R2D is OFDM-based waveform, therefore the channel bandwidth definition can follow the channel bandwidth definition in LTE as there is no corresponding maximum transmission bandwidth defined in NR. For 180kHz transmission bandwidth, the channel bandwidth can be defined as 200kHz following the NB-IoT channel bandwidth definition.

R2D channel bandwidth can follow the LTE channel definition.

200kHz can be defined as channel bandwidth for 180kHz transmission bandwidth.

Figure 5.3.1-1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one NR channel

For D2R, the relevant RAN1 agreements are below, it can be observed that a single device scheduled by a reader is defined and the guard band design refer to RAN4. RAN1 agrees that OOK and BPSK are feasible waveform for D2R for all device.

RAN1#116bis:

Agreement The following bandwidths for D2R are defined for the purpose of the study: - Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device. - FFS in agenda 9.4.2.3: how frequency resources scheduled by a reader are determined - Occupied bandwidth, Bocc,D2R: The transmission bandwidth plus the potential associated intra A-IoT guard-bands totalling Bguard,D2R - Note: this guard band is not for coexistence with NR/LTE - If/how to define guard band for coexistence between A-IoT D2R and NR/LTE is up to RAN4. - Bocc,D2R >= Btx,D2R - Possible values of each bandwidth are FFS

RAN1#118:

Agreement

- OOK and Binary PSK for baseband modulation are feasible for D2R for all devices.

- The variant of Binary FSK to study further for all devices is identified as MSK (and not GMSK)

  - FFS: whether MSK is feasible for all devices

To define the channel bandwidth, one criterion is that the occupied bandwidth RF requirement should be met. Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. This can be simulated and Figure 1 illustrates OOK signal in time domain and PSD in frequency domain. The data rate is set 20 kbps and OOK bit length is 96 bit. It can be observed from Table 2 that 100kHz channel bandwidth should be fine with these parameters. As RAN1 has not reach conclusion on the relevant parameters, RAN4 can wait RAN1 further progress on it.

Delay to study D2R chanel bandwidth and wait RAN1 progress.

Figure 3: OOK D2R signal in time and frequency domain

Table 1: Occupied channel bandwidth test case for 20kbps data rate

| Bandwidth (kHz) | Power Containment (%) |
| --- | --- |
| 60 kHz | 98.361301% of total power |
| 80 kHz | 98.771765% of total power |
| 100 kHz | 99.019734% of total power |
| 200 kHz | 99.522660% of total power |

TP start here:

### 6.7.1	System parameters

#### 6.7.1.1	Operating band

Table 6.7.1.1-1 summarized the A-IoT system operation cases according to coexisting scenario. It can be observed that spectrum usage for A-IoT operation depends on how the CW transmission is deployed, e.g inside or outside topology, and also depends on whether it is BS as reader or UE as a reader. For the case where same spectrum is used for R2D and D2R, e.g either DL spectrum or UL spectrum, as the same spectrum is used both for R2D and D2R, the TDD band could be defined. For the case where different spectrum is used for D2R and R2D, the FDD band definition can be kept. As CW transmission may interfere the R2D reception, the R2D and D2R should be TDM and therefore, for FDD band definition, it should be half duplex mode.

Table 6.7.1.1-1: Factors impacting the A-IoT BS operation

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | Comment |
| --- | --- | --- | --- | --- |
| A-IoT BS | DL spectrum | DL spectrum | DL spectrum | Coexisting scenario D1T1-A2 |
|  | DL spectrum | UL spectrum | UL spectrum | Coexisting scenario D1T1-B |
| UE as reader | UL spectrum | UL spectrum | UL spectrum | Coexisting scenario D2T2-A2 |
|  | UL spectrum | DL spectrum | DL spectrum | Coexisting scenario D2T2-B |

#### 6.7.1.2	Channel bandwidth

The R2D waveform is agreed to be OFDM-based waveform, therefore the channel bandwidth definition can follow the channel bandwidth definition in LTE as there is no corresponding maximum transmission bandwidth defined in NR. For 180kHz transmission bandwidth, the channel bandwidth can be defined as 200kHz following the NB-IoT channel bandwidth definition.

# Conclusions

In this contribution, we present our overview on the A-IoT general issues with below proposal:

Observation 1 The REFSENS of A-IoT BS or UE as intermediate node is limited by R2D link not D2R link for backscattered signal transmission.

Observation 2 For a network with mixing the device 1 and 2a, the cell range is decided by device 1.

Observation 3 The coexisting scope may set further limitation on the reader to device range.

Proposal-1: RAN4 discuss the impacting factors on the REFSENS of the A-IoT BS or UE as reader

0 CW signal is not designed as downlink symbol or uplink symbol.

Observation 4 CW signal is not compatible with FDD and TDD definition.

Proposal-2: Define either TDD or FDD bands depends on the whether same or different spectrum is used for A-IoT operation.

Observation 5 R2D channel bandwidth can follow the LTE channel definition.

Proposal-3: 200kHz can be defined as channel bandwidth for 180kHz transmission bandwidth.

Proposal-4: Delay to study D2R chanel bandwidth and wait RAN1 progress.

# References

- R4-24xyz, On CW cancellation capability, Ericsson