3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415453

Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			Coexisting study simulation assumptions and result for A-IoT

Agenda item:		6.21.2.2

Document for:		Approval

# Introduction

In this paper, we discuss additional simulation assumptions and also some simulation results for some of cases of coexisting for A-IoT.

# Discussion

The simulation cases are agreed in and for discussion purpose, it is quoted as below.

Issue 3-4-1: Evaluation cases for co-existence evaluation

| No. | deployment scenario and topology | spectrum | (Aggressor -> Victim) | note |
| --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 4 (O) | D1T1-B-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 6-1 (O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 6-2 (O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |

## Simulation methodology

In WF[1], the interference modelling is stated below:

Issue 4-1-1:  A-IOT reader and NR BS

Agreement:

|  | In-band |  |
| --- | --- | --- |
|  | Tx | Rx |
| NR UE/A-IOT Intermediate UE | Legacy UE IBE | In-band A-IOT signal interference to NR is not considered when calculating SINR. Companies to check the interference caused by non-orthogonal A-IOT signal |
| NR BS | Option 1: 30dBc Option 2: 17dBc Companies to check the leakage of legacy NR BS. | In-band A-IOT signal interference to NR is not considered when calculating SINR. Companies to check the interference caused by non-orthogonal A-IOT signal |
| A-IOT BS | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dBc，ACLR2:50dBc) | ACS of legacy gNB |

Figure 1: interference from NR BS to A-IoT BS receiver

For the case where the NR BS is aggressor in DL, the interference from NR BS on the A-IoT BS or A-IoT device mainly is decided by the iFFT sidelobe level. Figure 1 illustrates the case where NR BS DL interferer with A-IoT Reader.  The interference level can be illustrated by the simulation of OFDM modulation where one RB is blanked. Figure 2 shows that the interference to the blanked RB can be around 17 dB relative to the average NR transmission power.   The simulation does not consider the RF impairment and therefore, there is a need to add a margin of 2 dB on top of this to account the PA nonlinerity, the clipping noise etc.

The inband leakage from iFFT modulation should be 15 dB considering additional RF impairment

Figure 2: IBE for NR DL with one RB blanking

When NR receiver is victim, aggressor is A-IoT system (e. g A-IoT BS or A-IoT UE). Though the interferer from A-IoT BS DL is OFDM signal, it may not be orthogonal with the NR base station receiving. When macro NR BS operating at n8 is configured with 30kHz SCS, the A-IoT BS with 15kHz SCS will appear as the different numerology at the NR receiver. This creates a worst-case interferer to NR receiver. For a standard OFDM receiver, it may apply a rectangle time window to cut off the OFDM symbol in time domain to remove the CP, this corresponds to convolution of the sinc-like function time frequency domain and the sinc-like function and will pick up the interference from adjacent non-orthogonal OFDM with other numerology. The receiver windowing can be used in this case to reduce the interference pick up. The interference from inter-numerology receiving is analyzed in [2] and the power of inter-numerology interference caused by a given interfering subcarrier on a victim subcarrier depends on their spectral distance, the channel frequency response of interferer and the overlapping windows generated by the aggressor’s transmitter windows and victim’s receiver window. Figure 3 illustrate the SIR for the victim RB from the A-IoT BS transmission RB with different numerology. It can be observed that the SIR impact will be worse for the adjacent RBs and SIR is also relating to the power difference between A-IoT signal and NR signals. If A-IoT BS is transmitting in UL spectrum, the impact may be severe as the BS power is 33 dBm while UE is 23 dBm. In such a case, there will be more NR RBs impacted. If A-IoT BS transmit in DL spectrum, impact on the NR UE DL receiving may be ignorable if there is no NR UE in factory. Current coexisting simulation assume the same SCS with A-IoT BS, so there is no need to consider the inter-numerology interference.

No need to consider inter-numerology interference as current coexisting assumes the same SCS between NR BS and A-IoT BS.

Check if network has 30kHz SCS deployment in n8 and decide if inter-numerology should be considered in coexisting simulation scope.

Figure 3: The SIR for victim RB with one RB inter numerology receiving at RB index 0

Issue 4-1-3: Rx for device 1 and 2a

Agreement in RAN4#111: Assume no frequency selectivity for co-existence evaluation for calibration purposes for device 1 and 2a.

There is no agreement last meeting on how to consider the NR interference in the device 1/2a for RF envelope detector. LLS is performed based on simulation assumption from Table 1. The NR interferer is modeled with two parameters, number of RB of NR signal and the PSD difference between A-IoT RB and NR RB.

In Figure 5, the 4 RB is configured as NR interferer signal and it can be observed that as long as the PSD difference between NR signal and A-IoT signal is lower than 3 dB and SNR is higher than 24 dB, the block error rate reach the 10% BLER performance target. InFigure 6, 30 RB is configured as NR interferer signal. In Figure 6 it can be observed that the PSD difference between NR signal and A-IoT signal should be lower than 7 dB and SNR higher than 27 dB to reach the 10% BLER performance target. From Figure 5 and Figure 6, it can be observed that when there are wider NR interferer with more RBs along with the A-IoT R2D signal, it requires a lower PSD, which means wider NR interferer RBs worsen the device 1/2a performance by increasing the SNR threshold or requiring a lower PSD than A-IoT signal.

NR interfere worsens the device 1/2a receiver performance by either increasing the SNR threshold or requiring a lower PSD than A-IoT signal.

RAN4 assume a 10 MHz noise bandwidth for R2D and in simulations, the SINR is calculated by integrating the NR interferer within the 10MHz.

Issue 3-2-3: SINR definition for R2D

Agreement in RAN4#112:

- signal power of device to the noise and interference within 10MHz is baseline assumption

- Consider [180KHz] noise and interference bandwidth after BB LPF as optional

In legacy NR UE receiver, if it is configured with 10MHz channel bandwidth but only schedule one RB in DL from BS, the SNR will be calculated using one RB noise bandwidth. RBs scheduled for other UE will impact the SNR for this RB.So basically there is minimum impact on SNR of scheduled RB when other RBs is present at the same time (same PSD level). However, in Figure 7, when there is no NR signal, the SNR threshold is 17.6 dB. And when a NR signal with 12 dB lower PSD than A-IoT signal is present, the SNR increase is 4.4 dB (22 – 17.6). such SNR increase can be deemed as the noise increase from the NR signal interferer in receiver.

Assuming noise band of 10 MHz or 180kHz for R2D is not correct and no in line with the device 1/2a performance.

We suggest further studying how to consider the LLS simulation result in coexisting study and not use 10 MHz noise bandwidth agreed in RAN4#112.

Further study how to consider the LLS simulation in coexisting study.

The noise bandwidth should not be assumed as 10MHz or 180kHz for device 1/2a.

Figure 4: R2D detection in presence of the NR interfering signal

Table 1:Simulation parameters

BW=N PRBs, only 1 PRB for A-IoT and 1 PRB guard each side

TBS= 20

OOk-1

No CFO/No SFO

ADC=8 bits

LPF after RF ED

Channel: TDL_A, 30ns delay spread

Detect_sync= false

Figure 5: BLER performance with 4 RB NR signal

Figure 6: BLER performance with 30 RB NR signal

Figure 7: BLER performance with 104 RB NR signal

Issue 3-3-2: CW cancellation capability

Agreement in RAN4#112:

- Companies to report the CW cancellation capability used for co-existence evaluation, CW cancellation capability for inside topology and outside topology can be different.

  - total interference cancellation capability = spatial isolation + RF cancellation + digital cancellation

  - CW transmission impact on the received SINR, e.g. degradation

In our companion paper [3], the CW cancellation is discussed. For the CW cancellation capability, it relates to the receiver linearity and the leakage power input before LNA, these two parameters can desensitize the receiver. It is difficult to decide the receiver linearity requirements in study phase and therefore, we suggest RAN4 evaluate several SNR degradation factors in coexisting study. Such applies only CW inside topology A2.

Use SNR degradation of 1 dB, 3 dB and 10 dB in coexisting study to evaluate the CW cancellation impact for CW inside topology A2.

## Observations from initial simulation results

### SNR for outage observation

When there is no NR signal interference, the SNR of 17.8 dB for 10% BLER is used in Figure 7 for outage observation assuming OOK1. When NR signal interference presence, the SNR needs to be increased to account the NR in-band noise assuming. Further study is needed to see how to use the different SNR threshold when NR signal is presence.

SNR threshold = 17.8 dB is used for outage observation without NR signal

Further study is needed to see how to use different SNR threshold when NR signal is present.

### Some observations

The CDF curve for the SNR of device is illustrated in Figure 8. This is the scenario 1-1 and NR DL as aggressor to device. It can be observed that 20% device experience the outage in baseline and the outage is increased to 39% when NR DL system enabled. For inventory use case, we think the outage should be very low and even the baseline may pose the usefulness of the technology in risk. RAN4 has changed the active A-IoT BS rate assumption to counteract this issue for baseline but seem it is not enough and therefore, RAN4 needs to discuss further how to improve the baseline outage.

As an extension we model also simulate the case with only one A-IoT reader active in the building. The results are shown in Figure 10. For this case it can easily be seen that the achieved SNR high with 0% outage. This can be explained by the fact that there is no inter-system interference for the A-IoT network. When adding the interference from the NR network the outage is similar to the case with two active readers. This highlights the importance of understanding the impact of NR signals on the AIoT device.

Active only A-IoT BS at a time can have 0 percentage outage.

In Figure 9, it is illustration of one snapshot of the AIoT system. Red triangles are active AIoT readers, blue dots are AIoT devices with Rx power >-36 dBm, yellow dots are AIoT devices not served for this specific combination of reader activation. SINR of the served A-IoT device is collected in the simulation results.

As discussed above, the SINR is optimistic estimation when assuming the 180kHz  noise bandwidth for device 1/2a, further study is needed to see how to model the NR interferer signal and calculate the SINR at device in a reasonable way.

Baseline outage percentile is 20% and needs improvement of coexisting simulation assumption to reduce the outage percentile.

RAN4 study the R2D SINR modeling in future meeting.

20 % percentile

39% percentile

Figure 8: CDF for Scenario 1-1, legacy UE only outdoor:  NR DL (BS) --> Device​

Figure 9: Snapshot of the AIoT system ( Red triangles are active AIoT readers, blue dots are AIoT devices with Rx power >-36 dBm, yellow dots are AIoT devices not served for this specific combination of reader activation.)

Figure 10: Scenario 1-1 with only one reader active in the building

# Conclusions

In this contribution, we present our view on the coexisting simulation with below proposal and observation:

Proposal-1: The inband leakage from iFFT modulation should be 15 dB considering additional RF impairment

Proposal-2: No need to consider inter-numerology interference as current coexisting assumes the same SCS between NR BS and A-IoT BS.

Proposal-3: Check if network has 30kHz SCS deployment in n8 and decide if inter-numerology should be considered in coexisting simulation scope.

Observation 1 NR interfere worsens the device 1/2a receiver performance by either increasing the SNR threshold or requiring a lower PSD than A-IoT signal.

Observation 2 Assuming noise band of 10 MHz or 180kHz for R2D is not correct and no in line with the device 1/2a performance.

Proposal-4: Further study how to consider the LLS simulation in coexisting study.

Proposal-5:The noise bandwidth should not be assumed as 10MHz or 180kHz for device 1/2a.

Proposal-6: Use SNR degradation of 1 dB, 3 dB and 10 dB in coexisting study to evaluate the CW cancellation impact for CW inside topology A2.

Observation 3 SNR threshold = 17.8 dB is used for outage observation without NR signal

Observation 4 Further study is needed to see how to use different SNR threshold when NR signal is present.

Observation 5 Active only A-IoT BS at a time can have 0 percentage outage.

Proposal-7: Baseline outage percentile is 20% and needs improvement of coexisting simulation assumption to reduce the outage percentile.

Proposal-8: RAN4 study the R2D SINR modeling in future meeting.

# References

- R4-2414304, WF on Co-existence study for ambient IoT, CMCC

- Xiaoying Zhang , Lei Zhang etc,“ Mixed Numerologies Interference Analysis and Inter-Numerology Interference Cancellation for Windowed OFDM Systems” IEEE TRANSACTIONS ON VEHICULAR TECHNOLOGY, VOL. 67, NO. 8, AUGUST 2018

- R4-2415452, On CW cancellation capability, Ericsson

# Annex