# 3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415457

# Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			On WUR RF requirement testability

Agenda item:		6.23.2.4

Document for:		Approval

# Introduction

In this paper, we present our overview on the WUR RF requirement testability issue.

# Discussion

In WF[1], there are couple of issues relating to the testing aspects.

Issue 1-1-2: Performance metric for Rx RF requirements

Agreement:

- 1% MDR for evaluation [and core requirements]

- FFS on the testability issue.

Issue 4-1-1: General framework on LP-WUS testing

Agreements:

  - RAN4 further discuss the high-level approach on how to achieve LP-WUS performance verification.

For the test metric, the MDR of 1% could be used with quoted RAN1 agreement as below. In the simulation, it can be observed that if a higher MDR would be used, the SNR threshold will be lowered. Meaning UE may pass the test with more margin. In this sense, we think the 1% MRD should be used for core requirement also.

Agreement

For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS:

- The followings are considered when down-select among options:

  - The number of supported UE subgroups per PO: M

  - Average network overhead to indicate the number of UE subgroups M per PO

  - False wake up rate due to subgroup-based indication, which will impact the power saving gain

  - Paging latency

  - Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

1% MDR (BLER) should be used for core requriement.

RAN4 discussed the performance metric for several meetings now and main concern is the test time and some companies want to relax the MDR to reduce the test time. In our view, as test time may be concern in RAN5 and RAN5 could also introduce new test mode if they deem the test time would be long and further optimization for it. This can be decided by RAN5 and if RAN4 cannot reach the metric for requirement, RAN4 can send LS to RAN5 to trigger the discussion.

Send a LS to RAN5 to see if RAN4 cannot reach consensus on the core requirement metric.

LS to RAN5 to confirm this with text below:

In case a LS to RAN5, below LS text is proposed:

RAN4 are discussing the test metric for wake up receiver RF performance test. As the WUR only detect LP-WUS/LP-SS and no other signals, legacy throughout monitoring for PDSCH is not possible anymore. Therefore, RAN4 propose the test metric of Miss Detection Rate of LP-WUS (target 1%). To test MDR of LP-WUS, there are two options to do it and RAN4 agree that it will be up to RAN5 to decide which options is suitable for WUR receiver test.

Two options below to test

- Using the legacy paging procedure to detect successfully LP-WUS reception by WUR

- New test mode for testing the successfully LP-WUS reception by WUR

# Conclusions

In this contribution, we present our view on WUR RF testability with below proposal:

Proposal-1: One test condition is enough for same RF requirements, no need to repeat the same RF requirement with different test conditions.

Proposal-2: 1% MDR (BLER) should be used and postpone FAR discussion to performance phase.

Proposal-3: Send a LS to RAN5 to see if test time could be further optimized from testing perspective.

Proposal-4: LS to RAN5 to confirm this with text below:

RAN4 are discussing the test metric for wake up receiver RF performance test. As the WUR only detect LP-WUS/LP-SS and no other signals, legacy throughout monitoring for PDSCH is not possible anymore. Therefore, RAN4 propose the test metric of Miss Detection Rate of LP-WUS (target 1%). To test MDR of LP-WUS, there are two options to do it and RAN4 agree that it will be up to RAN5 to decide which options is suitable for WUR receiver test.

Two options below to test

- Using the legacy paging procedure to detect successfully LP-WUS reception by WUR

- New test mode for testing the successfully LP-WUS reception by WUR

# References

- R4-2414309, WF on LP-WUS UE RF requirements, vivo

# Annex 1

3GPP TSG-RAN WG4 Meeting # 112bis						R4-240xyz

Hefei, China, 14th – 18th  October, 2024

Title:	LS on WUR testibility

Response to:

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN WG4

To:	RAN WG5

Cc:

Contact Person:

Name:	Chunhui Zhang

E-mail Address:	Chunhui (dot) Zhang (at) ericsson (dot) com

Attachments:

1. Overall Description:

RAN4 are discussing the test metric for wake up receiver RF performance test. As the WUR only detect LP-WUS/LP-SS and no other signals, legacy throughout monitoring for PDSCH is not possible anymore. Therefore, RAN4 propose the test metric of Miss Detection Rate of LP-WUS (target 1%) as one option. To test MDR of LP-WUS, there are two options to do it and RAN4 agree that it will be up to RAN5 to decide which options is suitable for WUR receiver test.

Two options below to test

- Using the legacy paging procedure to detect successfully LP-WUS reception by WUR

- New test mode for testing the successfully LP-WUS reception by WUR

2. Actions:

To RAN WG1/WG2 group.

ACTION: 	RAN4 would like RAN5 consider the information above.

3. References:

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #113		Nov 18-22 , 2024						Orlando