# 3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415459

# Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			WUR RF requirement of REFSESN, ASC, ASCS

Agenda item:		6.23.2.2

Document for:		Approval

# Introduction

In this paper, we present our view on the WUR RF requirement of REFSENS, ASC, ASCS.

# Discussion

### REFSENS

In WF [2], it is agreed that RAN4 will study the SNR target used for REFSENS calculation. The Channel model is AWGN and no repetition is sued.

Issue 2-1-1: Channel model to specify LP-WUS RF requirements

Agreement:

  - Follow typical RF requirements approach, AWGN should be selected.

Issue 2-1-2: Target SNR simulation condition

Agreement:

  - Follow similar approach of MR NR, no repetition should be used when simulate target SNR.

Issue 2-1-3: Whether RAN4 should conclude a target SNR first

Agreement:

- Target SNR is the basis for many Rx requirements discussion. Group should conclude SNR definition and SNR values first.

Issue 2-1-4: Target SNR value

Agreements:

  - RAN4 further discuss target SNR value for LP-WUS

  - FFS single SNR for OOK-based and OFDM-based receiver

For OOK WUR, our opinion is that the ASCS LLS simulation result can be referred to derive the SNR target and no need to redo the LLS. The key parameter for ASCS is the filter order and the guard RB. It can be observed that the when guard RB greater than 1, the SNR threshold @ 1% MDR has 0.5 dB difference between filter order of 3 and 5. Table 1 lists the SNR points for different configuration according to Figure 2 and Figure 3.

Table 1: SNR  target for OOK1 and OOK4 with different filter order

|  | OOK1 | OOK4 |
| --- | --- | --- |
| Filter order 3 | -5 dB | - 2.5 dB |
| Filter order 5 | -4.5 dB | -1.5 dB |

For OFDM WUR, the 1% MSD is simulated and the detection performance is illustrated in Figure 1. It can be observed that the SNR target is around -15 dB which is quite low compared to the OOK WUR SNR point.

Figure 1: OFDM sequence detection performance

Table 2: WUS resources configuration (4.32 MHz WUS bandwidth).

WUS payload (modulation)

1bit (OFDM, SSS-based time-correlation)

1 sequence of word payload

The SNR for OFDM WUR detection is -15.6 dB.

The SNR for OOK WUR detection depend on the filter order OOK1 or OOK4 with range of -5 dB to -1.5 dB.

### ASCS

From the LLS simulation, when filter order is 3th or 5th. The BLER performance is illustrated in Figure 2 to Figure 5. It shows that larger guard RB size has penalty on the SNR threshold due to less received energy. Such observation holds for both AWGN and TDL-C channel and for OOK1 and OOK4. Because of this, the guard RB of less or equal than 1 RB at each side of WUS signal is good enough. The guard RB of 1 corresponds to the case where 11 RB WUS is configured and LP-WUS signal is confined within a 5MHz frequency block (13 RB in total within 5MHz block).

In Figure 6, it is illustrated that without guard RB, there is 0.5 dB SNR degradation compared with 1 guard RB at each side of WUS signal (WUS signal still 11 RB) for filter order of 3. This means that from testing perspective, half dB relaxation will be needed if there is no guard RB between NR signal and LP-WUS signal.

ASCS is to test the filter attenuation of the WUR and ACS is also test the filter attenuation with considering the dynamic range.  ACS is set a worst case test and therefore, there is no need to test ASCS as ACS test will be specified.

Larger guard RB size has penalty on the SNR threshold due to less received energy

Maximum of 1 guard RB at each side of WUS signal should be used in ASCS case.

Without any guard RB between NR signal and LP-WUS result in Half dB SNR degradation.

No need to test ASCS and use ACS testing to test filter attenuation.

The simulation case for the parameter setting is below.

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | 8/16 bits payload |  |  |
| Chip rate | M=1 |  |  |
| WUS duration | 16 OFDM symbols |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ASCS: 1/1.5/2 RB on each side of LP-WUS bandwidth |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |
| Wanted signal level | Same PSD with NR signal |  |  |
| Sampling rate | 7.68MHz |  |  |
| Phase noise | Not modelled |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER - The following false alarm rate is used - 0.1% |  |  |

Figure 2: BLER for ASCS with different LPF order (3 and 5) and guardRB size (1 and 2) with AWGN channel and OOK1

Figure 3: BLER for ASCS with different LPF order (3 and 5) and guardRB size (1 and 2) with AWGN channel and OOK4

Figure 4: BLER for ASCS with different LPF order (3 and 5) and guardRB size (1 and 2) with TDL-c channel and OOK1

Figure 5:BLER for ASCS with different LPF order (3 and 5) and guardRB size (1 and 2) with TDL-c channel and OOK4

Figure 6: BLER for ASCS with LPF order 3 and guardRB size (0 and 1) for OOK1

### ACS

The simulation assumption from below table is used.

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | Total 8 bits |  |  |
| Chip rate | M=1 | M=2 |  |
| WUS duration | FFS |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ACS: 1/2/3/4 RB |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = 33dB NOTE: decide the interference level depending on SNR |  |  |
| Sampling rate | 7.68MHz |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS |  |  |
| Phase noise | Not modelled |  |  |
| Non-linearities | Not modelled |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER - The following false alarm rate can be considered - 0.1% |  |  |

In LLS simulation, the ACS cases are simulated with ACI =0 and ACI =33 with different shifted RB and filter order. The purpose of such simulation is to identify the needed wanted signal increase to compensate the noise increase caused by adjacent carrier interferer. For OOK1 waveform, it is illustrated in Figure 7: The BLER for ACI=0 and ACI=33 with LPF order of 3 and shift RB number of 0 and OOK1Figure 7 that the potential SNR increase is around 6 dB with low order of filter and 0 shift RB. If the shift RB size is increased to 2, the SNR increase will be reduced less than 1 dB and this is illustrated in Figure 8. For OOK4 waveform, there is a need to add at least 2 shift RB for the case of filter order of 3 and ACI=33. The SNR increase is around 5 dB in this case. When shift RB size is increased to 4, the SNR increase is less than 1 dB.

Increasing the ACI from 0 to 33 for OOK-1 scenario with filter order 3 and no shift RB will deteriorate the BLER performance maximally by approximately 6 dB.

The SNR increase can be reduced when increasing either filter order or shifted RB size.

The SNR increase is around 5 dB for OOK4 with filter order 3 and shift RB size of 2 comparing between the ACI =0 and ACI=33 dB case.

Figure 7: The BLER for ACI=0 and ACI=33 with LPF order of 3 and shift RB number of 0 and OOK1

Figure 8:The BLER for ACI=0 and ACI=33 with LPF order of 3 and shift RB number of 2 and OOK1

Figure 9:The BLER for ACI=0 and ACI=33 with LPF order of 3 &5 and shift RB number of 0&2 and OOK4

Figure 10: The BLER for ACI=0 and ACI=33 with LPF order of 3 and shift RB number of 4 and OOK4

Figure 11:The BLER for ACI=0 and ACI=33 with LPF order of 3 and shift RB number of 0/2 and OOK1 for TDL-C

Figure 12: The BLER for ACI=0 and ACI=33 with LPF order of 3/5 and shift RB number of 0/4 and OOK4 for TDL-C

Figure 13: BLER performance for OOK1 and ACI=33 with ADC bit 8

Figure 14: BLER performance for OOK1 and ACI=33 with ADC bit 4

Figure 15: BLER performance for OOK4 and ACI=33 with ADC bit 4

Figure 16: BLER performance for OOK4 and ACI=33 with ADC bit 8

It is illustrated in Figure 13 to Figure 16 for comparison of 4 bits ADC and 8 bits ADC, it is observed that additional shifted RB will be needed for ACI=33 dB cases for LPF order of 3 for OOK1 and OOK4.

Additional shifted RB may be needed for OOK1 and OOK4 with a 4 bits ADC for LPF order of 3.

There are two test cases for ACS, case #1 and case #2.  For case #1, the ACS interferer is set relative to the wanted signal with the power difference of the selectivity value. From the LLS simulation, WUR can achieve the same selectivity with the MR. As such, the WUR ACS can be tested with the same selectivity with MR.  As the ACS test focus testing the filter attenuation and therefore, the WUR ACS interference should be set the same way as the MR ACS, meaning the ACS interferer level is set relative to the WUR REFSENS in addition to the relaxation to compensate the noise raise and the ACS.

WUR can achieve the same ACS as the MR.

WUR ACS of case #1 can be tested as the similar with MR ACS case #1.

For ACS test case #2, it can be the same with MR channel bandwidth of 5MHz. The difference is that there is no need to transmit the UL for FDD bands.  As the WUS signal can be placed at the near edge of the MR channel, therefore, it is possible to test it only for the minimum MR channel bandwidth where has the minimum guard band among all channel bandwidth supported by MR.

WUR ACS of case #2 can be tested with the same wanted and interference level

WUR ACS of case #2 can be test with only the minimum MR channel bandwidth.

Specify the same ACS selectivity for WUR with MR.

# Conclusions

In this contribution, we present our view on WUR RF requirement for REFSENS, ASCS and ACS with below proposal and observations:

Observation 1 The SNR for OFDM WUR detection is -15.6 dB.

Observation 2 The SNR for OOK WUR detection depend on the filter order OOK1 or OOK4 with range of -5 dB to -1.5 dB.

Observation 3 Larger guard RB size has penalty on the SNR threshold due to less received energy

Proposal-1: Maximum of 1 guard RB at each side of WUS signal should be used in ASCS case.

Observation 4 Without any guard RB between NR signal and LP-WUS result in Half dB SNR degradation.

Proposal-2: No need to test ASCS and use ACS testing to test filter attenuation.

Observation 5 Increasing the ACI from 0 to 33 for OOK-1 scenario with filter order 3 and no shift RB will deteriorate the BLER performance maximally by approximately 6 dB.

Observation 6 The SNR increase can be reduced when increasing either filter order or shifted RB size.

Observation 7 The SNR increase is around 5 dB for OOK4 with filter order 3 and shift RB size of 2 comparing between the ACI =0 and ACI=33 dB case.

Observation 8 Additional shifted RB may be needed for OOK1 and OOK4 with a 4 bits ADC for LPF order of 3.

Observation 9 WUR can achieve the same ACS as the MR.

Observation 10 WUR ACS of case #1 can be tested as the similar with MR ACS case #1.

Observation 11 WUR ACS of case #2 can be tested with the same wanted and interference level

Observation 12 WUR ACS of case #2 can be test with only the minimum MR channel bandwidth.

Proposal-3:Specify the same ACS selectivity for WUR with MR.

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo etc

- R4-2414309, WF on LP-WUS UE RF requirements, vivo