3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415461

Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			On inband deployment of NTN NB-IoT in NR carrier

Agenda item:		4.4.1

Document for:		Approval

# Introduction

In this paper, we present our overview on the  in-band deployment of NTN NB-IoT in NR carrier.

# Discussion

There have been discussion in past several meetings on the in-band deployment support for NTN NB-IoT in a NR carrier and some companies want to use TEI CR to add the in-band support of the NTN NB-IoT in Rel-18 specification.

In Rel-18 WID, the deployment mode is limited to the stand-alone mode only. Our understanding is that Rel-17/Rel-18 NTN NB-IoT may not support the in-band deployment mode as it was not required in the Rel-18 WID[1].

A Rel-17 or Rel-18 NTN NB-IoT UE is not supposed to support the in-band deployment mode according to the Rel-18 WID.

Therefore, introducing the in-band operation mode in TEI will introduce the new functionality which the legacy UE may not support, which in turn create the non-backward compatibility (NBC) issue.

Introducing the in-band deployment mode in TEI CR in Rel-18 creates the NBC issue.

It is unclear how the NTN NB-IoT UE is deployed in network till now, we see there is an opportunity to mandate the Rel-17 or Rel-18 UE with both in-band and standalone mode support if there is no NTN NB-IoT Rel-17 or Rel-18 is deployed.

Mandating the Rel-17 or Rel-18 UE with in-band deployment support is possible if there is no such NTN NB-IoT deployed in the field.

If there are Rel-17/rel-18 NTN NB-IoT UE deployed in field, mandating a feature which is not supported by such UE will not be possible. In such a case, RAN4 should discuss from which release the in-band and standalone mode should be mandatory to support.

RAN4 should discuss in which release the in-band and stand-alone support should be mandatory to support.

Meantime, there are RF impact on the TS 38.108. TS 38.108 needs to be updated to add requirements enabling NB-IoT operation with NR (e.g. REFSENS), like it was done in TS 38.104. There shall be a need to update 36.108 also to clarify the in-band support, e.g EARFCN to adopt the same with 36.102 and Rx requirements are for standalone only. SAN BS needs to declare the NB-IoT inband support. Without this it is not clear how NB-IoT would be supported, in 38.141-1, the declaration is quoted as below.

TS 38.108 and TS 36.108 needs to be updated to add the inband operation support.

| D.41 | NB-IoT operation | Manufacturer shall declare the support of NB-IoT operation in NR in-band and the number of supported NB-IoT carriers in total and for each supported band, frequency range and channel bandwidth. |
| --- | --- | --- |
| D.42 | NB-IoT sub-carrier spacing | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare if it supports 15 kHz sub-carrier spacing, 3.75 kHz sub-carrier spacing, or both for NPUSCH. |
| D.43 | NB-IoT power dynamic range | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of +6dB or +3dB as specified in clause 6.3.4 of TS 38.104 [2] (Note 5). |

Additionally, if there are NTN NB-IoT deployed in the network without support of the inband operation, the stand-alone operation needs to be TS 36.108 compliant. This means the SAN shall also be TS36.108 compliance..

The SAN   needs to comply with TS 38.108 and TS 36.108  if the both stand-alone and in-band operation to be supported.

As the NBC issue is most critical to solve, we propose a generic RAN4 common understanding is needed before any action taken to introduce the in-band operation support for NTN NB-IoT in NR carrier.

Align the RAN4 understanding on NBC issue and how to support in-band operation before making any specification updates.

# Conclusions

In this contribution, we present our overview on the  in-band deployment of NTN NB-IoT in NR carrier with below proposal and observations:

Observation 1 A Rel-17 or Rel-18 NTN NB-IoT UE is not supposed to support the in-band deployment mode according to the Rel-18 WID.

Observation 2 Introducing the in-band deployment mode in TEI CR in Rel-18 creates the NBC issue.

Observation 3 Mandating the Rel-17 or Rel-18 UE with in-band deployment support is possible if there is no such NTN NB-IoT deployed in the field.

Proposal-1: RAN4 should discuss in which release the in-band and stand-alone support should be mandatory to support.

Observation 4 TS 38.108 and TS 36.108 needs to be updated to add the inband operation support.

Observation 5 The SAN   needs to comply with TS 38.108 and TS 36.108  if the both stand-alone and in-band operation to be supported.

Proposal-2: Align the RAN4 understanding on NBC issue and how to support in-band operation before making any specification updates.

# References

- RP-223437, Revised WID on NB-IoT/eMTC core & performance requirements for NTN, MTK