# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415463

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	8.2

Source: 	Huawei, HiSilicon

Title:	Discussion on NR CA power class for single configured UL

Document for:	Approval

# 1	Introduction

RAN has assigned a task to RAN4 as shown below [1].

Way Forward

- RAN tasks RAN4 to investigate how to enable HPUE maximum transmit power in downlink CA with a single configured UL.

  - RAN task shall conclude by RAN#106

  - RAN4 online meeting time should be limited to <0.1 TUs

In the last meeting, our views were shared in [2] as well as during offline/online discussions. In the following, we further clarify our concerns towards the unapproved WF [3] as shown below.

Agreements Proposal:

To enable UE to support higher power class than default power class for single configured UL carrier with DL CA configuration when HPUE requirements for the band combination have not been specified:

- Reference sensitivity requirements (e.g. MSD) for the band combination are only verified with default power class transmission, unless otherwise specified

  - This approach is specific for the case under RAN task and de-coupled from MSD discussion under other WIs

- UE is allowed to enable higher power class than default power class of single configured UL carrier with DL CA configuration when single carrier requirements with higher power class requirements and default power class requirements for the CA configuration exist in specification

- It is optional for UE to support higher than default power class for the above band combinations.

# 2	Discussion

First of all, the key issue of the RAN task is to address the MSD requirements. As described in [3], the challenge is that:

“HPUE requirements for the band combination have not been specified”

The proposed solution in [3] is to “only verified with default power class transmission, unless otherwise specified”. This seems to imply that:

- Some HPUE band combinations (i.e. unspecified) are verified with only PC3, some (i.e. specified) are verified with the corresponding high-power classes. Hence, inconsistency arises.

- Some HPUE band combinations are initially verified with PC3, but later switched to PC2/PC1.5 when the corresponding MSD requirements are completed by the HPUE basket WIs. As a result, the MSD requirements for the same band combination would become unstable.

In summary, we observe that:

Observation 1: The proposed solution in [3] would lead to inconsistent and unstable HPUE MSD requirements for band combinations, resulting in great difficulty for product development as well as conformance tests.

On the other hand, there have been discussions on the potential changes to the MSD rules under the “High power UE for CA in terrestrial network” agenda. For example, the WF in [4] says:

## 4.1 MSD rules

Way forward:

- RAN4 can discuss the MSD general rules, and strive to down-select among the following options

- PC2/PC1.5 MSD requirements are not specified. UE shall meet the Rx requirements of default power class, with default power class transmission

- Using some equations or look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD

- Other solutions are not precluded

NOTE: RAN4 continues normal procedure specifying MSD for all PC’s band combination supports until down-selection is done

Whatever new MSD rules might become, consistency should be ensured.

Proposal 1: Consistent MSD rules should be maintained for all band combinations, and wait for the conclusion on the MSD rules in the HPUE CA agenda item.

The RAN task targets a specific type of band combination, i.e. downlink CA with a single configured UL. For many of such band combinations, the HPUE requirements have been completed since Rel-17. The signalling mechanism as described in [2] have been used to report the HPUE capabilities. Hence, no new signalling or UE capabilities are needed for the RAN task.

Proposal 2: No new signalling is needed for the RAN task. The same signalling mechanism utilised by the existing specified HPUE band combinations can be reused.

# 3	Conclusion

In this paper, we further discuss the RAN task and made the following proposals:

Observation 1: The proposed solution in [3] would lead to inconsistent and unstable HPUE MSD requirements for band combinations, resulting in great difficulty for product development as well as conformance tests.

Proposal 1: Consistent MSD rules should be maintained for all band combinations, and wait for the conclusion on the MSD rules in the HPUE CA agenda item.

Proposal 2: No new signalling is needed for the RAN task. The same signalling mechanism utilised by the existing specified HPUE band combinations can be reused.

# References

- RP-241687 Moderator's summary for offline discussion on CA Power Class Support, Moderator (AT&T), RAN#104

- R4-2413026 Discussion on NR CA power class for single configured UL, Huawei, HiSilicon, RAN4#112

- R4-2414436 Way Forward for RAN task on HPUE with 1UL and DL CA, Qualcomm, et al, RAN4#112

- R4-2414277 WF on HPUE for CA in TN, Samsung, RAN4#112