3GPP TSG-RAN WG4 Meeting #112bis	R4- 2415466

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.21.3.2

Source: 	Huawei, HiSilicon

Title:	On the RF Requirements for Ambient IoT Devices

Document for:	Approval

# 1	Introduction

Following the Ambient IoT SID [1], the WF [2] from last meeting captures the progress on the study of impacts of A-IoT on RF requirements. In the sequel, we share our views on the open issues.

# 2	Discussion

### 2.1 System Parameters

Agreement in RAN4#112:

- Requirements potentially needed:

  - channel bandwidth

- FFS whether requirements are needed or not:

  - Transmission bandwidth configuration;

  - Channel spacing

  - Channel raster

Note: The system parameters are defined from the network perspective. Some of them might not be needed for devices.

- Requirements not needed:

  - Synchronization raster

  - Guard band/Guard RB

Note: Guard band/RB can be considered as part of RF requirements and/or test cases.

The transmission bandwidth configuration defines the bandwidth of the R2D/D2R signal in baseband. For the R2D channel, one or multiple PRBs are under consideration in RAN1. For D2R channel, the transmission bandwidth of 15kHz or more is being discussed.

The configuration of transmission bandwidth affects the achievable data rate. For example, it’s necessary to know whether a device is capable of receiving and demodulating one PRB or multiple PRBs. Most likely, it’s limited by the BB LPF as well as BB data processing capability.

For devices using RF-ED receivers, the channel spacing and channel raster are unnecessary. This is because that the RF-ED receivers will not be able to differentiate different channels. On the other hand, the network work might need these two parameters for deployment purpose, for example, to co-deploy with existing NR network.

In summary, we propose that:

Proposal 1: For the following system parameters, we recommend:

| Transmission bandwidth configuration | potentially needed, affects achievable data rate |
| --- | --- |
| Channel spacing | not needed for devices, potentially needed for A-IoT BS |
| Channel raster |  |

### 2.2 Receiver

Agreement in RAN4#112:

- Requirements potentially needed:

  - Reference sensitivity

- FFS whether requirements are needed or not:

  - Maximum input power

  - Receiver intermodulation

  - Rx spurious emission

  - ACS

  - ACSC

  - In-band blocking

  - Out-of-band blocking

- Requirements not needed:

  - Spurious response

In a factory or a warehouse, the location of a device could be close to the reader installed on the ceiling. For a portable reader such as an intermediate UE, it could also get very close to a device. It’s important to ensure that the device can function properly at close distance, in other words, with large input power.

For UHF RFIDs, similar RF requirements are defined in ISO/IEC 18046-3 [6], such as: maximum operating power of tag, survival electromagnetic power of tag.

By definition, the blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels. For a NR receiver, it’s expected that the NR receiver has a BPF to attenuate the out-of-band blocker, and has multi-stage amplification, LPF and AGC to handle in-band blocker.

By contrast, an AIoT device has much simpler receiver. It might rely on the matching network to achieve coarse frequency selectivity, and the RF-ED would simply convert all received RF signals to the baseband. Apart from the simple LPF, there’s not much the device can do to suppress the interference. As long as the blocking interference is temporary, it’s not a big issue for the AIoT system.

Moreover, it’s not expected that an AIoT system from another operator will be deployed on the adjacent channels. Hence, ACS requirement is not needed, either. However, it’s expected that the AIoT system will be deployed within an NR channel from the same operator. The AIoT device should be able to receive in the presence of a wideband NR interferer. Additionally, the device might also suffer from CW interference from a nearby reader/CW node, if it’s not synchronised with the serving reader.

By definition, the spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the antenna connector. For RF-ED based receivers, there’s no LO for receiving. An LNA might be present for device 2a/b, but the gain would be much smaller than that for an NR receiver. Hence, Rx spurious emission requirement is unnecessary.

In summary, we propose that:

Proposal 2: Considering the simple receiver architecture for the AIoT devices, we recommend:

| Maximum input power | potentially needed |
| --- | --- |
| ACS, ACSC | not needed |
| In-band blocking |  |
| Out-of-band blocking |  |
| Receiver intermodulation |  |
| Rx spurious emission |  |

Proposal 3: Considering the potential deployment scenarios, we recommend to define a new requirement for

| Interference rejection | potentially needed. The interferer could be wide-band NR signal (e.g. in-band deployment) or CW signal (e.g. from unsynchronised reader/CW node). |
| --- | --- |

### 2.3 Transmitter

Agreement in RAN4#112:

- FFS whether requirements are needed or not:

  - Maximum output power

  - EVM

  - Transmit OFF power for device 1/2a

  - Transmit time mask

  - Minimum output power

  - Frequency error for device 2a

  - In band emissions (IBE) for device 2a

  - Carrier leakage for device 2a

  - Occupied bandwidth

  - Transmit intermodulation

- Maximum output power

The devices are expected to work in licensed frequency bands. Hence 23dBm or even 26dBm could be the upper limit for the output power. The device 1 and 2a transmit by backscatter the power received from the CW node. Given the path loss, reflection loss and other factors, it’s unlikely for the backscattered power to exceed 23dBm. Hence, the maximum output power requirement would be unnecessary for device 1 and 2a.

Device 2b generates the Tx signal internally. Given the power consumption constraint, it’s also unlikely for the Tx power to be more than 23dBm. However, for the sake of UL coverage, it’s necessary to check the max Tx power capability.

- Minimum output power

The device 1 and 2a are not expected to perform power control by themselves. The backscattered power is mainly affected by the distance from the CW node. For the sake of UL coverage, it’s worth checking the backscattered power level when the received CW power is low. For CW topology A2, this would correspond to the scenario when the received R2D power is close to the sensitivity level.

- Transmit OFF power for device 1/2a

There’s no such requirement for RFIDs. Hence, it’s unnecessary for device 1/2a which also relies on backscatter.

- Transmit time mask

The ON/OFF time mask seems unnecessary for device 1/2a, since the backscatter control is simple. For device 2b, the transmitter involves active components such as LO and amplifiers. Hence, the transmit time mask requirement might be needed.

- EVM

For UL ASK modulation, there’s no EVM requirement.

- Frequency error for device 2a

For a device 2a that supports large frequency shift, some frequency error requirement might be needed.

- IBE for device 2a

Similar to device 1, IBE is not needed for device 2a.

- Carrier leakage for device 2a

Similar to device 1, IBE is not needed for device 2a.

- Occupied bandwidth

To ensure the majority of the UL signal energy is confined within the allocated frequency resource (e.g. 1RB), it seems necessary to have the requirement of occupied bandwidth.

- Transmit intermodulation

By definition, the transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

Such requirement would be meaningless for device 1/2a, since the CW interferer would be simply backscattered.

In summary, we propose that:

Proposal 4: For the Tx requirements, we recommend:

| Maximum output power | not needed for device 1, 2a potentially needed for device 2b |
| --- | --- |
| Minimum output power | Potentially needed for device 1, 2a not needed for device 2b |
| Transmit OFF power for device 1/2a | not needed for device 1, 2a |
| Transmit time mask | not needed for device 1, 2a potentially needed for device 2b |
| EVM | not needed for UL ASK modulation |
| Frequency error for device 2a | potentially needed for device 2a that supports large frequency shift |
| IBE for device 2a | not needed |
| Carrier leakage for device 2a | not needed |
| Occupied bandwidth | potentially needed |
| Transmit intermodulation | not needed |

### 2.4 Testability

Agreement: in RAN4#111:

- FFS on whether conducted conformance test is feasible for AIOT devices.

- FFS on OTA test method, performance metric, etc.

Agreement in RAN4#112:

- Take OTA test as the baseline for at least Devices 1 and 2a

For RFIDs, other test methods such as conducted measurement, coupling with a test fixture have been specified [4] in addition to radiated measurement. For example, the following are stated in [4]:

5.4.2.2 EUT with an external RF connector

The EUT with an external RF connector offered for testing shall provide a 50 connector for conducted RF power measurements.

5.4.2.3 EUT without an external RF connector

5.4.2.3.1 General Considerations

Conducted measurements on an EUT with an integral antenna or with an antenna connection other than a 50Ωcoaxial connector may be made by:

- access to an internal connector; or

- fitting of a temporary connector, if no internal connector is available; or

- use of a test fixture, if none of above possibilities are available.

Proposal 5: In addition to OTA test, conducted measurement and the use of a test fixture can also be considered.

# 3	Conclusion

In this paper, we have shared further analysis on the potential RF requirements for A-IoT devices. The following observations and proposals are made:

Proposal 1: For the following system parameters, we recommend:

| Transmission bandwidth configuration | potentially needed, affects achievable data rate |
| --- | --- |
| Channel spacing | not needed for devices, potentially needed for A-IoT BS |
| Channel raster |  |

Proposal 2: Considering the simple receiver architecture for the AIoT devices, we recommend:

| Maximum input power | potentially needed |
| --- | --- |
| ACS, ACSC | not needed |
| In-band blocking |  |
| Out-of-band blocking |  |
| Receiver intermodulation |  |
| Rx spurious emission |  |

Proposal 3: Considering the potential deployment scenarios, we recommend to define a new requirement for

| Interference rejection | potentially needed. The interferer could be wide-band NR signal (e.g. in-band deployment) or CW signal (e.g. from unsynchronised reader/CW node). |
| --- | --- |

Proposal 4: For the Tx requirements, we recommend:

| Maximum output power | not needed for device 1, 2a potentially needed for device 2b |
| --- | --- |
| Minimum output power | Potentially needed for device 1, 2a not needed for device 2b |
| Transmit OFF power for device 1/2a | not needed for device 1, 2a |
| Transmit time mask | not needed for device 1, 2a potentially needed for device 2b |
| EVM | not needed for UL ASK modulation |
| Frequency error for device 2a | potentially needed for device 2a that supports large frequency shift |
| IBE for device 2a | not needed |
| Carrier leakage for device 2a | not needed |
| Occupied bandwidth | potentially needed |
| Transmit intermodulation | not needed |

Proposal 5: In addition to OTA test, conducted measurement and the use of a test fixture can also be considered.

# References

- RP-240826, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

- R4-2414305 WF on impacts of A-IoT on RF requirements, Moderator (Huawei), RAN4#112

- ISO/IEC 18046-3 Information technology – Radio frequency identification device performance test methods – Part 3: Test methods for tag performance

- ETSI EN 302 208, Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum