3GPP TSG-RAN4 Meeting #112bis	R4-2415467

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.20.1

Source: 	Qualcomm Incorporated

Title: 	Views on general aspects for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#112, a WF was agreed in [2] based on the topics listed in the moderator summary, that is summarized in [3]. In this contribution we share our views on general aspects for Rel-19 SBFD WI within RAN4.

# General aspects

## Band/channel bandwidth support for SBFD

In [2], the following was agreed on the support of SBFD in core specs.

- Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = [20] - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value. - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators. - FFS how to implement in the spec: - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz.

In principle, any TDD defined band in Section 5.2 in TS 38.104 could be a target band for SBFD operation, since SBFD is as an enhancement of TDD duplex operation via allowing the simultaneous existence of downlink and uplink subband at the gNB side within a TDD carrier in a conventional TDD band (hence, the term subband non-overlapping full duplex). On top of that, vendors can declare to support the feature via the WF captured in [2]. In order to provide a framework for introducing SBFD for the different 3GPP defined bands, given in Section 5.2 in TS 38.104, RAN4 needs to agree on a threshold value (i.e., X) for existing TDD bands, representing the difference between upper and lower TDD band edges. In [2], several companies were proposing X between 10 MHz and up to 50MHz. Accordingly, 20 MHz was selected as a compromise. In our view, we believe that 20MHz presents a good compromise between feature applicability to TDD bands and ensuring that system level gains can be achieved from the feature deployment. In addition, although option 1 and 2 addresses the same objective, but with different ways, we believe that adding text in option 1 presents a more elegant solution on how to introduce SBFD feature to the 3GPP defined bands in TS 38.104.

Observation 1: In RAN4#112, proposed value of X for SBFD applicability for existing TDD bands was between 10 MHz and 50 MHz.

Proposal 1: RAN4 to adopt X = 20 MHz and to include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1.

## DL/UL subband configurations support for SBFD

In [2], the following was agreed regarding how to handle guard band and subband configurations in the core specification

- To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB

For an SBFD-capable gNB, a guardband may be needed at the gNB to protect UL reception within the UL subband and reduce the impact of self-interference. Generally, there is a dependency between the amount of guardband needed and the gNB self-interference cancellation capability. During the study item phase, 5 PRBs were typically assumed, as highlighted in Section 9 in [3]. However, RAN4 did not study the exact amount of guard band needed for each subband configuration, since it is implementation dependent.

Observation 2: During study item phase, 5 PRBs were generally considered by several companies when studying the feasibility of wide area SBFD deployments. However, for different SBFD configuration and different cancellation techniques, that value of the guardband will vary.

In our view option 2 above provides a compromise between flexibility and elements requirements to progress the normative work during the Rel-19 work item. By setting the UL subband configuration to be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1, while the  DL subband size(s) and guardband size(s) depends on BS implementation, RAN4 would provide flexibility for implementation aspects that might differ from one vendor to another.

Observation 3: By setting the UL subband configuration to be selected from the existing values of BS transmission bandwidth configuration, as provided TS 38.104, while letting the DL subband size(s) and guardband size(s) depends on BS implementation, RAN4 would provide flexibility for implementation aspects that might differ from one vendor to another.

Regarding the definition of the UL/DL subbands configurations for RF requirements, it is viable to consider a configuration-agnostic approach, such as the RF requirements are defined independent of the subband configuration. The supported configuration themselves can be declaration-based in order not to restrict SBFD deployment to a given set of configurations.

Observation 4: An agnostic configuration approach in relation to the subband configuration should be considered in order not to restrict the normative work to a set of subband configurations.

Additionally, how much guardband is needed also depends on the employed subband configuration (e.g., DUD configuration might have different guardband assumptions compared to DU configuration) as well as the employed mitigation techniques. Thus, there is dependency on the implementation aspects that might be difficult to agree on. As a results, it is viable for RAN4 to agree on option 2b as a way forward when defining the DL, UL, and GB subband configurations.

Proposal 2: RAN4 to agree on selecting UL subband configuration to be selected from the existing values of BS transmission bandwidth configuration, as provided TS 38.104, while letting the DL subband size(s) and guardband size(s) depends on BS implementation.

## BS RF Specification structure for SBFD requirements

In [2], the following was agreed regarding how to include SBFD requirements in the core specifications

- Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104) - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable. - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements - Option 3: A new specification for SBFD BS RF requirements

In our view, option one, i.e., creating new and standalone sub-clauses, is preferred as it makes the readability of the spec easier and aligns with UE core specification (TS 38.101-1). Such an approach has also been adopted in other studies (e.g., incorporation of mobile IAB feature into the IAB core and conformance specs TS 38.174, TS 38.176-1, and TS 38.176-2). An additional clause (e.g., Suffix B) should be used for indicating SBFD-specific requirements. Additionally, a SBFD-capable gNB, which supports the SBFD feature needs to meet both the general requirements in the core spec and the additional requirement contained in the additional clause (suffixes B). In case there is a difference in requirement between the general requirements and the additional clause requirements (suffix B), the tighter requirements are applicable unless stated otherwise in the additional clause.

Proposal 3: To enhance the readability of the spec, it is preferable to have standalone sub-clauses in core specs for SBFD-specific existing or new gNB RF requirements.

# Conclusion

In this contribution, we provided our views on general aspects for SBFD operation with the following observations and proposals:

Observation 1: In RAN4#112, proposed value of X for SBFD applicability for existing TDD bands was between 10 MHz and 50 MHz.

Proposal 1: RAN4 to adopt X = 20 MHz and to include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1.

Observation 2: During study item phase, 5 PRBs were generally considered by several companies when studying the feasibility of wide area SBFD deployments. However, for different SBFD configuration and different cancellation techniques, that value of the guardband will vary.

Observation 3: By setting the UL subband configuration to be selected from the existing values of BS transmission bandwidth configuration, as provided TS 38.104, while letting the DL subband size(s) and guardband size(s) depends on BS implementation, RAN4 would provide flexibility for implementation aspects that might differ from one vendor to another.

Observation 4: An agnostic configuration approach in relation to the subband configuration should be considered in order not to restrict the normative work to a set of subband configurations.

Proposal 2: RAN4 to agree on selecting UL subband configuration to be selected from the existing values of BS transmission bandwidth configuration, as provided TS 38.104, while letting the DL subband size(s) and guardband size(s) depends on BS implementation.

Proposal 3: To enhance the readability of the spec, it is preferable to have standalone sub-clauses in core specs for SBFD-specific existing or new gNB RF requirements.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, RAN4#112, Samsung.

- R4-2413407, “Topic summary for [112][307] NR_duplex_evo_General”, RAN4#112, Samsung.

- TR 38.858, “Study on evolution of NR duplex operation”, 3GPP.