3GPP TSG-RAN4 Meeting #112bis	R4-2415468

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.20.2.1

Source: 	Qualcomm Incorporated

Title: 	Views on new requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#110-bis, a WF was agreed in [2]. In this contribution we share our views on existing BS RF requirements for Rel-19 SBFD.

# New BS RF requirements for SBFD operation

In [2], the following was agreed regarding the potentially new in-channel adjacent subband leakage ratio:

- FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline. - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline. - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.

Within in-band operation, the in-channel adjacent subband leakage will impact the receiving subband of the same BS. Such leakage might contribute to REFSENSE. It was noted during the SI [3] that OTA REFSENSE will be derived considering self-interference, inter-site and inter-sector interference. The in-channel adjacent subband leakage (selectivity) is the same as the ACLR (ACS), but on a subband in-channel granularity level. Consequently, there is a direct implication of the coexistence scenarios and assumptions on the subband leakage and selectivity definition.

Additionally, during the study item phase [3], it was for further study whether new requirements will be needed or not. The dependency on the self-interference capability yields the definition of a new inter-subband requirement difficult to agree between different companies. As observed in Section 9 in [3], different implementations can be assumed between vendors, leading to different self-interference, co-sector interference, and inter-site interference mitigation capabilities. Moreover, employed technologies and mitigation techniques in the SBFD-capable gNB is quite difficult within RAN4, and thus, room for implementation differences should be taken into account when defining the new inter-subband leakage, selectivity, and blocking requirements. As a result, it might be difficult to propose technically meaningful requirements taken into consideration the above aspects.

Observation 1: The in-channel subband sub-band Tx leakage, selectivity, and blocking depend on the self-interference, inter-site and inter-sector interference considerations, thus, they are implementation dependent.

Taking into consideration also the techniques introduced by other WGs to address inter-gNB and inter-UE CLI, the definition of new RAN4 requirements should take into consideration such techniques. As a result, we prefer that RAN4 would adopt option 1 in [2], where no new requirements for in-channel leakage, selectivity, and blocking are defined for Rel-19 SBFD-capable gNBs.

Proposal 1: RAN4 to agree that no new requirements for in-channel leakage, selectivity, and blocking are defined for Rel-19 SBFD-capable gNBs.

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Observation 1: The in-channel subband sub-band Tx leakage, selectivity, and blocking depend on the self-interference, inter-site and inter-sector interference considerations, thus, they are implementation dependent.

Proposal 1: RAN4 to agree that no new requirements for in-channel leakage, selectivity, and blocking are defined for Rel-19 SBFD-capable gNBs.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, RAN4#112, Samsung.

- R4-2413407, “Topic summary for [112][307] NR_duplex_evo_General”, RAN4#112, Samsung.

- TR 38.858, “Study on evolution of NR duplex operation”, 3GPP.