3GPP TSG-RAN4 Meeting #112bis	R4-2415473

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Text proposal on 7125 – 8400 MHz IMT parameters in TR 38.922

Source:	Qualcomm Incorporated

Agenda item:	6.2.3

Document for:	Approval

# Introduction

In this document we provide our text proposals for the duplexing mode for the 7125-8400 MHz frequency ranges based on the agreed TP in RAN4#112 R4-2414300.

******************Begin changed sections******************

# 5	7125 - 8400 MHz frequency range

## 5.1	General parameters

### 5.1.2 Channel bandwidth

While a number of channel bandwidth will be specified for this frequency range, 100 MHz is considered as a typical channel bandwidth. Higher channel bandwidths compared to 100MHz are not precluded for this range. Annex B entails additional information on the impact of higher channel bandwidth on ACIR.

## 5.3	UE Parameters

### 5.3.1	Transmitter Characteristics

#### 5.3.1.5	Maximum output power

The UE maximum output power for the considered frequency ranges could be 23 dBm. Other UE power classes, e.g. 20dBm, 26dBm and 29dBm, are not precluded (corresponding ACLR limit will be adapted accordingly to avoid additional interference). Annex B entails additional information on the impact of higher output power on ACIR.

## Annex B: Additional observations for 7125-8400 MHz frequency range

An important aspect to consider is the implications of higher channel bandwidth on the adjacent channel coexistence findings. Accordingly, simulation studies were conducted to compare the throughput loss% as a function of ACIR for both downlink and uplink transmissions in UMa deployments for both 100MHz and 200MHZ, as shown in Figure 1. Note that all the network, BS, and UE parameters follow the adjacent channel coexistence conducted in TR 38.921. It can be observed that the ACIR required to meet the 5% throughput loss degradation target is nearly identical for the 100 MHz and 200 MHz channel bandwidths.

Figure 1 TP loss for UMa scenario for CHBW 100MHz (black) and 200MHz dBm (magenta)

In order to ensure that the adjacent channel coexistence (i.e., BS and UE ACLR/ACS) derived for PC3 (i.e., UE maximum output power equals 23 dBm) is not impacted when considering higher UE maximum output power (e.g., PC2, with maximum output power equals 26 dBm), we compare the throughput loss % as a function of ACIR for both downlink and uplink transmissions in UMa deployments for both PC3 and PC2, as shown in Figure 2. Note that all the network, BS, and UE parameters follow the adjacent channel coexistence conducted in TR 38.921. It can be observed that the ACIR required to meet the 5% throughput loss degradation mark is nearly the same for both PC3 and PC2.

Figure 2 TP loss for UL UMa scenario for 23 dBm (magenta) and 26 dBm (black) UE maximum output power

******************End changed sections******************