3GPP TSG-RAN4 Meeting #112bis	R4-2415475

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Views on Radio parameters for 14800 - 15350 MHz

Source:	Qualcomm Incorporated

Agenda item:	6.2.4.2

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. In RAN4#112, a WF was agreed in [2] and [3] regarding BS and UE parameters for the 14800 – 15350 MHz frequency range. In this contribution, we provide our views on the open items regarding system parameters and RF parameters for BS and UE items for the 14800 – 15350 MHz frequency range.

# 2 	Discussion

## 2.1	System parameters

### 2.1.1	Duplexing

Since there is no 14800 – 15350 MHz frequency ranges, it is expected that the discussion regarding duplexing would be similar to the 7125 – 8400 MHz frequency range. Accordingly, RAN4 is expected to consider TDD as a candidate duplexing for the 14800 – 15350 MHz Frequency range in its LS response to WP5D, while capturing additional information on SBFD in TR 38.922.

Proposal 1: RAN4 to agree on TDD as a baseline duplexing for 14800 – 15350 MHz frequency range and capture the following text in TR 38.922.

“Even though FDD is not precluded, most likely TDD will be used in this frequency range. In addition, SBFD is considered as a candidate duplexing method. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].”

### 2.1.2	Channel bandwidth

In general, wider channel bandwidths should be enabled with increasing frequency following same approach as FR1 (3 to 100 MHz) to FR2 (50 to 400 MHz). This comes with increasing SCS and increasing RB width and availability of the spectrum. Since the 14800 - 15350 MHz frequency range is not in the FR1 frequency range, higher channel bandwidths compared to 100MHz should be the baseline. Such channel bandwidths can be achieved via higher SCS or intra-band CA employment. Thus, we propose to consider 200MHz as a baseline for this frequency range, while mentioning that other channel bandwidths are not precluded.

Proposal 2: RAN4 to adopt 200MHz is a typical channel bandwidth. Additionally, it should be stated clearly in TR 38.922 that higher channel bandwidths should not be precluded for this frequency range.

### 2.1.3	Signal bandwidth

Signal bandwidth depends on the channel bandwidth and the considered SCS. The transmission bandwidth configuration defined in RAN4 core spec for BS and UE can be reused to derive the necessary signal bandwidth, where the signal bandwidth is calculated based on the NR spectrum utilization (number of PRBs and SCS) as follows:

Signal bandwidth = NPRB x SCS x 12

For the targeted bandwidth, 30 and 60 KHz might be possible candidates for this frequency range. Since it will be difficult to agree within RAN4 on a given SCS till RAN4#112 (i.e., RAN4 target deadline for 7GHz parameters), it is proposed to mention the dependency of signal bandwidth on the SCS and number of RBs.

Proposal 3: As signal bandwidth depends on CHBW and SCS, RAN4 to not specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs, similar to what was done to 7125-8400 MHz frequency range.

## 2.2	BS parameters

### 2.2.1	Array size

From a feasibility point of view and with the expectation to provide wide-area coverage for this frequency range, considering at least 1k elements at the BS side is a viable option. When considering large antenna elements at this frequency range, there has been some discussion within RAN4 on the near-field impacts. This is justified under the fact that in the context of large antenna arrays and short UE-gNB distances where the overall array dimensions are of the same order as the transmitter to receiver separation, such large panels may potentially test the limits of planar wavefront modelling and may necessitate spherical wavefront modelling where the exact distance between any pair of transmit and receive antennas needs to be taken into account. Spherical wavefronts occur naturally when the antennas are modelled as point sources. Such a phenomena is currently being under investigation as part of the Rel -19 SI on FR3 channel modelling (see RP-234018  for more details). The impact of modelling near-by users using spherical waves on system level evaluations is still not clear and thus, it is recommended that RAN4 postpones the near-field aspects for this frequency range in the normative phase and not address it within its reply to WP5D until RAN1 concludes its study

Observation 1: The impact of modelling near-by users using spherical waves on system level evaluations is not clear in RAN4 and currently being studied in RAN1.

Proposal 4: RAN4 to not consider near-field impacts in this frequency range in its correspondence to WP5D.

### 2.2.2 Spectral mask

For 10.0 - 10.5 GHz range derivation of the spectral mask in TR 38.921, RAN4 considered that channel bandwidths less than 50MHz are not foreseeable, thus, the basic limits for the SEM were based on 50MHz. RAN4 could follow the same approach considering scaling the basic limits for OOB emission mask with an appropriate minimum channel bandwidth value.

Observation 2: To derive the BS SEM, RAN4 could follow the same approach as the BS SEM definition in TR 38.921 with the appropriate minimum foreseeable channel bandwidth.

### 2.2.3	BS ACLR/ACS

Generally, the BS ACLR and ACS are based on the outcome of the adjacent channel coexistence. Since this frequency range falls between FR1 and FR2-1, RAN4 should make sure that the defined ACLR/ ACS are consistent with the defined ACLR/ACS for FR1 and FR2-1.

Proposal 5: For BS ACLR and ACS will be based on the outcome of the adjacent channel coexistence. In addition, RAN4 should make sure that the defined ACLR/ ACS are consistent with the defined ACLR/ACS for FR1 and FR2-1.

### 2.2.4	Blocking response

By definition, the in-band blocking requirement should apply from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the operating band. In TR 38.921, ΔfOOB = 100 MHz was assumed for 10GHz frequency range. Adopting ΔfOOB = 100 MHz for the 14800-15300 MHz frequency range should be further discussed. On top of that, it is proposed that in-band and out-of-band blocking requirements follow what was agreed in TR 38.921.

Proposal 6: RAN4 if ΔfOOB = 100 MHz should be revisited or not for the definition of in-band and out-of-band blocking.

## 2.3	UE parameters

### 2.3.1	Maximum Output power

PC3 was assumed during Rel-17 work on 7GHz and 10GHz as captured in [3]. On the other hand, n104 considers both PC2 and PC3. Additionally, 3GPP has studied high power UEs during Rel-17 covering high-power UE (power class 1.5) operation in NR bands n77 and n78 (RP-202912), high power UE for NR TDD intra-band Carrier Aggregation in frequency range FR1 (RP-221774), and high-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands (RP-221795). To capture the trend of higher UE power classes, we propose to communicate with WP5D PC3 (i.e., 23 dBm) as the typical UE output power, while capture in the TR that higher UE maximum output power values (26 dBm or 29 dBm) are not precluded in the future.

Proposal 7: RAN4 to agree on 23 dBm as UE typical maximum output power in the LS reply and to include in TR that higher power classes are not precluded in the normative phase of this frequency range.

### 2.3.2	UE power dynamic range

To derive the power dynamic range, RAN4 needs to agree on the UE minimum and maximum output power. We propose to reuse FR1 UE minimum output power of -33 dBm while 23 dBm as maximum output power. Accordingly, we propose 59 dBm for the UE power dynamic range.

Proposal 8: RAN4 to agree on 56 dBm as UE power dynamic range.

### 2.3.4 	UE beamforming

In [3] the following was agreed on the adopted model for UE beamforming

- FR1-like UE: - Option 1: 1TX assumption with omnidirectional antenna - Option 2: Two discrete elements following Table 1-1 below with maximum-gain across elements selection criterion. - Option 3: For [N] Tx/Rx, [G] dBi gain for link of interest regardless the AoA, and 0dB for signals from all non-serving co-channel and adjacent-channel links regardless the AoA. - Companies to clearly state what value of N and G assumed. - FR2-like UE: - FR2-likUE: Two panels with 1x2 array at each panel following Table 1-2 below with maximum gain across panels criterion.

The “FR1-like” option has three options, firstly isotropic assumption, which follows that of TR 38.921 where an isotropic omnidirectional antenna with 0 dBi. Second option considers two discrete antenna elements, each with a maximum antenna gain, and then antenna selection diversity is considered to select the antenna with the maximum gain for a given link between a UE and its serving BS.  Third assumption represents a simplified model to account for the digital beamforming gain. On the other hand, the “FR2-like” option represents a similar model that was adopted in NR Release 15 studies (for more info, reader is kindly referred to TR 38.803).

Additionally, In WP5D initial LS (R4-2400333) to RAN, we have the following text:

Whether AAS at IMT user equipment is likely or not. If AAS at IMT user equipment is considered as a possibility WP 5D welcomes information relating to the structure of the AAS at the IMT user equipment, associated frequency bands, and the associated parameters of the AAS.

For the 4400-4800 MHz and 7125-8400 MHz Frequency range, TR 38.922 clearly states that the UE is isotropic with 0 dBi (similar to WRC-23 assumptions) so that in the LS; no new info was shared. However, for the 14800-15350 MHz frequency range, RAN4 is expected to provide a clear response on the AAS model at the UE, in case a model different than isotropic 0 dBi model is agreed.

Observation 2: Per WP5D request in LS R4-2400333, RAN4 is expected to provide a clear response on the AAS model at the UE, in case a model different than isotropic 0 dBi model is agreed.

Since RAN4 has not converged on what option to consider, it is plausible that if RAN4 responds in the LS to WP5D with two options, WP5D will select the one with worst case implications (e.g., higher interference impact) for the sharing studies. Thus, it is recommended that RAN4 would select single UE beamforming model for WP5D to consider in their sharing studies. In the TR, the different models and the underlying motivation of each model can be captured.

Observation 3: IF RAN4 responds with more than one model for UE beamforming, WP5D will chose the model with worst case implications from sharing studies point of view.

Proposal 9: If RAN4 cannot converge on a UE beamforming model, RAN4 could reply to WP5D with two variants; UE without beamforming and with beamforming, without putting a label of FR1 or FR2 on the response.

### 2.3.5	UE Noise figure

In [3], different options for the UE noise floor were proposed:

Issue 3-3: UE noise factor for both simulation and LS reply - FFS the below options: - Option 1: 11 dB i.e. follow simulation assumption - Option 2: 8 dB - Option 3: More than 11 dB

As stated in the WF from [3], in the simulation campaign, 11 dB was agreed for the UE noise figure. Since such assumption will be captured in the TR after the completion of the SI, it is recommended that RAN4 keeps the simulation assumptions as consistent as possible with the response to WP5D on the different RF parameters. Accordingly, adoption 11 dB for both simulation and LS reply is a favourable WF from our point of view.

Proposal 10: RAN4 to consider 11 dB as UE noise figure for both simulation and LS reply, to ensure consistency within the TR between simulation assumptions and RF parameters.

### 2.3.6	Spectral mask

For 10.0 - 10.5 GHz range derivation of the UE SEM in TR 38.921, is relaxed compared to NR general NR FR1 SEM at the FOOB edge ± 0 – 5 MHz by 3 dB. For the 14800 – 15300 MHz frequency range, the adoption of high SCS is not precluded (e.g., 60KHz). In order to ensure that the region of the SEM close to the CHBW is compliant with the SEM future requirements, it is viable to consider similar relaxation at the FOOB edge ± 0 – 5 MHz by 3 dB.

Proposal 11: RAN4 to consider UE SEM given in TR 38.921, which is relaxed compared to NR general NR FR1 SEM at the FOOB edge ± 0 – 5 MHz by 3 dB.

### 2.3.3	UE ACLR/ACS

Typically, the UE ACLR and ACS should be derived based on the outcome of the adjacent channel coexistence. However, additional consideration should be considered in order to make sure that the ACIR ( BS/UE ACLR and ACS) value is reasonable. As an example, for FR2-1 UE, the uplink ACIR resulting from the adjacent channel coexistence work, captured in TR 38.803, resulted in 16 dB ACIR, which is evaluated based on 17 dB UE ACLR and 23.5 dB BS ACS. In reality, the UE need to also meet the occupied bandwidth requirement, which is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. For NR, it is equal to the channel bandwidth with a maximum of 1% of emissions allowed outside of it (0.5% on each side), which result in around 24 dBc UE ACLR requirement. Accordingly, RAN4 should make sure that the BS and UE ACLR/ACS values communicated to WP5D are reasonable considering other RF requirements.

Proposal 12: For UE ACLR and ACS, RAN4 to decide on the UE ACLR and ACS based on the outcome of the adjacent channel coexistence study. RAN4 should make sure that the BS and UE ACLR/ACS values communicated to WP5D are reasonable considering other RF requirements.

### 2.3.7	 Spurious emissions

Since it will be difficult to finalize if this frequency range will be delt as FR1 or FR2-1 band during the study item period, our preference is to follow what was done for the 10GHz during Rel-17 study item. Thus, we propose that RAN4 would adopt the general spurious emissions defined in TS 38.101-1 for the 14800-15300 MHz frequency range.

Proposal 13: RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1.

### 2.3.8	Blocking response

Given that no 3GPP band is yet specified for this frequency range, it is viable to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied for 14800 – 15300  MHz frequency range.

Proposal 14: For UE blocking response, RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

# 3	Conclusion

In this paper we have shared our views on the RF parameters for 14800 – 15350 MHz Frequency range. Our observations and proposals can be summarized for that frequency range as follows:

Proposal 1: RAN4 to agree on TDD as a baseline duplexing for 14800 – 15350 MHz frequency range and capture the following text in TR 38.922.

“Even though FDD is not precluded, most likely TDD will be used in this frequency range. In addition, SBFD is considered as a candidate duplexing method. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].”

Proposal 2: RAN4 to adopt 200MHz is a typical channel bandwidth. Additionally, it should be stated clearly in TR 38.922 that higher channel bandwidths should not be precluded for this frequency range.

Proposal 3: As signal bandwidth depends on CHBW and SCS, RAN4 to not specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs, similar to what was done to 7125-8400 MHz frequency range.

Observation 1: The impact of modelling near-by users using spherical waves on system level evaluations is not clear in RAN4 and currently being studied in RAN1.

Proposal 4: RAN4 to not consider near-field impacts in this frequency range in its correspondence to WP5D.

Proposal 5: For BS ACLR and ACS will be based on the outcome of the adjacent channel coexistence. In addition, RAN4 should make sure that the defined ACLR/ ACS are consistent with the defined ACLR/ACS for FR1 and FR2-1.

Proposal 6: RAN4 to discuss if ΔfOOB = 100 MHz should be revisited or not for the definition of in-band and out-of-band blocking.

Proposal 7: RAN4 to agree on 23 dBm as UE typical maximum output power in the LS reply and to include in TR that higher power classes are not precluded in the normative phase of this frequency range.

Proposal 8: RAN4 to agree on 56 dBm as UE power dynamic range.

Observation 2: Per WP5D request in LS R4-2400333, RAN4 is expected to provide a clear response on the AAS model at the UE, in case a model different than isotropic 0 dBi model is agreed.

Observation 3: IF RAN4 responds with more than one model for UE beamforming, WP5D will chose the model with worst case implications from sharing studies point of view.

Proposal 9: If RAN4 cannot converge on a UE beamforming model, RAN4 could reply to WP5D with two variants; UE without beamforming and with beamforming, without putting a label of FR1 or FR2 on the response.

Proposal 10: RAN4 to consider 11 dB as UE noise figure for both simulation and LS reply, to ensure consistency within the TR between simulation assumptions and RF parameters.

Proposal 11: RAN4 to consider UE SEM given in TR 38.921, which is relaxed compared to NR general NR FR1 SEM at the FOOB edge ± 0 – 5 MHz by 3 dB.

Proposal 12: For UE ACLR and ACS, RAN4 to decide on the UE ACLR and ACS based on the outcome of the adjacent channel coexistence study. RAN4 should make sure that the BS and UE ACLR/ACS values communicated to WP5D are reasonable considering other RF requirements.

Proposal 13: RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1.

Proposal 14: For UE blocking response, RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

# Reference

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN #103, Maastricht, Netherlands.

[2] R4-2414283, “WF on 15GHz BS parameters”, ZTE, RAN4#112, Maastricht, Netherlands.

[2] R4-2414284, “WF on 15GHz UE parameters”, Qualcomm, RAN4#112, Maastricht, Netherlands.

[4]	TR 38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz”, 3GPP V17.1.0.